3GPP TSG-RAN4 Meeting #115	R4-2507992

# Malta, MT, 19th – 23rd, May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1 to Introduce R19 ATG enhancement UE RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce R19 ATG enhancement UE RF requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify operating band and configurations for ATG with CA and UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R19 ATG enhancement UE RF requirements would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2J, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2507061 |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-1>>

5.2J	Operating band for ATG

5.2J.1	General

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

5.2J.1A	Operating band for ATG CA

NR carrier aggregation operating bands defined in Table 5.2J.1A.1-1 and Table 5.2J.1A.2-1, can be applied for ATG CA operation.

5.2J.1A.1	Operating band for ATG intra-band CA

Table 5.2A.1-1: ATG intra-band contiguous CA operating bands

| NR ATG CA Band | NR ATG Band (Table 5.2-1) |
| --- | --- |
| CA_n79 | n79 |
| NOTE: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. |  |

5.2J.1A.2	Operating band for ATG inter-band CA

Table 5.2J.1A.2-1: ATG inter-band CA operating bands

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 1) |
| --- | --- | --- |
| CA_n3-n39 | n3, n39 |  |
| NOTE 1: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

5.2J.1D	Operating band for ATG UL MIMO

NR operating bands in Table 5.2J.1D-1 to support UL MIMO, can be applied for ATG UL MIMO operation.

Table 5.2J.1D-1: NR operating bands for UL MIMO in FR1

NR operating band

n1

n3

n34

n38

n39

n41

n78

n79

<<Next of Change>>

5.5J	Void

5.5J.1A	Configurations for ATG CA

The ATG intra-band contiguous CA configurations and channel bandwidths are specified in clause 5.5J.1A.1. The ATG inter-band CA configurations and channel bandwidths are specified in clause 5.5J.1A.2.

5.5J.1A.1	Configurations for ATG intra-band contiguous CA

Table 5.5J.1A.1-1 Configurations and bandwidth combination set for ATG intra-band contiguous CA in FR1

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | n79 | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  |  | 200 | 4 and 5 |

5.5J.1A.2	Configurations for ATG inter-band CA

Table 5.5J.1A.2-1 Configurations and bandwidth combination set for ATG inter-band CA in FR1

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | n3 n39 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
| NOTE 1: For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs. |  |  |  |  |

<<End of Change>>