3GPP TSG-RAN WG4 Meeting #115	R4-2508036

Malta, MT, 19th – 23rd May, 2025

Title: 	WF on FR2 power class 8

Agenda Item:	5.1.1

Source: 	Huawei, HiSilicon, China Telecom, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRT, Apple

Document for:	Approval

<Topic #1> FR2 power class 8 (in [115][104])

- Back ground

The proposals for FR2 power class 8 (PC8) with the minimum peak EIRP 16.4dBm were proposed and discussed extensively in the previous meetings [1]. The agreements were reached [2], but the CRs were not completed unfortunately.

The intention of the FR2 PC8 is to address the potential FR2 UE availability issue, if the mmWave BS is widely deployed in the future. It is not to replace the existing FR2 power class 3 or other power classes. Instead, it provides the solution for low and middle end UEs to support FR2 with the good trade-off between performance and complexity/power consumption. Without such solution, it would be expected that only a small portion of high end of UE supports FR2 and it is not beneficial to promote mmWave industry.

According to analysis in [1], the power consumption of adding one FR2 band with PC3 for a UE is 3~4 times higher compared to adding one FR1 band, and the complexity/cost of adding one FR2 band with PC3 is 2 times higher than adding one FR1 band. Those aspects would limit the wide support of FR2 UE.

The main concern on the introduction of FR2 PC8 is related to FR2 uplink coverage of PC8 compared to PC3. But in our view, the mmWave BS would be deployed primarily in the dense urban, where the FR1 coverage is always available and UE can fall back to FR1 when FR2 coverage is not stable. And the network can control the access of PC8 if it thinks that PC8 impacts the system performance and wants to serve the PC3 users.

The purpose of the FR2 PC8 proposal is to provide a more reasonable starting point for mmWave UEs to fulfil the real deployment request, since the mmWave spectrum might be issued in some regions recently and some operators own the mmWave already and may consider the plan to deploy mmWave. Like FR1, although NR UE supports 2Tx and 4Rx already, such UE capability does not exist from Day 1 for wireless communication. Similarly, we should have a reasonable starting point for FR2 considering the trade-off between the complexity and performance. In the long run the UE capability will evolve like what happens for FR1 UE in terms of antenna numbers and transmission power.

In order to address the companies’ concerns and meet the potential deployment request, we would like to provide the following compromise:

- Agree on the CRs to capture the impacts of introduction of FR2 PC8 on RAN4 specifications and hold on approving them in the upcoming RAN plenary. The intention is to get the technique solution ready.

  - Keep FFS on the applicability of FR2 PC8 for which UE type

    - For non-handheld UE, the example main use case is for portable CPE which connects with network by Uu and connects with UE by WiFi.

  - The applicability of FR2 PC8 can be limited to a certain region, and consider the mechanism for operators to control the access of FR2 PC8.

- Approve a way forward

  - Re-open the discussions on whether to formally approve the formal CRs in RAN plenary upon the trigger from operators when they have clear plan to deploy the mmWave, with understanding that

    - The more evaluations would be needed, which may be based on the outcome of more field trial

    - It is allowed to discuss and decide which UE types the FR2 PC8 can be applied to.

- Way forward

The following way forward is proposed for approval:

- Agree on the CRs to capture the impacts of introduction of FR2 PC8 on RAN4 specifications and hold on approving them in the upcoming RAN plenary.

  - Keep FFS on the applicability of FR2 PC8 for which UE type

  - Limit the applicability of FR2 PC8 to a certain region and have mechanism for operators to control the access of FR2 PC8 UE to their network.

    - FFS on the region to be limited to.

- Re-open the discussions on whether to formally approve the formal CRs in RAN plenary upon the trigger from operators when they have clear plan to deploy the mmWave, with understanding that

  - The more evaluations would be needed, which may be based on the outcome of more field trial

  - It is allowed to further discuss and decide which UE types the FR2 PC8 can be applied to.