3GPP TSG-RAN WG4 Meeting #115									                          R4-2508050

St Julian’s, Malta, 19-23 May 2025

Agenda item:			5.1.1

Source:	Skyworks, (…)

Title:	WF on optional CHBW across releases

Document for:	Approval

# 1		Introduction

This WF captures the agreements on the optional 3MHz CBW, 5MHz CBW for band n41/n90, 35MHz CBW for band n71 and the impact on MSD requirements.

# 2		WF#1 on optional CBW.

Agreement 1:

- 3MHz CBW:

  - For MSD analyses, do not consider 3MHz CBW except when the DL affected band is band n106.

  - Companies are invited to share their views at next meeting whether 3MHz CBW should become mandatory for bands n26, n28, n31, n72, n85 and n100.

  - Note: For band n28, one operator can only deploy 3MHz in this band.

- 5MHz CBW for band n41/n90:

  - For MSD analyses, consider 10MHz as the smallest DL CBW.

  - Companies are invited to further study whether 5MHz CBW for bands n41/n90 could be removed from Rel-19.

- 35MHz CBW for band n71

  - Companies are invited to further study whether 35MHz CBW for band n71 should become mandatory support.

# 2			WF#2 on MSD requirements

Agreement 2: PC3 MSD for CA_n5A-n71A:

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 680.5 | 35 | 15 | 20 (RBstart=168) | 871.5 | 5 | 2.07 | >ACLR2 |
| NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. |  |  |  |  |  |  |  |  |  |

Agreement 3: PC3 MSD for CA_n78A-n102A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n102 | 3750 | 100 | 30 | 270 (RBstart=3) | 5935 | 20 | 14.4 | >ACLR2 |
| n102 | n78 | 5975 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 15.8 | >ACLR2 |

Agreement 4: PC3 MSD for CA_n3A-n74A and CA_n3A-n75A:

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1735 | 50 | 15 | 50 (RBstart=0) | 1515.5 | 5 | 6.4 | >ACLR2 |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 6.4 | >ACLR2 |
| n3 | n75 | 1712.5 | 50 | 15 | 50 (RBstart=0) | 1514.5 | 5 | 6.4 | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1514.5 | 5 | 6.4 | >ACLR2 |

Agreement 5: PC2 MSD for CA_n2A-n66A

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1890 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.76 0.97 | >ACLR2 |

# 3			References