3GPP TSG-RAN WG4 Meeting #115	R4-2508086

Malta, MT, 19th – 23rd May, 2025

Title: 	WF on 6Rx and SRS IL imbalance

Agenda Item:	7.1.1

Source: 	Huawei, HiSilicon

Document for:	Approval

<Topic #1> Reference sensitivity requirements

<Sub-topic 1-1> FWA terminology

Agreement:

- Remove “form factor” from the notes in the corresponding tables for 4Rx, 6Rx and 8Rx, and use “FWA CPE” or “FWA” for ΔRIB,6R requirement in the note of the table.

  - Consider adding the definition of FWA CPE or FWA, and industrial UE in the specifications

  - Focus on modifying the notes of the receiver requirements for 4Rx, 6Rx and 8Rx for Rel-19 specifications in RAN4#115

  - FFS on whether and how to align the existing notes between 4Rx, 6Rx and 8Rx since the notes were specified according to the separate WIDs.

- Further modifications will be done in the future meetings

  - Address the similar issues for previous releases under the maintenance agenda

  - Address the similar issue for transmission requirements under the maintenance agenda.

<Topic #2> MIMO layer evaluation for 6Rx UE

<Sub-topic 2-1> Release independency for 6Rx UE

Agreement:

- Option 1:

  - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA  from Rel-18

  - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19

- Option 2:

  - Support 6Rx from Rel-19

- Option 3:

  - Support 6Rx from Rel-18

<Topic #3> SRS antenna switching requirements

<Sub-topic 3-1> UE capability signalling related to 3t6r

Agreement: (online)

- Since RAN1 has already sent the LS on UE feature list to RAN2 and asked them to design corresponding UE capability and 3T6R Antenna Switching was introduced in NR_MIMO_Ph5, it is not necessary for RAN4 to send LS to RAN2 to introduce UE capability signalling for t3r6 SRS antenna switching.

<Topic #4> SRS IL imbalance

<Sub-topic 4-1> Solutions for SRS IL imbalance issue

Agreement:(online agreement)

- Do not introduce any solution for SRS-IL imbalance issue for UE in Rel-19

  - It is FFS on if and how to introduce [R17] RMC and test for SRS. The related discussion should be contribution driven and in the agenda item of R17 maintenance.

<Topic #5> Feature list for 6Rx UE

Agreement:

- The following feature group list is agreed.

Note: RAN4 has already sent out the LS to RAN2 for 6 layer related capability

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 46-6 | Support maximum 6 MIMO layers for DL reception | Indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA. |  | Yes | N/A | FWA does not support maximum 6 MIMO layers for DL reception | Per FSPC | N/A | Yes (only FR1) | N/A |  | Optional with capability signalling |