3GPP TSG RAN WG4 Meeting #115		                                                                  R4-2508092

St Julian, Malta, May 19th – 23rd , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR> | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS 38.101-1 on UE channel bandwidth for LP-WUS/WUR (sub-clause 5.3M) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding a new sub-clause 5.3M for UE channel bandwidth for LP-WUS/WUR |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding a new sub-clause 5.3M |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LP-WUS/WUR is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

## 5.3I	Channel bandwidth for (e)RedCap

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

3MHz channel bandwidth is not applicable for (e)RedCap UE in the current release.

## 5.3M		UE channel bandwidth for LP-WUS/WUR

### 5.3M.1	General

As illustrated in Figure 5.3M-1, tThe LP-WUS carrier bandwidth corresponding to the UE channel bandwidth for LP-WUS/WUR is defined as the sum of resource blocks (RBs) occupied by the LP-WUS signal and the guard RBs separating it from the NR signal. The LP-WUS carrier is embedded within the NR channel and is flexibly positionable, provided alignment with the NR PRB grid is maintained.

The LP-WUS signal shall occupy 11 resource blocks (RBs).

A guard RB is referred to as an ASCS guard RB when located between an NR RB and an LP-WUS RB, and as an ACS guard RB when positioned between the NR guardband and an LP-WUS RB.

Fig. 5.3M.1-1 Definition of LP-WUS/WUR carrier bandwidth

### 5.3M.2 	Maximum transmission bandwidth

The maximum transmission bandwidth NRB_WUR for LP-WUS within each NR UE channel bandwidth and subcarrier spacing is specified in Table 5.3M.2-1.

Table 5.3M.2-1: Maximum transmission bandwidth configuration NRB_WUR for WUR

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR |
| 15 | N/A | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| 30 | N/A | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

<End of changes>