3GPP TSG-RAN4 Meeting #115	R4-2508098

# St Julian’s, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to Rel-19 TS38.101-1 on receiver characteristics for LP-WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, OPPO, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-05-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce receiver characteristics for LP-WUR, including spurious response, intermodulation and spurious emissions requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clauses to define receiver characteristics for LP-WUR, including spurious response, intermodulation and spurious emissions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious response, intermodulation and spurious emissions requirements for LP-WUR are not defined in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7M(new), 7.8M.1(new), 7.8M.2(new), 7.9M(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

7.7K	(Reserved)

7.7L	(Reserved)

7.7M	Spurious response for LP-WUR

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in clause 7.6M.3 is not met.

The MDR criterion with the side conditions specified in 7.1M shall be met with parameters for LP-WUS as specified in Table 7.7M-1 and Table 7.7M-2 for Type 1 LP-WUR and Type 2 LP-WUR respectively, and for the interference as specified in Table 7.7M-3.

Table 7.7M-1  Spurious response parameters for Type 1 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENS + [TBD]dB |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

Table 7.7M-2  Spurious response parameters for Type 2 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENS + [TBD]dB |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

Table 7.7M-3  Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | [-44] |
| FInterferer | MHz | Spurious response frequencies |

# << Next change >>

7.8K	(Reserved)

7.8L	(Reserved)

7.8M	Intermodulation characteristics for LP-WUR

- 7.8M.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

- 7.8M.2	Wide band Intermodulation for LP-WUR

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively.

The MDR criterion with the side conditions specified in 7.1M shall be met with parameters specified in Table 7.8M.2-1 and Table 7.8M.2-2 for Type 1 LP-WUR and Type 2 LP-WUR respectively.

Table 7.8M.2-1: Wide band intermodulation parameters for Type 1 LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + [TBD]dB |
| PInterferer 1 (CW) | dBm | [-46] |
| PInterferer 2 (Modulated) | dBm | [-46] |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference and with channel bandwidth BWChannel ≥ 5 MHz. NOTE 2: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 3: The Finterferer 1 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

Table 7.8M.2-2: Wide band intermodulation parameters for Type 2 LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + [TBD]dB |
| PInterferer 1 (CW) | dBm | [-46] |
| PInterferer 2 (Modulated) | dBm | [-46] |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference and with channel bandwidth BWChannel ≥ 5 MHz. NOTE 2: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 3: The Finterferer 1 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

# << Next change >>

7.9J	Spurious emissions for ATG

For ATG UE, the spurious emissions as specified in clause 7.9 applies.

7.9M	Spurious emissions for LP-WUR

For LP-WUR, the spurious emissions as specified in clause 7.9 applies.

# << End of change >>