3GPP TSG-RAN WG4 Meeting #115	R4-2508268

Malta, Malta, May 19th – May 23rd, 2025

Agenda item:			7.23.1

Source:	Moderator (Nokia)

Title:	WF on AI/ML Mobility Part 1

Document for:	Approval

# Introduction

- This WF captures the discussions and agreements carried out on AI/ML Mobility SI under [115][215] FS_NR_AIML_Mob Part 1 thread.

# Agreements.

## RAN4 requirements for RRM Measurement prediction

### Agreements in the RRM session

Issue 2-1: Relative RSRP accuracy definition

Agreement:

- For intra-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 1 and cell 2 are on the same frequency

    - the reported RSRP of cell2 can be measured or predicted.

    - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

  - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

- For inter-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

    - the reported RSRP of cell2 can be measured or predicted.

    - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

### Agreements in the Adhoc session

Issue 2-5: Impacts on beam/cell level measurements

Agreement:

Consider L3 beam-level (Point E) measurement prediction depending upon the progress in RAN2.

Note: RAN4 didn’t study impact of L3 beam level measurements. However, it doesn’t preclude the discussion on L3 beam level measurements in the work item phase.

Issue 2-7: Inter-frequency prediction scenario

Agreement:

RAN4 to study whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

## RAN4 requirements for measurement event prediction

### Agreements in the Adhoc session

Issue 3-1: Performance metric for event prediction use case

Agreement:

In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.