3GPP TSG-RAN WG4 Meeting #115										     R4-2508272

Malta, MT, May 19 – 23, 2025

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part2

Agenda Item:	7.24.1

Source: 	MediaTek inc.

Document for:	Approval

# <Topic 1: UE reporting enhancement for CJT calibration>

The simulation results for the delay, frequency offset reporting for CJT calibration are collected from companies’ contributions and are captured in R4-2508292.

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

- RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting.

- RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration?

Way forward:

- Option 1: The scheduling restriction is specified as:

  - For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting.

- Option 2: No scheduling restrictions needed to be applied for CJT reporting.

Issue 1-3: How to define measurement restriction requirements for delay and frequency offset reporting for CJT calibration?

Way forward:

- Option 1: Reuse the legacy measurement restriction in FR1 for delay and frequency offset measurement.

  - Measurement restriction does apply,

    - If UE does not support simultaneousRxDataSSB-DiffNumerology and

    - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols.

  - Measurement restriction does not apply,

    - If UE support simultaneousRxDataSSB-DiffNumerology or

    - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH.

- Option 2: If CJTC measurement collides with other L1 measurement, follow legacy principle, UE is required to measure one of but not both of different measurements when measurement resources are at the same time.

# <Topic 2: Enhancement for asymmetric DL sTRP/UL mTRP scenarios>

Moderator’s note on the scenarios’ definition discussed in this section:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

## Sub-topic 2-1 Scenario#1

Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward: FFS

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward:

- Option 1: Gradual timing adjustment applies for each TAG and shall be met for each TAG.

- Option 2: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements?

Way forward: FFS

## Sub-topic 2-2 Scenario#2

Issue 2-4: For Scenario#2, how to update PRACH and PUCCH/PUSCH/SRS transmission requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

For Scenario#2, inter-cell operation, PRACH reference point:

  - reference point for PRACH transmission is

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL [or UL] TCI state(s) of the reference cell associated with the second TAG

For inter-cell multi-TRP, for PUCCH/PUSCH/SRS:

    - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [or UL TCI states] having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

Issue 2-5: For Scenario#2, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

- Autonomous timing adjustment applies to both TAGs or TRPs.

- Two independent gradual timing adjustments with potentially different values should be applied to both TAGs.

Issue 2-6: For Scenario#2, if the UE is configured with SSB-MTC-additionalPCI, how to define MRTD and MTTD requirements?

Agreement:

- Reuse Rel 18 values of MRTD and MTTD for Rel 19.

Issue 2-7: Whether to define UL TCI state activation requirement for Scenario#2?

Agreement:

The existing requirements for unified TCI state with mTRP are applicable to Scenario#2 and no new requirements to be introduced. FFS on spec impact.