3GPP TSG-RAN WG4 Meeting # 115					                           R4-2508279

Malta, 19 May – 23 May, 2025

Agenda item:			7.28.1

Source:	Moderator (vivo)

Title:				         Ad hoc minutes for NR_LPWUS

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the discussion in [115][223] NR_LPWUS for the following issues:

Issue 1-2-6-1: On the number of SSBs (for RAN1 LS R1-2501624)

Issue 2-2-2: Summary on proposal/observations based on simulation results

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Issue 1-1-11: RRM requirements for FR2

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

Issue 2-1-7: On interference in simulation

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

Issue 2-1-11: On I and Q branches in simulation

Issue 1-2-4-1: On LR measurement filtering requirement

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Discussion

Issue 1-2-6-1: On the number of SSBs (for RAN1 LS R1-2501624)

- Proposals

  - P1: [3 5 5] SSBs for [70]ms, [500]ms and [900] ms cases when SSB is 20ms (vivo oppo Apple LG Ericsson MTK)

    - P1-1: If necessary, the number of SSBs is under the side conditions LR SINR >= -3 dB. (vivo Apple LG MTK)

    - P1-2:The number of SSBs can be derived without linking to any side conditions. (vivo oppo)

  - P1a: [5 6 6] SSBs for [70]ms, [500]ms and [900] ms cases when SSB is 20ms when LR SINR < -3 dB. (MTK)

  - P2: [5 5 5] for [70]ms, [500]ms and [900] ms when SSB is 20ms (Xiaomi);

  - P3: [2 2 2] for [70]ms, [500]ms and [900] ms when SSB is 20ms (Nokia, Qualcomm)

    - P3-1: 1 SSB samples is enough for sync in good radio conditions where LR is assumed to be used, 1 sample can be used for AGC at -2 dB; (Nokia)

  - P4: [2 4 4] for [70]ms, [500]ms and [900] ms when SSB is 20ms (CMCC);

  - P5: [3 3 3] for [70]ms, [500]ms and [900] ms when SSB is 20ms (Huawei ZTE);

Recommendations:

1.

For deep sleep mode:

- 2 (Nokia, Qualcomm CMCC)

Not support: Apple MTK HW oppo  xiaomi

- 3 (vivo oppo Apple LG Ericsson MTK Huawei ZTE)

Not support:

- 5 (xiaomi) (MTK only when LR SINR < -3 dB)

Not support:

MTK: link to particular SNR conditions

Apple: RAN1 has already consider the scenario after waking up

Xiaomi:

Tentative agreement:

For the 400ms and 800ms ramping up time;

Use [5 5]

Final check time is Wednesday this week, if no further issue, this tentative agreement will be turned into agreement.

Support: Apple MTK LG xiaomi cmcc oppo CATT QC

Not support: ZTE

Use [4 4]

Support: ZTE CMCC QC Nokia

Not Support: MTK Oppo Apple Xiaomi

Tentative agreement:

For the 10ms ramping up time

Down-select from option 1 and 2. Other Options are precluded.

Option 1: 2 and 3

Support: Ericsson QC Apple Nokia MTK ZTE

Not Support: HW

Option 2: 3

Support: ZTE HW oppo Apple MTK Nokia LG Xiaomi CMCC

Not Support: QC

[3 3 3]  Huawei ZTE

Not support: MTK Apple xiaomi oppo LG

[3 5 5] vivo oppo Apple LG Ericsson MTK

Not Support: ZTE QC Nokia

[3 4 4]

Apple: Based on reporting value, fix LO location on the time domain regardless of sample number

[4 4 4]

QC: first number 4 is not acceptable

CMCC: Still need to differentiate deep sleep mode and ultra-deep sleep mode

Ericsson: Report multiple value for each state

For ultra-deep sleep mode:

- 2 (Nokia, Qualcomm)

- 3 (Huawei ZTE)

- 4 (CMCC)

- 5 (vivo oppo Apple LG Ericsson MTK xiaomi)

- 6 (MTK only when LR SINR < -3 dB)

2: whether to have different SSB number for different cases (wake up from deep sleep state or wake up from ultra-deep sleep state):

Option 1: Yes

vivo oppo Apple LG Ericsson MTK CMCC

Option 2: No

xiaomi Nokia Qualcomm Huawei ZTE

3. On the number of SSB, whether to consider a particular SINR perspective or only consider the minimum number under any SINR

Option 1: Yes

Option 2: No

Agreement

For the side conditions, RAN4 assume the side conditions is the side conditions of the [LP-WUS monitoring] used by RAN1

The wording in the [] can be revisited.

Nokia: for the number for different state, NW may assume the worst case

QC: for the different case, the MR implementation is the same hence the same number should be applied for different case

MTK: depends on UE implementation, wake up from deelp sleelp and ultra-deep sleep have different UE behaviour

Background (Agreement at RAN4 114 bis)

Issue 1-2-6-1: On the number of SSBs

Agreement: Aim to conclude the issue by Tuesday in RAN4 #115.

Issue 1-2-6-2: On SSBs number for different SSB periodicities

Agreement

Same SSB number for different SSB periodicity

Agreement

When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay.

Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

RAN1#119 meeting:

For future meetings:

Consider the following alternatives for UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

RAN1#120 meeting: R1-2501624

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

Issue 2-2-2: Summary on proposal/observations based on simulation results

OOK based LR

| Results on number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRP for AWGN and TDL-C channel at -3 dB SINR, for information purpose |  |
| --- | --- |
| CMCC | 2 |
| oppo | 2 |
| ZTE | 3 |
| LG | 1 |
| vivo | 1 3 samples are required if the ideal RSRP calculation is not passed the fading channel under TDL-C channel |
| xiaomi | 2 5 LP-SS samples to achieve 2.5 dB absolute baseband accuracy at SNR = -3dB in TDL-C channel |
| Apple | 3 Beyond 3 samples we observe diminishing returns. Relax the accuracy requirement to 4dB if considering 2 samples for L1 filtering |
| Ericsson | 2 |
| Huawei | 2 |
| Nokia | 2 |
| MTK | 3 5 when SINR = -6dB |

| Initial results on number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR, for information purpose |  |
| --- | --- |
| CMCC |  |
| ZTE |  |
| LG |  |
| vivo | 3 The target RSRP measurement accuracy can be achieved by 1 and 3 samples for AWGN and TDL-C channel at SINR = -3dB based on 320ms periodicity of LP-SS |
| Ericsson | 2 |
| Huawei | 2 |
| Nokia | 2 |
| MTK |  |

OFDM based LR

| Initial results on number samples for achieving ±3.5 dB accuracy for SSB based RSRP for AWGN and TDL-C channel at -3 dB SINR, for information purpose |  |
| --- | --- |
| CMCC |  |
| ZTE |  |
| LG |  |
| vivo | 1 |
| Ericsson |  |
| Huawei | 2 |
| Nokia |  |
| MTK |  |

| Initial results on number samples for achieving ±3.5 dB accuracy for SSB based RSRQ for AWGN and TDL-C channel at -3 dB SINR, for information purpose |  |
| --- | --- |
| CMCC |  |
| ZTE |  |
| LG |  |
| vivo | 1 |
| Ericsson |  |
| Huawei | 2 |
| Nokia |  |
| MTK |  |

Recommendations:

Suggest to consider the following working assumption:

Working assumption 1:

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRP at -3 dB SINR is [from 1 to 3];

When LP-SS samples number is larger than 3 at -3 dB SINR, the extra gain on accuracy for LP-SS RSRP is smaller;

Working assumption 2:

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR is [2 or 3];

Working assumption 3:

The number of SSB samples for LR to achieving ±3.5 dB accuracy for SSB based RSRP and RSRQ for AWGN and TDL-C channel at -3 dB SINR is [1 or 2 ];

Apple: The simulation assumption may not be fully aligned.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

- Proposals

- P1: The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined as x+1 and y + 1 (vivo)

- P2: The evaluation requirement (x1/y1) can be decided after RAN4 has simulation alignment for measurement requirement (x/y). Consider using x1=2*x and y1=2*y for the evaluation requirement. (Apple)

- P3: When entry condition is configured for LP-WUR, the UE shall evaluate the corresponding entry condition before entering LP-WUS monitoring, RRM relaxation or RRM offloading within evaluation period; Discuss how often LP-SS / PSS/SSS evaluation is performed; (Nokia)

- P4: Define LR evaluation requirement based on x1 = y1 = [4] if x = y = 2 (Huawei)

- P5: x1,y1=N*x, M*y (N>1,M>1), where N=2 and M=2 (ZTE)

- P6: For LR evaluation requirements a higher number of samples is required compared to MR evaluation (where MR reuses legacy Nserv) to achieve reliable evaluation. (MTK)

Background:

RAN4 114 agreement:

Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases.

Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement.

LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y.

Number x and y are defined as measurement requirement as  [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

Recommendations:

Working assumption:

Option 1: using x1=2*x and y1=2*y for the evaluation requirement.

Option 2: The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined by adding extra [1 or 2 ] samples

ZTE: the extra number of samples in x1 or y1 is to avoid ping-pong.

Issue 1-1-11: RRM requirements for FR2

- Proposals

  - P1: Prioritize defining RRM requirements for FR1 in this WI (Apple Nokia)

  - P2: There is not any concrete proposal on FR2 requirements so far. Suggest RAN4 to confirm whether FR2 RRM requirement needs be defined in this release, or confirm the status of FR2 requirement will not impact the completion of WI (vivo, Nokia)

Recommendations:

QC: One solution is to reuse the same number of scaling factors.

Apple: In the current table, sweeping factor is decreased according to different DRX value.

QC: Try to conclude the sweeping factor for FR2 in the next meeting.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

- Proposals

  - P1: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms (xiaomi)

    - Option 1: define the lower limit for measurement delay requirements; (Huawei)

    - Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

  - P2: No need for lower bound. (oppo Nokia CMCC ZTE Apple)

  - P3: The LR requirements based on LP-SS can be classified as requirements based on 160ms and requirements based on 320ms. (vivo Nokia)

  - P4: Define lower bound for LP-SS measurement periodicity as 320ms (Huawei)

Background:

RAN4 112bis meeting agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

RAN1 120bis meeting agreement:

Agreement

Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms.

  - Additionally support LP-SS periodicity of 160ms.

Recommendations:

The LR requirements based on LP-SS can be defined as requirements based on 160ms and requirements based on 320ms

Issue 2-1-7: On interference in simulation

- Proposals

  - P1: For the case {M, L}={1,6} when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case. (vivo)

  - P2: RAN4 to discuss if RSRP/RSRQ accuracy requirements should be based on the worst case scenario of colliding LP-SS binary patterns between serving cell and interfering cell (Apple)

  - P3: As an alternative to mandate LP-SS across cells to operate in different time/frequency resources, RAN4 to discuss if the condition of interfering cell vs serving cell power is -6dB is overly pessimistic and whether to relax it to 1) a lower value (<-6dB), or 2) simply remove the interfering cell from the simulation setup. (Apple)

  - P4: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences. (Huawei)

Background:

- Set of binary sequences for LP-SS:

- For M=1, L=6, the set of LP-SS sequence is:

[1 0 1 0 1 0]

[0 1 0 1 0 1]

[1 0 0 1 0 1]

[1 0 1 0 0 1]

- For M=1, L=8, the set of LP-SS sequence is:

[1 0 1 0 0 1 0 1]

[1 0 1 0 1 0 0 1]

[1 0 0 1 0 1 0 1]

[0 1 0 1 0 1 0 1]

- For M=2, L=12, the set of LP-SS sequence is:

[1 0 0 1 1 0 0 1 1 0 0 1]

[0 1 1 0 1 0 0 1 1 0 0 1]

[0 1 1 0 0 1 1 0 1 0 0 1]

[0 1 1 0 0 1 0 1 1 0 0 1]

- For M=2, L=16, the set of LP-SS sequence is:

[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

[1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

[1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1]

[1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

- For M=4, L=16, the set of LP-SS sequence is:

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

- For M=4, L=32, the set of LP-SS sequence is:

[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

[0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

[0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

[0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1]

For M=1, L=6, the following set is the only case with fully opposite sequence:

[1 0 1 0 1 0]

[0 1 0 1 0 1]

Recommendations:

For the case {M, L}={1,6} when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case.

MTK: we have similar observation as HW. We have worse results.

Apple: more discussion is needed for this issue.

CMCC: We also have similar observation as HW and MTK

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

- Proposals

  - P1-1: We observe the target LP-RSRP measurement accuracy can be achieved by 1 LP-SS sample if the ideal RSRP is pass the fading channel. 3 samples are required under TDL-C channel if the ideal RSRP calculation is not passed the fading channel (vivo)

  - P1-2: RAN4 to confirm that ideal RSRP is derived from the serving cell signal after the fading channel; (Huawei)

  - P2-1: Ideal RSRQ is derived based on ideal RSRP divided by the actual RSSI (Huawei)

  - P2-2: The ideal RSSI is calculated by passing signals and interference through corresponding fading channel (vivo).

Background:

For ideal RSRP:

  - Method 1: The first one is the RSRP is pass the fading channel and with the perfect channel estimation and the real RSRP pass the same channel.

  - Method 2: The second one assumes the impact of the fading channel can be completely removed hence the ideal RSRP does not need pass the channel whereas the real RSRP will pass the fading channel.

Agreement:

Use method 1 as a base, further check the difference between method 1 and method 2 in P2 and check the channel estimation assumption in ideal RSRP for OOK based LR

Recommendations:

Check whether the P1-2 “the ideal RSRP is derived from the serving cell signal after the fading channel” is ok providing companies are fine with observation P1-1a.

Agreement:

Use method 1 in future simulation alignment

Issue 2-1-11: On I and Q branches in simulation

- Proposals

  - P1: The RSRP measurement accuracy of 1-branch receiver will deteriorate compared to 2-branch receiver, but the requirements, in terms of the number of measurement samples, will not change. (vivo)

  - P2: RAN4 simulations assume both I & Q branches. If needed, define single branch as a capability in core requirements. (Nokia)

  - P3: Simulation results using both I and Q branches (vivo LG CMCC Huawei)

  - P4: RAN4 not to align simulation assumption on whether OOK based LR is using I-branch only or using both I- and Q-branch. (Huawei)

  - P5: RAN4 needs to align the assumption for the number of branches (I/Q) used by the OOK receiver for the provided simulation results. The ideal RSRP and the measured RSRP to be derived based on the assumption that the OOK-based LP-WUR is equipped with both I and Q branches. (MTK)

Recommendations:

RAN4 simulation is based on using both I and Q branches

Agreement:

For RAN4 simulation alignment, RAN4 simulation should consider power from both I and Q branches

Issue 1-2-4-1: On LR measurement filtering requirement

- Proposals

  - P1: No need define filtering requirement for LP-RSRP and LP-RSRQ measurements. (vivo CMCC Huawei ZTE)

    - P1-1: UE shall filter the LP-RSRP and LP-RSRQ measurement of serving cell using at least 2 measurements. (CMCC)

    - P1-2: This can be revisited if RAN1 introduces LP-SS periodicity lower than 160ms. (Huawei)

  - P2: The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by LP-SS periodicity for both OFDM and OOK based LR. (Apple CT)

Background:

Recommendations:

From RRM requirement perspective, P1 or P2 may not have any difference.

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

- Proposals

  - P1: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements. (vivo Huawei Apple)

  - P2: Consider 3.5 dB RF impairment margin for LR (xiaomi)

Background:

Recommendations:

Check whether P1 is agreeable

Xiaomi: We understand the 2.5 dB is the number for normal UE. Prefer to consider a larger number.

MTK: Support P2

CMCC: Support P1

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

- Proposals

  - P1: No LR measurement and evaluation requirements apply at the legacy state, i.e., for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3). (vivo xiaomi Apple CT Huawei ZTE)

  - P1-1: The requirements in clause apply when the LP-WUR is in ON state. Before entering LP-WUS, RRM offloading and RRM relaxation mode and after exiting LP-WUS, RRM offloading and RRM relaxation mode, it is up to LP-WUS UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement. (Nokia)

Background:

- Summary of entry/exit conditions based on existing RAN2’s agreements

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | MR threshold and optional LR threshold |  |

Agreement at RAN4 114bis:

- At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

Recommendations: