3GPP TSG-RAN WG4 Meeting # 115			                                R4-2508280

Malta, 19 May – 23 May, 2025

Agenda item:			7.28.1

Source:	Moderator (vivo)

Title:	WF on RRM requirements for NR_LPWUS

Document for:	Approval

Topic #1: Agreement on RRM core requirements for LP-WUS/WUR

- Sub-topic 1-1 General aspects

Issue 1-1-10: Higher priority frequency layer measurement requirements

Agreement:

For case 1:

  - MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60

    - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1.

For case 3:

  - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60

  - When the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met, the same requirement for higher priority, equal priority and lower priority carriers:

    - 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied

Issue 1-1-17 Reply RAN1 LS R1-2503103

Agreement on the reply LS to RAN1:

- Confirm to RAN1 that for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- RAN4 agreed that: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

- Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-4-1: On LR measurement filtering requirement

Agreement:

No need to define filtering requirement for LP-RSRP and LP-RSRQ measurements.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

Agreement: This issue is closed.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Working assumption:

Option 1: using x1=2*x and y1=2*y for the evaluation requirement.

Option 2: The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined by adding extra [1 or 2 ] samples

Issue 1-2-6-1: On the number of SSBs (for RAN1 LS R1-2501624)

Agreement:

For the 400ms and 800ms ramping up time: {5, 5} samples

For the 10ms ramping up time: 3 samples

Agreement

For the side conditions, RAN4 assume the side conditions is the side conditions of the LP-WUS monitoring used by RAN1

- Sub-topic 1-3 MR RRM relaxation

- Sub-topic 1-4 LP-WUR CONNECTED mode

- Sub-topic 1-5 Others

Topic #2: Agreement on simulation assumptions and results

Sub-topic 2-1 On simulation assumptions and parameters

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

Agreement:

Use method 1 in future simulation alignment

Issue 2-1-11: On I and Q branches in simulation

Agreement:

For RAN4 simulation alignment, RAN4 simulation should consider power from both I and Q branches

Sub-topic 2-2 Simulation results alignment and summary

Issue 2-2-2: Summary on proposal/observations based on simulation results

Working assumption 1:

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRP at -3 dB SINR is [from 1 to 3];

When LP-SS samples number is larger than 3 at -3 dB SINR, the extra gain on accuracy for LP-SS RSRP is smaller;

Working assumption 2:

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR is [2 or 3];

Topic #3: Issues to be closed

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Up to RAN2, close the issue

Issue 1-1-2: Threshold for switch between different cases

Up to RAN2, close the issue

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation

Close the issue

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

Close the issue

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

Close the issue

Issue 1-1-8: LP-WUR operating carrier frequency

Close this issue. No impact on RAN4 RRM requirements from this issue.

Topic #4: Recommendation for open issues

Details on all open issues can be found in [1] and [2].

Continue discussion on issues in performance part.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Continue discussion

Issue 1-1-11: RRM requirements for FR2

Continue discussion

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Continue discussion

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

Continue discussion

Issue 1-2-4-2-1: On applicability LR evaluation requirements

Discuss in the CR

Issue 1-2-5-1: On applicability MR evaluation requirements

Discuss in the CR

Issue 1-2-6-3: On RAN4 related requirements and other issues

Discuss in the CR

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

Discuss in the CR

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

Discuss in the CR

Issue 1-4-1: LP-WUR at CONNECTED mode

Continue discussion for interesting companies

Issue 1-5-2: eDRX related

Contribution driven

Issue 1-5-3: MR measurements after wake-up

Contribution driven

Reference

[1] R4-2505572, Topic summary for [115][223] NR_LPWUS, vivo, RAN4 115

[2] R4-2508279, Ad hoc minutes for NR_LPWUS, vivo, RAN4 115