3GPP TSG-RAN WG4 Meeting #115	                                                	         R4-2508281

Malta, MT, 19 – 23 May, 2025

Agenda item:			7.29.1

Source:	Apple

Title:	Ad-hoc minutes for NR_Mob_Ph4

Document for:	Information

# Topic #1: General aspects and work plan

## Open issues summary

#### Issue 1-1: RAN4 UE feature list for event triggered L1 reporting

Candidate options:

- Option 1: not needed (vivo)

Recommended WF:

- No need to introduce any new RAN4 UE capability for event triggered L1 reporting for LTM.

#### Issue 1-2: RAN4 UE feature list for CSI-RS based L1 measurement

Candidate options:

- Option 1: Apple

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for L1-RSRP measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | Yes | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} Note: It is also counted in FG 2-24 | Optional with capability signaling |
|  | 56-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and neighboring cells across all frequency layers for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighboring cells across all frequency layers for L1 measurement. | Per BC | No | No | N/A | Candidate values: {2,4,8,12,16,32,64} Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 56-3 | Skipping SSB based L1 measurement in FR2-1 | - Support of skipping SSB based L1 measurement for FR2-1 for CSI-RS based L1 measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | N/A | The network cannot have optimized measurement configuration | Per UE | no | FR2-1 only | N/A | - For a UE not supporting the capability, SSB based L1 measurement is necessary for FR2-1. - The CSI-RS resources for L1 measurement should be at least Type-D QCL’ed with the SSB for L1 measurement. - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped: - The CSI-RS resources for L1 measurement should be Type-D QCL’ed with the SSB for L3 measurement. - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor. | Optional with capability signalling |

- Option 2: vivo

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1-1 | Number of neighbour cells to be measured per frequency layer | The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for CSI-RS based intra-frequency | 63-1 or 63-2 | Yes | No | There is no limitation on the number of neighbour cells per frequency layer for CSI-RS based L1 measurement. | Per BC | No | No | N/A | Candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signalling |
| 56. NR_Mob_Ph4 | 56-1-2 | Number of CSI-RS resources for L1-RSRP measurement within a slot | The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra-frequency L1-RSRP measurement | 63-1 or 63-2 | Yes | No | There is no limitation on the number of CSI-RS resources of intra-frequency for L1 measurement within a slot. | Per BC | No | Yes | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} | Optional with capability signalling |
| 56. NR_Mob_Ph4 | 56-1-3 | Number of CSI-RS resources for L1-RSRP measurement per frequency layer | The max number of CSI-RS resources UE can measure for L1-RSRP per frequency layer for intra frequency | 63-1 or 63-2 | Yes | No | There is no limitation on the number of CSI-RS resources per frequency layer for L1 measurement. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8} | Optional with capability signalling |

- Option 3: E///

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1 | Skip SSB based L1-RSRP measurement for neighboring cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during neighboring cell CSI-RS-based L1-RSRP measurement. The UE, indicating this capability, supports RX beam sweeping using CSI-RS resources while performing CSI-RS-based L1 RSRP measurements on the neighboring cell, thereby skipping the SSB-based L1-RSRP measurement to acquire a fine RX beam for L1-RSRP computation. | 2-21, 2-22 or 2-23, 2-23a | yes | N/A | SSB based L1-RSRP cannot skipped while performing CSI-RS based L1-RSRP | Per Band | NO | Yes, only for FR2 | NO |  | Optional with capability signalling |

Recommended WF:

- Online discussion:

- Number of CSI-RS resources for L1-RSRP measurement within a slot. (Apple, vivo, MTK, HW, E///, ZTE, CATT)

Option A: 2 is needed, FFS for 3+4 (MTK, HW, CATT, ZTE, QC, Nokia, CMCC, E///)

- Number of total CSI-RS resources [across all serving carriers] to be measured.

  - Number of total CSI-RS resources across all RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource

Option B: 3 + 4 only (vivo)

- Number of neighbour cells to be measured [per serving carrier]. (vivo)

- Number of CSI-RS resources for L1-RSRP measurement [per serving carrier]. (vivo)

Note: the wording of [per serving carrier] can be further discussed.

Option C: 2+3+4 (vivo)

- Skipping SSB based L1 measurement in FR2-1 (already agreed, discuss details)

Agreement during online session:

Define UE capabilities:

- Number of CSI-RS resources for L1-RSRP measurement within a slot.

The above agreements can be revisited if not aligned with the agreements in RAN1/2.

Send the following capabilities to Xiaoran:

- Number of CSI-RS resources for L1-RSRP measurement within a slot.

- Number of total CSI-RS resources [across all serving carriers] to be measured.

- [FFS: Number of CSI-RS resources for L1-RSRP measurement per serving carrier]

- Skipping SSB based L1 measurement in FR2-1

# Topic #2: event triggered L1 measurement reporting

## Open issues summary

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements: (vivo, Xiaomi)

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

- Option 2: Seems no need to capture assumptions listed under option-1 into core requirements. (Apple)

- Option 2a: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases. (HW)

Recommended WF:

- Discuss the options

#### Issue 2-2: Event triggered reporting delay for L1 reporting

Agreement in RAN4#114bis:

- Reuse the framework from L3 event triggered reporting. Discuss the details further.

Candidate options:

- Option 1: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period on the cells corresponding to the event. (OPPO, Xiaomi, Apple, HW)

  - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. (Apple, HW)

  - For periodic MR: after the first report, UE follow the network configuration on the reporting interval. (Apple, HW)

- Option 2: (Nokia, CMCC)

  - The event triggered measurement reporting delay without the UL resource waiting time taken into account shall be no longer than the L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB of the measurement that triggers the report, where TL1-RSRP_Measurement_Period_SSB is the measurement period of serving or candidate cell depending on which measurement triggers the report.

Tentative Agreement during online session:

- The event triggered measurement reporting delay shall be no larger than the maximum of L1-RSRP measurement periods on the cells corresponding to the event.

  - [In at least one of the test cases, verify the event is triggered by both serving cell and neigboring cell with different measurement periods.]

#### Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///)

  - Details:

    - Option 1a:(vivo)

      - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability.

      - Reporting criteria for LTM event triggered reporting is 4 per frequency layer.

      - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured.

    - Option 1b: E///

      - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

      - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS

- Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO)

- Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

Recommended WF:

- Define a separate capability for supported number of events triggered L1 measurement reporting.

- Continue discussion on details based on option 1a and 1b:

    - Option 1a:(vivo)

      - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability.

      - Reporting criteria for LTM event triggered reporting is 4 per frequency layer.

      - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured.

    - Option 1b: E///

      - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

      - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS

#### Issue 2-4: Impact on other LTM requirements

Agreement in RAN4#114bis:

- For FR1:

  - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

Candidate options:

- Proposal 1: considering it is invalid that only event entering condition is configured and leaving condition is not configured, update previous agreement to: (Huawei)

  - For FR1:

    - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met

    - The related cell switch delay and TCI state activation requirements are applicable

  - The above optimization is not applicable to FR2

- Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable. (Nokia)

- Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report.

- Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia)

- Proposal 5: add the following condition on agreements under issue 2-7 in RAN4#114bis: (Apple)

  - Number of active TCI does not exceed that UE can support.

- Proposal 6: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases. (MTK)

- Proposal 7: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable. (ZTE)

- Proposal 8: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///)

  - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured

Recommended WF:

- Discuss the proposals one by one.

#### Issue 2-5: CR

- Please provide comments on CR below:

| TDoc | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2505711 | draftCR on capabilities for Support of Event Triggering and Reporting Criteria | OPPO |  |
| R4-2505732 | DraftCR for LTM L1 event-triggered reporting | Nokia |  |
| R4-2506617 | DraftCR on event-triggered reporting for CSI-RS based L1 measurement | CMCC |  |
| R4-2507017 | Draft CR on event-Triggered Reporting for Intra-Frequency L1 Measurements on Neighboring Cells | Ericsson |  |

# Topic #3: CSI-RS based L1 measurement

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

#### Issue 3-1: definition on CSI-RS based intra-frequency L1 measurement

- Proposals

  - Option 1: Xiaomi, Apple, CATT

A measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

•	It is applied for SCS = 60KHz

    - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

  - Option 2: China Telecom, OPPO, MediaTek, Apple, ZTE, Huawei, Ericsson

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

-	the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

•	It is applied for SCS = 60KHz

-	the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

-	at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

  - Option 3: vivo

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

  - Option 4: Nokia

Intra-frequency condition 1: the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

Intra-frequency condition 2: the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP. (It is applied for SCS = 60KHz)

Intra-frequency condition 3: at least 48 RBs of the CSI-RS resource needs to be confined within the active DL BWP.

Not to have condition where “the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP”

  - Option 5: CMCC

Considering that RAN4 requirements only apply to measurement without gap, no need to introduce definition of CSI-RS based L1 intra-frequency/inter-frequency measurement.

- Recommended WF

  - Use option 2 as baseline.

ZTE: we are fine with option 2. But we would like to consider inter-frequency w/o gap as well.

Tentative Agreement (need to check with CMCC):

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and

-	the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and

•	It is applied for SCS = 60KHz

-	at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP.

Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement.

    - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19.

#### Issue 3-2: whether step 2 is skipped for serving cell?

- Proposals

  - Option 1: OPPO

Whether step 2 is skipped for serving cell or not can also be based on the same UE capability.

  - Option 2: Xiaomi

For the serving cell, SSB-based L1 measurements shall not be skipped in FR2-1, even if the UE supports capability reporting to skip them for L1-RSRP for neighbour cell (e.g., for LTM purposes).

  - Option 3: Apple, CATT, MediaTek, ZTE, Huawei, Ericsson

It is proposed to keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

  - Option 4: Nokia

At least step 2 can be skipped if the serving cell is current candidate cell. Further discuss in which conditions step 3 can be skipped if the candidate cell is the current serving cell.

- Recommended WF

  - Agree on option 3.

OPPO: we prefer option 1.

QC: not define requirements for this case:

    - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW)

    - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one fo the FR2 serving cells. (QC, OPPO)

    - Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

#### Issue 3-3: Sharing factor for resource collision between serving and neighbour cell

- Proposals

  - Option 1: Huawei

For step-2 skipped CSI-RS L1 measurement requirement, if the CSI-RS resources in serving cell and neighbour cell are colliding in time domain, sharing factor shall be introduced.

- Recommended WF

  - Discuss the option.

For FR2 CSI-RS L1 measurement requirement,

    - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,.

      - Option 1: sharing factor shall be introduced. (HW, QC, E///)

      - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

#### Issue 3-4: Scaling factor for Rx beam sweeping

- Proposals

  - Option 1: Ericsson

For a UE supporting CSI-RS based RX beam sweeping, beam sweeping factor is equal to 4.

  - Option 2: Xiaomi

For neighbor cell CSI-RS based L1-RSRP measurement in FR2, if SSB based measurement is skipped, the measurement period for CSI-RS based L1-RSRP will be scaled by 8

  - Option 3: MediaTek

For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, when step 2 is skipped, the beam sweeping factor can be 8 or 8* maxNumberRxBeam.

- Recommended WF

  - Discuss the options.

Agreement:

    - For neighbor cell CSI-RS based L1-RSRP measurement in FR2, if SSB based measurement is skipped, the measurement period for CSI-RS based L1-RSRP will be scaled by 8.

#### Issue 3-5: UE measurement behaviour if CSI-RS configuration exceeds UE measurement capability

- Proposals

  - Option 1: Apple

It is proposed that Number of CSI-RS resources for L1-RSRP measurement within a slot and Number of total CSI-RS resources to be measured are based on UE capability reporting. The measurement requirement is only applicable within UE’s capability. If the number of CSI-RS resources exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure

  - Option 2: Ericsson

RAN4 should define the number of intra-frequency and inter-frequency layers that the UE must be capable of measuring for the CSI-RS based L1-RSRP on the neighbouring cells.

  - Option 3: Nokia

Discuss if RAN4 supports scenario where CSI-RS configuration exceeds UE measurement capability.

Discuss how many CSI-RS measurements within UE is capable of performing during each measurement period.

- Recommended WF

  - Focus on UE measurement behaviour (option 1 and first sentence under option 3). Other can be discussed under UE feature list.

Agreement:

    - RAN4 requirements apply assuming network will not configure CSI-RS based L1 RSRP measurement for LTM candidate cell exceeding UE measurement capability.

      - FFS on whether to capture this agreement in RAN4 spec.

#### Issue 3-6: time restriction in measurement requirements applicability.

- Proposals

  - Option 1: Apple, ZTE, Ericsson

In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

  - Option 2: vivo

When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

- Recommended WF

  - Discuss the options.

#### Issue 3-7: Measurement with repetition ON/OFF

- Proposals

  - Option 1: Nokia

For both FR1 and FR2. If the repetition RRC object is not configured the requirements treat this as the repetition would be “off”.  In other words, UE ignores the repetition parameter from RAN4 requirement point of view.

- Recommended WF

  - Discuss the options.

#### Issue 3-8: Requirement applicability

- Proposals

  - Option 1: MediaTek

Proposal 4: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell (when RTD<CP) in FR1, legacy measurement period, measurement restriction and scheduling restriction requirements are all applicable.

Proposal 5: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when two CSI-RS resources from difference cells overlap in time domain.

Proposal 6: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define scheduling restriction.

Proposal 7: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, legacy measurement period for CSI-RS L1 measurement with repetition off is applicable when SSB based L1 measurement is not skipped.

Proposal 8: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, the structure of legacy measurement period for CSI-RS L1 measurement is applicable and the value of beam sweeping factor shall be updated when SSB based L1 measurement is skipped.

  - Option 2: Nokia

After TCI state activation command, CSI-RS measurement(s) for the cells whose TCI state(s) are in the active TCI state list shall be prioritized

  - Option 3: Qualcomm

RAN4 requirements apply only when all CSI-RS resources from candidate cells on the same layer are configured with the SCS and CP type.

- Recommended WF

  - Discuss the options.

Agreement:

    - For intra-f CSI-RS based L1-RSRP measurement on neighbor cell (when RTD<CP) in FR1, reuse legacy measurement period, measurement restriction and scheduling restriction requirements as baseline.

    - For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define scheduling restriction.

#### Issue 3-9: early CSI acquisition

- Proposals

  - Option 1: OPPO

For early CSI acquisition before LTM cell switch command MAC-CE, RAN4 considers to define the potential requirements including the start time and corresponding pre-requisite conditions.

  - Option 2: vivo

RAN4 to define RRM requirements for target cell early CSI-acquisition.

    - A component of TCSI_reporting is added to LTM cell switch delay if the target cell early CSI-acquisition is configured.

    - TCSI_reporting is defined as the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331.

  - Option 3: MediaTek

For the baseline (CSI acquisition after LTM CSC MAC-CE reception), there is no RRM impact.

For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), not to define any RRM requirements in R19.

  - Option 4: Huawei

Not to define requirements for both baseline and optional early CSI acquisition on candidate cells.

  - Option 5: Ericsson

For the baseline CSI acquisition, RAN4 to define early CSI acquisition delay (TCSI_reporting_LTM) as the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), DLTM).

  - Option 6: Qualcomm

For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements.

For optional CSI acquisition (i.e., downlink CSI acquisition before LTM CSC MAC-CE reception), the same requirements and applicability rules as those for CSI-RS-based L1-RSRP measurements on candidate cells apply—meaning no separate requirements for early CSI acquisition.

  - Option 7: Nokia

Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition and what is the impact to the cell switch delay.

- Recommended WF

  - Discuss the options.

Candidate options:

    - Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///)

    - Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC)

    - Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

#### Issue 3-10: CR

- Please provide comments on CR below:

| T-doc number | Title | Company | Comments |
| --- | --- | --- | --- |
| R4-2505712 | On CSI-RS based L1 measurements | OPPO |  |
| R4-2505866 | draft CR on CSI-RS based L1 measurement requirement | Apple |  |
| R4-2506107 | Draft CR to TS 38.133 on number of CSI-RS, number of cells and number of CSI-RS resources | CATT |  |
| R4-2506140 | DraftCR to 38.133 on LTM CSI-RS applicability | Nokia |  |
| R4-2506461 | Draft CR on measurement reporting requirements for CSI-RS based LTM | vivo |  |
| R4-2506502 | Draft CR on introduction of CSI-RS L1-RSRP measurement | MediaTek Inc. |  |
| R4-2506568 | Discussion on CSI-RS based L1-RSRP measurement | ZTE Corporation, Sanechips |  |
| R4-2506577 | draftCR on measurement restriction for CSI-RS based L1-RSRP | ZTE Corporation, Sanechips |  |
| R4-2506732 | Draft CR on scheduling availability of UE during CSI-RS based L1-RSRP measurement | Huawei, HiSilicon |  |

# Topic #4: CLTM

#### Issue 1-3-1: General aspects of Subsequent Conditional LTM delay

- Candidate solutions:

- Proposal 1: The subsequent conditional LTM delay is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL transmission on the new target cell. (CTC, Nokia)

    - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. (CTC, CATT, Apple, ZTE, E///)

    - Proposal 1b: TEvent_DU needs to be updated. (CTC, Apple, ZTE)

    - Proposal 1c: Apart from the starting point of the delay, the delay requirement on conditional LTM can be reused to define the remaining components of subsequent CLTM. (vivo, Huawei, Nokia, ZTE)

- Recommended WF

  - Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

    - TRRC will not be considered in subsequent conditional LTM delay.

    - TEvent_DU needs to be updated

Agreement:

  - Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

    - TRRC will not be considered in subsequent conditional LTM delay.

    - TEvent_DU needs to be updated

#### Issue 1-2-5: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

discuss whether P4a and/or P4c can be agreed:

    - Proposal 4a: Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list.

          - Note: the limitation of 160/480 ms from Rel-18 still applies.

    - Proposal 4c: If UE received PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TAT is running.

    - Other proposals can be considered if consensus can be reached.

Agreement:

    - Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list.

      - Note: the limitation of 160/480 ms from Rel-18 still applies.

#### Issue 1-2-2: Tmeasure

For CSI-RS based L1-RSRP measurement:

discuss the conditions for TCSI-RS_SFN_intra = 0

discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS.

# Topic #5: performance

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Open issues summary

### Event-triggered L1 reporting

#### Issue 4-1-1: test case list for event-triggered L1 reporting

Candidate options:

- Option 1: Nokia

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

  - At least 1 test case for event-triggered periodic reporting.

- Option 2: Apple

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

- Option 3: CATT

- The new test cases for event triggered L1 measurement reporting need to defined in the clauses defined for R18 LTM.

  - TC for LTM Intra-frequency event triggered SSB based L1 measurement reporting tests without gap under non-DRX

  - TC for LTM Inter-frequency event triggered SSB based L1 measurement reporting tests with gaps under non-DRX

  - TC for LTM Intra-frequency event triggered SSB based L1 measurement reporting tests without gap under non-DRX with SSB index reading

  - For event triggered CSI-RS based L1 measurement reporting, only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined.

- Option 4: cover both SSB and CSI-RS based, in FR1 and FR2, repsectively (vivo)

- Option 5: follow aspects should be considered (ZTE)

  - With gap &without gap, intra-f & inter-f

  - non-DRX & DRX

  - With SSB index reading & without SSB index reading

  - Other aspects are not precluded.

- Option 6: CMCC

  - Both FR1 and FR2-1 are considered.

  - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered.

  - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered.

- Option 7: RAN4 to consider the list of test cases agreed in Release 18 to perform event-triggered L1-RSRP testing, thereby defining the same list of test cases for event-triggered L1-RSRP. Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting. (E///)

  - Intra-f L1-RSRP measurement for LTM

    - Intra-f L1-RSRP measurement in FR1

    - Intra-f L1-RSRP measurement in FR2

      - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated

    - Intra-f L1-RSRP measurement in FR2

      - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated

  - Inter-f L1-RSRP measurement with MG

    - Inter-f L1-RSRP measurement with MG in FR1

    - Inter-f L1-RSRP measurement with MG in FR2

  - Inter-f L1-RSRP measurement without gap for LTM

    - Inter-f L1-RSRP measurement without gap in FR1

    - Inter-f L1-RSRP measurement without gap in FR2

      - 2 neighbor cells, none of neighbor cells’ TCI state activated

- Option 8: HW

  - LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

  - LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

  - LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

  - LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

  - FFS on LTM Inter-frequency event triggered reporting tests for CSI-RS based L1 measurement

Recommended WF:

- Discuss the options

#### Issue 4-1-2: test configuration for event-triggered L1 reporting

Candidate options:

- Option 1: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met. (Nokia)

Recommended WF:

- Discuss the options

#### Issue 4-1-3: others

Candidate options:

- Option 1: it is proposed to define test cases for LTM event 2/3/4/5 (CMCC)

- Option 2: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///)

Recommended WF:

- Discuss the options

### CSI-RS based L1 measurement

#### Issue 4-2-1: CSI-RS based L1 measurement accuracy

Candidate options:

- Option 1: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell. (Apple, Nokia, CATT, vivo, MTK, ZTE, CMCC, E///)

- Option 1a: reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3. (vivo)

- Option 1b: (HW)

  - The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

    - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

    - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

    - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

  - The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

    - Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

    - Absolute and relative CSI-RS based inter-frequency L1-RSRP accuracy requirements for FR1 and FR2

  - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

- Option 1c: (CATT)

  - RAN4 to define new clauses for CSI-RS based L1-RSRP requirements for neighbor cell in clauses 10.1.19D, 10.1.19E, 10.1.20A and 10.1.20B as R18 SSB based L1-RSRP requirements for neighbor cell.

Recommended WF:

- Reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the following conditions:

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

  - Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

  - Absolute and relative CSI-RS based inter-frequency L1-RSRP accuracy requirements for FR1 and FR2

- Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

#### Issue 4-2-2: test case list for CSI-RS based L1 measurement

Candidate options

- Option 1: Nokia

  - FR1 intra-frequency without gaps test case.

  - FR2 intra-frequency without gaps test case with CSI-RS beam sweeping and without FR2 SSB measurements.

  - FR2 intra-frequency without gaps test case without CSI-RS sweeping and with FR2 SSB measurements.

- Option 2: Apple

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Option 3: CATT

  - CSI-RS based Intra-frequency L1-RSRP measurement in FR1. (CATT)

- Option 4: cover both delay and accuracy (vivo)

- Option 5: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested. (CMCC)

- Option 6: E///

  - Intra-f L1-RSRP measurement in FR1

  - Intra-f L1-RSRP measurement in FR2

  - Inter-f L1-RSRP measurement without gap in FR1

  - Inter-f L1-RSRP measurement without gap in FR2

Recommended WF:

- Discuss the options

#### Issue 4-2-3: test configuration for CSI-RS based L1 measurement - BW

Candidate options:

- Option 1: contain only 48 PRBs within the active BWP. (Nokia)

Recommended WF:

- Discuss the options

#### Issue 4-2-4: test configuration for CSI-RS based L1 measurement - resource type

Candidate options:

- Option 1: Define at least one test case with semi-persistent CSI-RS resource. Test cases not defined as semi-persistent have periodic CSI-RS resource configuration. Discuss whether the semi-persistent test case is an independent test case. (Nokia)

Recommended WF:

- Discuss the options

#### Issue 4-2-5: others

Candidate options:

- Option 1: Define test case(s) to verify early CSI acquisition if core requirements are specified. (vivo)

- Option 2: As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement. (HW)

- Option 3: In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. (HW)

Recommended WF:

- Discuss the options

### Conditional LTM

#### Issue 4-3-1: CLTM test scope – frequency range

Candidate options:

- Option 1: (Nokia, Apple, vivo, ZTE)

  - FR1 to FR1

  - FR2 to FR2

- Option 2: (CMCC)

  - PCell switch to a neighboring LTM candidate cell

    - FR1 to FR1

    - FR1 to FR2

    - FR2 to FR2

    - FR2 to FR1

  - PCell switch to an LTM candidate cell that is a serving SCell in MCG

    - FR1 cell to FR1 cell

    - FR2 cell to FR2 cell

- Option 3: E///

  - FR1 Pcell switch to a FR2 neighboring LTM candidate cell,

  - FR2 Pcell switch to a neighboring FR1 LTM candidate cell,

  - Pcell FR1 switch to an LTM candidate cell that is a FR1 serving SCell in MCG,

  - FR2 PCell switch to an LTM candidate cell that is a FR2 serving SCell in MCG.

Recommended WF:

- Discuss the options

#### Issue 4-3-2: CLTM test scope - intra/inter-frequency

Candidate options:

- Option 1: cover both intra and inter-frequency CLTM. (Nokia, Apple, vivo, ZTE, HW)

Recommended WF:

- Discuss the options

#### Issue 4-3-3: CLTM test scope – L1/L3 based CLTM

Candidate options:

- Option 1: cover both L1 and L3 based CLTM. (Nokia, CATT, ZTE)

- Option 2: RAN4 to discuss how to verify L1 and L3 measurement based candidate evaluation for CLTM such as whether to reuse partial LTM test procedure. (E///)

Recommended WF:

- Discuss the options

#### Issue 4-3-4: CLTM test scope – RACH-less and RACH-based CLTM

Candidate options:

- Option 1: cover both RACH-less and RACH based CLTM. (Nokia, CATT, vivo, HW)

- Option 2: RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure. (E///)

Recommended WF:

- Discuss the options

#### Issue 4-3-5: CLTM test scope – early DL sync

Candidate options:

- Option 1: cover both CLTM with and without early DL sync. (Nokia)

Recommended WF:

- Discuss the options

#### Issue 4-3-6: CLTM test scope – PCell/PSCell

Candidate options:

- Option 1: cover both PCell swith and PSCell switch. (ZTE)

Recommended WF:

- Discuss the options

#### Issue 4-3-7: subsequent CLTM test scope

Candidate options:

- Option 1: Apple

  - Intra-frequency subsequent CLTM from FR1 to FR1

  - Inter-frequency subsequent CLTM from FR2 to FR2

- Option 2: HW

  - RACH based Intra-frequency subsequent conditional PCell swith

  - RACH based Inter-frequency subsequent conditional PCell swith

  - RACH less Intra-frequency subsequent conditional PCell swith

  - RACH less Inter-frequency subsequent conditional PCell swith

Recommended WF:

- Discuss the options

#### Issue 4-3-8: others

Candidate options:

- Proposal 1: On high level, CLTM test cases to have serving cell and candidate cell, and the UE shall trigger cell switch based on signal level of one of these cells changing so that the cell switch condition becomes met. (Nokia)

- Proposal 2: Ensure in the test cases with early UL and DL synchronization that the UE has valid DL synchronization for the target TCI state and a valid TA at the time of condition becoming met. (Nokia)

- Proposal 3: for UE which can pass subsequent CLTM, test case for CLTM can be skipped. (Apple)

- Proposal 4: There is no need to define the new test cases for PDCCH ordered RACH for (subsequent) CLTM. (CATT, ZTE)

- Proposal 5: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases. (CATT)

Recommended WF:

- Discuss the options