3GPP TSG-RAN WG4 Meeting # 115	R4-2508314

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on Inter-RAT measurements E-UTRAN FDD for Rel-19 XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce requirements for Inter-RAT measurements for E-UTRAN FDD for Rel-19 XR MG skipping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce requirements for Inter-RAT measurements for E-UTRAN FDD for Rel-19 XR MG skipping. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 9.4.2	NR − E-UTRAN FDD measurements

#### 9.4.2.1	Introduction

The requirements are applicable for NR−E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN FDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in clause 10.2.2 are fulfilled for a corresponding band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [15],

-	RSRQ related conditions in the accuracy requirements in clause 10.2.3 are fulfilled for a corresponding band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [15],

-	RS-SINR related conditions in the accuracy requirements in clause 10.2.5 are fulfilled for a corresponding band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [15].

#### 9.4.2.2	Requirements when no DRX is used

When the UE requires measurement gaps or NCSG to identify and measure inter-RAT cells and an appropriate measurement gap pattern or NCSG is scheduled, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are scheduled, or an appropriate pre-MG is scheduled and activated, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable FDD cell within TIdentify, E-UTRAN FDD according to the following expression:

TIdentify,  E-UTRAN FDD=TBasicIdentify∙480TInter1∙CSSFinterRAT∙Kgap_EUTRA[+ Lcancel∙MGRP∙CSSFinterRAT ∙Kgap_EUTRA]      ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in clause 9.4.1,

CSSFinterRAT = CSSFwithin_gap,i when measurement gaps are configured, or CSSFwithin_ncsg,i when NCSGs are configured, is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in clause 9.1.5.2.

For a UE supporting and configured with concurrent GAPs or MUSIM gaps or both concurrent GAPs and MUSIM gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated GAP pattern. Kgap_EUTRA = 1 when the UE is not configured with concurrent GAPs nor MUSIM gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent GAPs.

For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met, Lcancel is the number of gap occasions cancelled during TIdentify,  E-UTRAN FDD for PSS/SSS detection. Lcancel = 0 for UE not supporting [FFS feature name] or the configuration and conditions in clause 9.1.y.4 are not met.

For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the MG pattern associated to the inter-RAT layer. Otherwise, MGRP is the Measurement Gap Repetition Period of the configured MG.

For a window W of duration xRP_max, where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and per-FR measurement GAPs for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated GAP occasions within the window, including those dropped and non-dropped occasions within the window, and

Navailable is the number of non-dropped associated GAP occasions after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for the GAP and MUSIM gap in clause 9.1.8.3 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements do not apply for UE configured with concurrent GAPs or MUSIM gaps, if Navailable =0

When UE supports musim-GapPreference-r17 and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target inter-RAT carrier is expected.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN FDD defined in table 9.4.2.2-1.

Table 9.4.2.2-1: Measurement period and measurement bandwidth

| Configuration | Physical Layer Measurement period: TMeasure, E-UTRAN FDD [ms] | Measurement bandwidth [RB] |
| --- | --- | --- |
| 0 | 480[+Lcancel x MGRP) ] x [CSSFinterRAT x Ceil(Kgap_EUTRA)] | 6 |
| 1 (Note 1) | 240[+Lcancel x MGRP) ] x [CSSFinterRAT x Ceil(Kgap_EUTRA)] | 50 |
| NOTE 1: This configuration is optional. NOTE 2: Kgap_EUTRA is only applicable for a UE supporting concurrent measurement gaps. Otherwise Kgap_EUTRA =1 NOTE 3: For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met, Lcancel is the number of gap occasions cancelled during Tmeasure, E-UTRAN FDD for measurement. Lcancel = 0 for UE not supporting [FFS feature name] or the configuration and conditions in clause 9.1.y.4 are not met. |  |  |

When measurement gaps are scheduled for E-UTRAN FDD inter-RAT measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting RSRP, RSRQ, and RS-SINR measurements to higher layers with measurement period TMeasure, E-UTRAN FDD given by table 9.4.2.2-1.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in clause 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in clause 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in clause 10.2.5.

#### 9.4.2.3	Requirements when DRX is used

When DRX is in use and an appropriate measurement gap pattern or NCSG is configured, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are configured, or an appropriate pre-MG is scheduled and activated, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in table 9.4.2.3-1. When highSpeedMeasFlag-r16 is configured and UE supports the enhanced inter-RAT E-UTRAN measurement requirements, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in table 9.4.2.3-2.

For a UE supporting and configured with concurrent GAPs, or MUSIM gaps or both concurrent GAPs and MUSIM gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated GAP pattern. Kgap_EUTRA = 1 when the UE is not configured with concurrent GAPs nor MUSIM gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent GAPs.

For a window W of duration xRP_max, where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gap(s), and per-FR GAPs for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated GAP occasions within the window, including both dropped and non-dropped instances of the associated GAP within the window, and

Navailable is the number of non-dropped associated GAP occasions after accounting for measurement gap and MUSIM gap collisions by applying the collision rules for the GAP and MUSIM gap in clauses 9.1.8.3 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements do not apply for UE configured with concurrent GAPs, or MUSIM gaps, if Navailable = 0

When UE supports musim-GapPreference-r17 and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target inter-RAT carrier is expected.

Table 9.4.2.3-1: Requirement to identify a newly detectable E-UTRAN FDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap/NCSG period = 40 ms, 20 ms | Gap/NCSG period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.2.2 apply | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.256 | (5.12[+Lcancel *0.256])* CSSFinterRAT x Ceil(Kgap_EUTRA) ((20[+Lcancel *0.256)]*CSSFinterRAT x Ceil(Kgap_EUTRA)) | (7.68[+Lcancel *0.256])* CSSFinterRAT x Ceil(Kgap_EUTRA) ((30[+Lcancel *0.256])*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32 | (6.4[+Lcancel *0.32])* CSSFinterRAT x Ceil(Kgap_EUTRA) ( (20[+Lcancel *0.32])*CSSFinterRAT x Ceil(Kgap_EUTRA)) | (7.68[+Lcancel *0.32])* CSSFinterRAT x Ceil(Kgap_EUTRA) ((24[+Lcancel *0.32])*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32< DRX-cycle ≤10.24 | Note1 ((20[+Lcancel *DRX-cycle])*CSSFinterRAT x Ceil(Kgap_EUTRA)) | Note1 ((20[+Lcancel *DRX-cycle])*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs. Otherwise Kgap_EUTRA = 1 NOTE 4: If multiple concurrent gaps are configured, the gap period is the periodicity of the MG pattern associated to the E-UTRA inter-RAT frequency layer. NOTE 5: For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met, Lcancel is the number of gap occasions cancelled during TIdentify, E-UTRAN FDD for PSS/SSS detection. Lcancel = 0 for UE not supporting [FFS feature name] or the configuration and conditions in clause 9.1.y.4 are not met. |  |  |

Table 9.4.2.3-2: Requirement to identify a newly detectable E-UTRAN FDD cell when highSpeedMeasFlag-r16 is configured

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap/NCSG period = 40 ms, 20 ms | Gap/NCSG period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.2.2 apply | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.16<DRx cycle≤0.32 | Note 1(15*CSSFinterRAT x Ceil((Kgap_EUTRA)) |  |
| 0.32<DRx cycle ≤ 0.64 | Note 1(10*CSSFinterRAT x Ceil(Kgap_EUTRA)) |  |
| DRX cycle = 1.024 | Note 1(10*CSSFinterRAT x Ceil( Kgap_EUTRA)) | Note 1(10*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| DRX cycle = 1.28 | Note 1(8*CSSFinterRAT x Ceil( Kgap_EUTRA)) | Note 1(8*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT x Ceil( Kgap_EUTRA)) | Note1 (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or interRAT-MeasurementEnhancement-r16. NOTE 4: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs. Otherwise Kgap_EUTRA = 1 NOTE 5: If multiple concurrent gaps are configured, the gap period is the periodicity of the MG pattern associated to the E-UTRA inter-RAT frequency layer. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN FDD cells per E-UTRA FDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA FDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN FDD specified in table 9.4.2.3-2.

Table 9.4.2.3-2: Requirement to measure E-UTRAN FDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN FDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.08< DRX-cycle ≤ 10.24 | Note1 ((5*[+ Lcancel * DRX-cycle)] CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs. Otherwise Kgap_EUTRA =1 NOTE 4: For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met, Lcancel is the number of gap occasions cancelled during Tmeasure, E-UTRAN FDD for measurement. Lcancel = 0 for UE not supporting [FFS feature name] or the configuration and conditions in clause 9.1.y.4 are not met. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in clause 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in clause 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in clause 10.2.5.

<End of Change 1>