3GPP TSG-WG4 Meeting #115	R4-2508413

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for requirements in signalling characteristics for (e)Redcap UEs with FR1-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-5-1 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R19, signaling characteristics for (e)RedCap UEs with FR1-NTN were introduced for RedCap UEs with FR1-NTN, including: - Active BWP switch delay for Redcap UEs with NTN - Active TCI state switching delay for Redcap UEs with NTN - UE-specific CBW change for Redcap UEs with NTN - Pathloss reference signal switching delay for Redcap UEs with NTN These corresponding mechanisms require the definition of relevant requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define corresponding requirements on: - Active BWP switch delay for Redcap UEs with NTN - Active TCI state switching delay for Redcap UEs with NTN - UE-specific CBW change for Redcap UEs with NTN - Pathloss reference signal switching delay for Redcap UEs with NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No corresponding requirements defined for signaling characteristics for (e)RedCap UEs with FR1-NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6X, 8.10X, 8.13X, 8.14X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2506553 |  |  |  |  |  |  |  |  |

### <Start of Change 1>

8.6X	Active BWP switch delay for RedCap UEs with satellite access

8.6X.1	Introduction

The requirements in this clause apply for a RedCap UE configured with only PCell, which is served by satellite access node (SAN). The requirements in this clause also apply for a UE configured with more than one BWP on PCell.

UE shall complete the switch of active DL and/or UL BWP within the delay defined in this clause.

8.6X.2	DCI and timer based BWP switch delay on a single CC

The requirements in clause 8.6C.2 shall apply.

8.6X.3	RRC based BWP switch delay on a single CC

The requirements in clause 8.6C.3 shall apply.

### <End of Change 1>

### <Start of Change 2>

8.10X	Active TCI state switching delay for RedCap UEs with satellite access

8.10X.1	Introduction

The requirements in this clause apply for a RedCap UE configured with one or more TCI state configurations on PCell in satellite access. UE shall complete the switch of active TCI state within the delay defined in this clause.

8.10X.2	MAC-CE based TCI state switch delay

The requirements in clause 8.10C.2 shall apply.

8.10X.4	DCI based TCI state switch delay

The requirements in clause 8.10C.4 shall apply.

8.10X.5	RRC based TCI state switch delay

The requirements in clause 8.10C.5 shall apply.

8.10X.6	Active TCI state list update delay

The requirements in clause 8.10C.6 shall apply.

### <End of Change 2>

### <Start of Change 3>

8.13X	UE-specific CBW change for RedCap UEs with satellite access

8.13X.1	Introduction

The requirements in this clause apply for a RedCap UE with NTN receiving reconfiguration of offsetToCarrier or carrierBandwidth to change channel bandwidth.

8.13X.2	UE-specific CBW change delay

The requirements in clause 8.13C.2 shall apply.

### <End of Change 3>

### <Start of Change 4>

8.14X	Pathloss reference signal switching delay for RedCap UEs with satellite access

8.14X.1	Introduction

The requirements in this clause apply for pathloss reference signal activated or updated on PCell in clause 7.1.1 in TS 38.213 [3] and the RedCap UE is configured with only PCell, which is served by satellite access node (SAN).

UE shall complete the switch of pathloss reference signal within the delay defined in this clause.

8.14X.2	Known conditions for pathloss reference signal

The requirements in clause 8.14C.2 shall apply.

8.14X.3	MAC-CE based pathloss reference signal switch delay

The requirements in clause 8.14C.3 shall apply.

### <End of Change 4>