# 3GPP TSG-RAN WG4 Meeting #115	R4-2508434

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.7.4

Source:	Apple

Title:	TP to TR38.768 on RRM requirements for low band aggregation via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

This contribution provides a text proposal to TR38.768[1] with two changes: (1)create sub-clause structure based on  RRM requirement discussion aspects, and (2)add agreements based on agreed WF in [2][3].

# 3	References

[1]	R4-2505146, “TR38.768 v0.0.3 for low band aggregation via switching,” Apple, RAN4 #114bis

[2]	R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #114

[3]	R4-2504895,	 WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #114bis

[4]	R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #115

# 2	Text Proposal

<< begin change 1 >>

# 6	RRM aspects

<Editor’s note: capture RRM aspects in this clause>

## 6.1	General

RAN4 has reached the following agreements on the switching pattern applicability for RRM requirement in R19 LB CA via switching in RAN4#114bis meeting:

For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133),

- The switching pattern for SDL SCell is applied.

For the SCell which is in activation procedure,

- The switching pattern for SDL SCell is not applied.

- RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation, where the RS occasions are FFS

For the SCell which is deactivated,

- The switching pattern for SDL SCell is not applied.

- If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirement for deactivated SDL SCell measurement.

Note: the above FFS parts will be discussed and decided in RAN4.

The necessity of requirement for SDL SCC was agreed in RAN4 #115 meeting, as summarized in table 1.

Table 1. Necessity of requirement on SDL CC

| Requirement | Whether or not RAN4 will have requirement |
| --- | --- |
| interruption requirement for CC switching | No (can be revisited if not aligned with RAN1 further agreement.) |
| Activated SDL SCell L3 measurement | Yes |
| Deactivated SDL SCell measurement [and corresponding interruption] | Yes |
| SDL SCell activation/deactivation and corresponding interruption | Yes |
| [BFD/CBD] | Yes |
| TCI state switch delay | Yes |
| PL-RS switch delay | No |
| L1-RSRP/SINR | Yes |

Following SSB assumptions for R19 LB CA via switching are agreed in RAN4 #115:

  - The baseline assumption is:

- SDL SCell is transmitting SSB

      - -	RAN4 to use SSB based requirement as baseline for Rel-19 low band CA via switching

      - -	requirement of PCell will still use the existing principle as baseline but the impact of carrier switching in LB CA will be considered

Following MRTD assumptions for R19 LB CA via switching is agreed in RAN4 #115:

- The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

- Whether or not above RTD can be used as side condition for SCell activation will be further discussed.

## 6.2	Discussion of switching delay or interruption requirement

## It was agreed in RAN4 #115 meeting that, for SDL to FDD switching and FDD to SDL switching, RAN4 will not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

## Note: the above agreement can be revisited if not aligned with RAN1 further agreement.

## 6.3	Discussion of SDL SCell related requirement

#### 6.3.1	Activated SDL SCell L3 measurement requirement

RAN4 #115 agreed to define the requirement for activated SDL SCell L3 measurement. For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern.

#### 6.3.2	Deactivated SDL SCell L3 measurement requirement

RAN4 #115 agreed to define the requirement for deactivated SDL SCell L3 measurement. The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement.

#### 6.3.3	SDL SCell activation/deactivation requirement

RAN4 #115 agreed that PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI. The side condition for SDL SCell activation/deactivation requirement follows the legacy case and side conditions for SSB based inter-band FR1 SCell activation in existing spec.

The following principle design for SDL SCell activation/deactivation delay requirement was agreed in RAN4#115:

- The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature.

  - R16 Direct SCell activation with HO is not considered for R19 LB CA.

  - R17 PUCCH SCell activation is not considered for R19 LB CA.

  - R18 SSB-less SCell activation for NES is not considered for R19 LB CA.

  - R17 Fast SCell activation is not considered for R19 LB CA.

- Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation.

- Known/unknown conditions are reused from existing SCell activation requirements

#### 6.3.4	SDL SCell L1 requirement

If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

RAN4 will not define requirement for PL-RS switch delay for SDL SCell in LB CA via switching

For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

## 6.4	Discussion of impact on FDD PCell RRM

RAN4#115 meeting agreed that the existing Tx timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) can apply for R19 LB CA via switching.

<< end change 1 >>