3GPP TSG-RAN WG4 Meeting #115														R4-2508435

# Malta, MT, 19th - 23rd May, 2025

Agenda item:			7.12.1

Source:	Moderator (CMCC)

Title:	 Coffee break discussion minutes on NR_ATG_enh

Document for:	Information

Issue 1-1-1: UE Beam Type capability signaling

Further check with following:

Tentative agreement:

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx beam Type | Indicates whether the UE supports one common Rx beam or two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | Per BC | No | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table [TBD] in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination |

Issue 1-2-1: Interruption requirements

- Proposals

  - Proposal 1: (LGE, CMCC, HW)

    - The existing interruption requirements caused by SCell addition / release and SCell activation / deactivation can be reused for ATG CA depending on intra-band CA and inter-band CA (i.e., no need to consider different requirement for common Rx beam ATG UE in inter-band CA).

  - Proposal 2: UE antenna types have impact on some types of interruption requirements. (ZTE, Ericsson)

    - Proposal 2-1: (ZTE)

      - UE antenna types have impact on some types of interruption requirements. Classify the UE antenna types into two categories:

        - Category 1: antenna array assumption in intra-band CA and Case 4 in inter-band CA

        - Category 2: other cases

      - For the interruptions caused by SCell addition/release and any types of SCell activation/deactivation, distinguish the interruption requirements for Category 1 and 2.

      - For other interruptions, define unified interruption requirements for all UE antenna types.

    - Proposal 2-2: (Ericsson)

      - For clauses 8.2.2.2.1, 8.2.2.2.2, 8.2.2.2.5, and 8.2.2.2.17 distinguish the interruption requirements for Category A and B.

        - Category A: Common Rx beam assumption (e.g., co-located beams with aligned timing and shared spatial domain)

        - Category B: Separate Rx beam assumption (e.g., independent beams with distinct directionality and control)

- Recommended WF

  - To be discussed

ZTE: Check with companies, SCell addition/.. requirement, FR2 has two category requirement, including intra-band and inter-band IBM case. Separate RF chains and phase shifter can lead to shorter interruption. But for Case 4 UE, separate RF chain and common phase shifter may be used, so will common phase shifter will cause longer interruption?

LGE: Use shorter interruption for Case#4, separate RF chain can guarantee the shorter interruption side condition.

CMCC: Similar understanding as LGE

HW: Similar as LGE

ZTE: Even single phase shifter, consider co-located scenario, one common beam is enough, no need to adjust phase shifter.

Tentative agreement:

- The existing interruption requirements caused by SCell addition / release and SCell activation / deactivation can be reused for ATG CA depending on intra-band CA and inter-band CA (i.e., no need to consider different requirement for common Rx beam ATG UE in inter-band CA).

Issue 1-2-2: SCell activation delay enhancement

- Proposals

  - Proposal 1: Reuse the legacy SCell activation delay requirement for ATG (CMCC, ZTE, LGE, CATT)

  - Proposal 2: (Ericsson)

    - In Known SCell with common Rx beam, the Tactivation_time time can be 3ms

    - If the target SCell is unknown with common Rx beam, Tactivation_time is 3ms+TFirstSSB+ 2ms.

- Recommended WF

  - To be discussed

E///: With common Rx beam, UE can skip the Beam sweeping, sync and AGC

CMCC: Fine sync is needed.

ZTE: Share similar view with CMCC, 3ms need many side conditions.

E///: Does that means same value for both Case 4 and Case 5?

ZTE: Reuse FR1.

Tentative agreement:

Reuse the legacy SCell activation delay requirement for ATG

Issue 1-2-3: Active BWP switch delay of single CC requirement

- Proposals

  - Proposal 1: Introduce SCell dormancy related impact for active BWP switch delay of single CC requirement. The legacy TN requirement can be reused. (CMCC)

- Recommended WF

  - To be discussed

- QC: Is SCell dormancy is typical case for ATG? What means of single CC?

Tentative agreement:

- Introduce SCell dormancy related impact for active BWP switch delay of single CC requirement. The legacy TN requirement can be reused.

- Can be revert if technical issue found.

Issue 1-3-1: Sharing factor between L1 measurement and RRM measurement - Condition for Klayer1_measurement=1 and definition format

  - Background: Agreement in RAN4#114

- The Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example): For UE with the antenna array, - Klayer1_measurement=1, - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement; - Klayer1_measurement=1.5, otherwise. For ATG UE with the antenna array, - P value for SSB resource to be measured is defined as - Ntotal / Navailable with Navailable > 0 - Psharing factor * Ntotal / Noutside_MG with Navailable = 0 - For a window W of duration max(TL1, MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion: - Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, or not overlapped with any measurement gap occasion nor any SMTC occasion [configured] by same serving cell within the window W when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4]. - TL1 is periodicity of the target SSB - Psharing factor = 3.

  - Proposal 1: (HW)

    - Klayer1_measurement for ATG UE with common Rx beam in inter-band CA could be defined as:

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of PCC or SCC, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap and fully-overlapped by intra-frequency SMTC occasions of PCC or SCC are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

  - Proposal 2: (ZTE)

    - For UE capable of [antennaArrayType-r18] and [intra-band or inter-band carrier aggregation is configured],

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in different band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbols before each consecutive SSB symbols and the RSSI symbols, and 2 symbols after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured and the UE is capable of [common Rx beam between PCC band and SCC band], where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

- if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting are on any serving frequency in different band outside measurement gap and UE is not capable of [common Rx beam between PCC band and SCC band].

Otherwise, Klayer1_measurement=1.5.

  - Proposal 3: (CATT)

    - RAN4 to follow the legacy method for defining the sharing factor and only to specify the case for Klayer1_measurement=1 and otherwise Klayer1_measurement=1.5.

      - Klayer1_measurement=1,

        - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

        - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or

        - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or

        - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

      - Klayer1_measurement=1.5, otherwise.

- Recommended WF

  - Firstly discuss ‘no configured SSB-ToMeasure and SS-RSSI-Measurement’ case, and apply same format to ‘configured SSB-ToMeasure and SS-RSSI-Measurement case’ after drawing the conclusion of Issue 1-3-2

  - Moderator summary the proposals from companies as:

  - For UE capable of [antennaArrayType-r18] on the measured carrier and single carrier case, Klayer1_measurement=1,

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

  - For UE capable of [antennaArrayType-r18] on the measured carrier and intra-band CA case and inter-band CA case#4 case, Klayer1_measurement=1,

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on PCC and SCC/on any serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

  - For UE capable of [antennaArrayType-r18] on the measured carrier and inter-band CA except for case#4 case, Klayer1_measurement=1,

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of same serving cell

  - Could the first two cases merged with the condition ‘on any serving frequency’?

  - If Yes, could the three cases merged as following?

    - For UE capable of [antennaArrayType-r18] on the measured carrier

      - Klayer1_measurement=1,

        - If inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4]

          - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of same serving cell

        - Otherwise,

          - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

HW: Is common understanding, common Rx beam means same beam for both PCC and SCC, and L1 and L3 measurements?

ZTE: If we check FR2 requirement, we can find the PCC and SCC for L1 and L3 only for the same band

HW: With one beam on PCC and SCC.

ZTE： Fine with suggestion, if we follow this structure, we will combine R18 and R19. Another solution is separate R18 and R19

HW: Remove the highlight part of the otherwise paragraph

LGE: clarify the otherwise (single carrier, intra-band CA, inter-band CA Case#4)

Option 1: merge

Option 2: separate

Agreement:

    - For UE capable of [antennaArrayType-r18] on the measured carrier

      - Klayer1_measurement=1,

        - If inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4]

          - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of same serving cell

        - Otherwise,

          - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement - effective non-overlapping duration

- Proposals

  - Proposal 1: For the ‘SSB-ToMeasure and SS-RSSI-Measurement are configured’, use MRTD as the reference when calculate the margin for non-overlapping duration (LGE, ZTE)

    - Proposal 1-1: (LGE)

      - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

        - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

    - Proposal 1-2: (ZTE)

      - For UE capable of [antennaArrayType-r18] and [intra-band or inter-band carrier aggregation is configured],

        - Klayer1_measurement=1,

          - -	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

          - -	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in different band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbols before each consecutive SSB symbols and the RSSI symbols, and 2 symbols after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured and the UE is capable of [common Rx beam between PCC band and SCC band], where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

  - Proposal 2: (CMCC)

    - Firstly align the understanding about the reference timing of SSB symbols and the RSSI symbols in the definition of Klayer1_measurement.

      - If the serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then extend the margin considering the maximum propagation delay difference between serving cell and neighbour cells.

      - If the neighbour cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then reuse the legacy 1 symbol margin

  - Proposal 3: (Ericsson)

    - reuse the legacy specification, i.e.

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

- Recommended WF

  - To be discussed

  - Some background for information

    - If SSB-ToMeasure and SS-RSSI-Measurement are configured, then network/UE knows the coarse SSB/RSSI position which need be measured for L3 (both serving and neighbour). Then when reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, Klayer1 can be 1.

    - In legacy, the coarse SSB/RSSI position which need be measured for L3 (both serving and neighbour) includes the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols.

      - Based on moderator’s understanding, the reference timing of the SSB symbols and the RSSI symbols is the serving cell, and the propagation delay difference and sync among cells are included in the 1 symbol before and after.

      - For ATG, the ISD can be up to 300km, the 1 symbol margin may not enough for the propagation delay difference between serving and neighbour

Issue 1-3-3 Measurement enhancement for ATG UE with antenna array

- Background: WF in RAN4#114bis

Candidate options: - UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control. - Option 1: this can already be achieved by the existing configuration. - Option 2: Consider enhancement for Rel-19 ATG CA

- Proposals

  - Proposal 1: Introduce a network signaling to indicate that the UE is not required to measure the neighbour cell on the configured MO for SCC. LS shall be sent to RAN2 for signaling design (CMCC)

    - The existing network configuration can not achieve the functionality that UE only skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell}

    - From requirement point of view, if UE skip the neighbour cell measurement of SCC and/or CC with deactivated SCell, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed, the scheduling restriction caused by antenna array will not be applied

  - Proposal 2: UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control, which has already be achieved by the existing configuration. (CATT, HW)

    - Proposal 2-1: (CATT)

      - The existing network can explicitly delete or disable measurement objects related to neighbour cells on SCC through RRC configuration, or filter neighbour cells on SCC through limitations on measurement object parameters (e.g. allowedPCI-List or blackPCI-List), or delete related measurement task via dedicated signaling

  - Proposal 3: RAN4 should not support Measurement enhancement for ATG UE with antenna array.   (Ericsson)

- Recommended WF

  - To be discussed

  - UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control. LS will sent to RAN2 about the signaling assistance due to this can’t be achieved by the existing configuration

Ericsson: Still have concern. Only applicable to PCell quality is good enough.

HW: Prefer network indication than RSRP condition.

ZTE: Based on previous discussion, the enhancement shall be fully controlled by network. The basic intention is throughput enhancement.

HW: Just network indication, RAN2 design the signaling.

E///: Request more time.

ZTE: Whether should be RRC based or MAC-CE based? Dynamic indication may be needed?

Option 1: Network indication, after indication, UE can skip the neighbouring cell measurement

Option 2:  Network indication and RSRP condition, if fulfilled by UE, UE can skip the neighbouring cell measurement

Tentative agreement:

UE shall skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network indication.

FFS RSRP condition is needed or not

FFS RRC and/or MAC-CE level

Issue 2-1-1: Test applicability rule

- Proposals

  - Proposal 1: (LGE, CMCC, ZTE)

    - UE is only required to be tested in one of the supported test configurations

    - UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

  - Proposal 2: (CMCC)

    - Test configuration for NR PCell and test configuration for NR SCell shall be chosen independently

  - Proposal 3: (HW)

    - UE is required to be tested in one of the supported test configurations randomly

- Recommended WF

  - Test configuration for NR PCell and test configuration for NR SCell shall be chosen independently

  - UE is only required to be tested in one of the supported test configurations

  - To be discuss:

    - Option 1: UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

    - Option 2: Random method

Issue 2-1-2: Test setups - TDD pattern for SCell

- Proposals

  - Proposal 1: Reuse the legacy TDD pattern (CATT, LGE, CMCC, ZTE)

    - Proposal 1-1: (CATT)

      - For intra-band CA, TDD pattern for SCell need to be the same with PCell.

      - For inter-band CA, the pattern used in R18 ATG test cases can also be reused.

    - Proposal 1-2: (ZTE)

      - For TDD case, the TDD pattern for SCell could be same as PCell (ZTE)

  - Proposal 2: RAN4 to discuss whether or not to define a new TDD pattern for ATG (HW)

  - Proposal 3: Define all ‘D’ slot TDD pattern for ATG (Ericsson)

- Recommended WF

  - Check whether Proposal 1 can be agreed.

Issue 2-1-3: Test method and scaling factor for UE with antenna array

- Proposals

  - Proposal 1: (CATT, CMCC, HW, ZTE, Ericsson)

    - RAN4 to apply scaling factor = 1 for antenna array related test requirements and use conductive test method.

  - Proposal 2: (LGE)

    - RAN4 not to consider separate test cases for ATG UE to support common Rx beam

  - Proposal 3: (Ericsson)

    - Test parameters, test requirements for UE with antenna array on both PCC and SCC can reuse the NR-CA baseline test parameters and requirements

- Recommended WF

  - Use conductive test method, apply scaling factor (including N1, Klayer1 and P) = 1 for antenna array related test requirements

  - Not to consider separate test cases for ATG UE to support common Rx beam