3GPP TSG-RAN4 Meeting #115	R4-2508458

# Malta, MT, 19th - 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft Big CR on RRM requirements of NR_MIMO_Ph5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-05-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This draft Big CR includes the following endorsed CRs in RAN4#115 meeting for RRM requirements of NR_MIMO_Ph5. - R4-2508388, Draft CR on event trigger Reporting based on L1-RSRP measurements for serving cell, Samsung - R4-2508389, Draft CR on Event triggered L1 reporting - SSB from different PCI, Apple - R4-2508452, DraftCR on timing requirements for asymmetric DL sTRP/UL mTRP deployment, CMCC - R4-2508455, Draft CR on MRTD for asymmetric DL sTRP&UL mTRP deployment of R19 MIMO, ZTE Corporation, Sanechips - R4-2508404, Draft CR MTTD requirements for UL only TRP, Nokia |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRM requirements for NR_MIMO_Ph5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements for NR_MIMO_Ph5 are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.3.X 9.13.3.X 7.1 7.6 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ---Start Change #1 R4-2508388-------------------------

9.5.3.X	Event Triggered Reporting [for the UE initiated beam management]

For a UE configured with a CSI-ReportConfig with eventType-r19 and and with dl-OrJointTCI-StateList，reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than the maximum of L1-RSRP measurement periods on the beams corresponding to the event plus Tfirst_UL_channel.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period.

If the newBeamResourceSet-r19  is based on SSB, or if the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is based on SSB:

- the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam.

- the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

If the reference signal in the indicated TCI state and newBeamResourceSet-r19 are all based on CSI-RS:

- the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-2 assuming TReport. = 0, and TCSI-RS is the periodicity of the reference signal in the indicated TCI state for the current beam.

- the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-2 assuming TReport. = 0, and TCSI-RS is the periodicity of the the reference signal configured by the newBeamResourceSet-r19 for new beams.

[Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH]

Editor’s note: FFS on Event-1 and event-7.

---End Change #1-------------------------

# ---Start Change #2 R4-2508389-------------------------

9.13.3.X	Event triggered reporting [for UE initiated beam management]

For a UE configured with a CSI-ReportConfig with eventType-r19 and and with dl-OrJointTCI-StateList，reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clause 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report after condition is met over the air and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than the maximum of L1-RSRP measurement periods on the beams corresponding to the event plus Tfirst_UL_channel.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, UE shall perform L1-RSRP measurement based on current beam and new beam’s L1-RSRP measurement period.

- If the SS/PBCH block is QCLed with the reference signal in the indicated TCI state, the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB_CDP with TReport. = 0, and TSSB_CDP is the periodicity of the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam

- If the reference signals in newBeamResourceSet-r19 are based on SSB for a cell with additional PCI, the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB_CDP with TReport. = 0, and TSSB_CDP is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

[Tfirst UL channel is the time to first PUCCH after event triggered condition is realised at the UE].

Editor’s note: FFS on Event-1 and event-7.

---End Change #2-------------------------

# ---Start Change #3 R4-2508452-------------------------

7	Timing

7.1	UE transmit timing

7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state or when transmitting PUSCH on CG resources for SDT in RRC_INACTIVE. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

In the requirements of clause 7.1.2, the term reference cell on a carrier frequency subject to CCA is not available at the UE refers to when at least one SSB is configured by gNB, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available during at least one discovery burst transmission window, at the UE due to DL CCA failures at gNB during the last 1280 ms; otherwise the reference cell on the carrier frequency subject to CCA is considered as available at the UE.

For UE supporting rach-EarlyTA-Measurement-r18 for LTM, and if the candidate cell is a neighbour cell or a secondary serving cell without uplink carrier,

-	UE shall have capability to follow the frame timing of the reference cell. The PRACH transmission for early TA acquisition takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE shall use this candidate cell as the reference cell for deriving transmit timing. UE initial transmit timing accuracy is defined in the following requirements.

For multi-DCI based multi-TRP operation with two TAs and for asymmetric DL sTRP/UL mTRP deployment with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point.

For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState TS 38.331 [2] having the same TAG as the uplink signal.

For  asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState or [UL TCI states] TS 38.331 [2] having the same TAG as the uplink signal.

Editor note:  For terminology of asymmetric DL sTRP/UL mTRP deployment with two TAs, they will be updated based on UE capability name further.

For multi-DCI based multi-TRP operation with two TAs if the UE is not provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1.

For multi-DCI based multi-TRP operation with two TAs if the UE is provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or [UL] TCI state(s) of the reference cell associated with the second TAG.

Editor note:  For terminology of asymmetric DL sTRP/UL mTRP deployment with two TAs, they will be updated based on UE capability name further.

Where NTA and NTA offset are commanded by the network independently for each TAG TS 38.331 [2]. NTA offset for first TAG and second TAG are as defined in n-TimingAdvanceOffset and n-TimingAdvanceOffset2 respectively, if configured, otherwise it is NTA offset defined in table 7.1.2-2.

In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports ncd-SSB-BWP-Wor-r18.

7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission, or it is the first transmission sent on the PSCell for activating the deactivated SCG without RACH.

-	when it is the transmission for PUSCH on CG resources for SDT in RRC_INACTIVE.

-	when it is the first transmission on target cell after UE receives LTM cell switch command.

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc. The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna. NTA for PRACH is defined as 0.

NTA+NTA offset×Tc (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value of NTA offset depends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). NTA offset is defined in table 7.1.2-2.

For multi-DCI based multi-TRP operation with two TAs and for asymmetric DL sTRP/UL mTRP deployment with two TAs for first transmission in a DRX cycle for PUCCH, PUSCH, SRS and PRACH transmission triggered by PDCCH order, UE initial transmission timing error requirements specified in this clause is applicable for each TAG and shall be met for each TAG separately. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

Editor note:  For terminology of asymmetric DL sTRP/UL mTRP deployment with two TAs, they will be updated based on UE capability name further.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  |  | 480 | 1.58*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  |  | 480 | 1.35*64*Tc |
|  |  | 960 | 0.90*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |
|  |  | 480 | 1.13*64*Tc |
|  |  | 960 | 0.86*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

Table 7.1.2-2: The Value of NTA offset

| Frequency range and band of cell used for uplink transmission | NTA offset (Unit: TC) |
| --- | --- |
| FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 0 (Note 1) |
| FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies NTA offset based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of NTA offset is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of NTA offset can also be provided for an FDD serving cell. Note 2: Void |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied.

Table 7.1.2-3: void

If the UE uses a reference cell on a carrier frequency subject to CCA for deriving the UE transmit timing, then the UE shall meet all the transmit timing requirements defined in clause 7.1.2 provided that the reference cell is available at the UE. If the reference cell is not available at the UE on a carrier frequency subject to CCA, then the UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise the UE shall not transmit any uplink signal.

If a reference cell on a carrier frequency belonging to the pTAG, which is subject to CCA, is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in pTAG as a new reference cell. If the SCell used as reference cell is deactivated, or becomes not available, the UE is allowed to use another active serving cell in pTAG as new reference cell.

If a reference cell on a carrier frequency belonging to the sTAG, which is subject to CCA is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in sTAG as a new reference cell.

7.1.2.1	Gradual timing adjustment

Requirements in this section shall apply regardless of whether the reference cell is on a carrier frequency subject to CCA or not.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE shall adjust the timing such that timing error is to within Te. The reference timing shall be NTA+NTA offset×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms for SCS of UL signals smaller or equal to 120 kHz and 100 ms for SCS of uplink signals larger or equal to 480 kHz.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in table 7.1.2.1-1.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

Editor note:  FFS for the scenario that UE is configured with PL offset in joint/UL TCI state(s) and UE does not expect to receive SSB from UL TRP(s). For terminology of asymmetric DL sTRP/UL mTRP deployment with two TAs, they will be updated based on UE capability name further.

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2-1 | 60 | K*64*Tc | 2.5*64*Tc |
|  | 120 | K*64*Tc | 2.5*64*Tc |
| 2-2 | 120 | 2.5*64*Tc | 2.5*64*Tc |
|  | 480 | 0.8*64*Tc | 0.8*64*Tc |
|  | 960 | 0.8*64*Tc | 0.8*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: When highSpeedMeasFlagFR2-r17 is configured for UE supporting power class 6, K = 4.5; otherwise, K = 2.5. |  |  |  |

7.1.2.2	Void

Table 7.1.2.2-1: Void

---End Change #3-------------------------

# ---Start Change #4 R4-2508455-------------------------

7.6	Maximum Receive Timing Difference

7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of an E-UTRA cell belonging to the MCG and the closest slot timing boundary of a cell belonging to SCG to be aggregated for EN-DC operation.

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of an E-UTRA cell belonging to the SCG to be aggregated for NE-DC operation and the closest slot timing boundary of a cell belonging to MCG.

A UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG in FR1 or FR2-1 and the closest slot timing boundary of a cell belonging to the SCG FR1 or FR2-1 to be aggregated for NR-DC operation.

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of a cell belonging to MCG in FR1 and the closest subframe timing boundary of a cell belonging to the SCG in FR2-2 to be aggregated for NR-DC operation.

A UE shall be capable of handling a relative receive timing difference among the closest slot timing boundaries of different carriers in FR1 and/or FR2-1 to be aggregated in NR carrier aggregation.

A UE shall be capable of handling a relative receive timing difference among the closest subframe timing boundaries of different carriers to be aggregated in FR1 and FR2-2 NR inter-band carrier aggregation.

An FR2-1 PC6 UE supporting simultaneousReceptionTwoQCL-r18 shall be capable of handling a relative receive timing difference between the subframe boundaries of signals on the same CC received using two different Rx chains simultaneously in HST FR2 bidirectional deployment.

A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling a relative receive timing difference between the slot boundaries of signals on the same CC received from two TRPs.

A UE supporting [FG 30-1 or 30-2] shall be capable of handling a relative receive timing difference among the subframe timing boundary of different TCI states indicated for simultaneous reception on a single carrier.

The requirements defined in clause 7.6 are also applicable when UE is configured to receive multiple PDSCH transmission occasions from one or more QCL sources on any one of the aggregated NR carriers.

[The requirements defined in clause 7.6 are also applicable for the UE capable of [singleDCI multi TRP with 2 TA], provided the UE is configured with 2 TAs and not configured with PL offset in joint/UL TCI state(s). In this scenario, when UE is configured with SSB-MTC-additionalPCI, the UE may expect to receive SSB from UL TRP(s).]

---

Unchanged text omit

---

7.6.8	Minimum requirements for Multi-TRPs

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in table 7.6.8-1 below, if the UE does not indicate support of rxTimingDiff-r18.

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18  shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in table 7.6.8-2 below, provided that the UE indicates that it is capable of rxTimingDiff-r18.

[A UE supporting [singleDCI multi TRP with 2 TA] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in table 7.6.8-4 below, if the UE does not indicate support of [TimingDiffwithSSBRxfromULTRP-R19], and in Table 7.6.8-5, if the UE indicates that it is capable of [TimingDiffwithSSBRxfromULTRP-R19].]

Table 7.6.8-1: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of rxTimingDiff-r18

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR1 | CP lengthnote 1 |
| FR2-1 | CP lengthnote 1 |
| NOTE 1: CP length of the maximum SCS on the carrier. |  |

Table 7.6.8-2: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of rxTimingDiff-r18

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 |

A UE supporting [FG 30-1 or fastBeamSweepingMultiRx-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different TCI states on the same carrier at the UE receiver as shown in table 7.6.8-3.

Table 7.6.8-3: Maximum receive timing difference requirement for UE supporting multi-RX

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR2-1 | CP length |

[

Table 7.6.8-4: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of [TimingDiffwithSSBRxfromULTRP-R19]

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR1 | CP lengthnote 1 |
| FR2-1 | CP lengthnote 1 |
| NOTE 1: CP length of the maximum SCS on the carrier. |  |

Table 7.6.8-5: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of [TimingDiffwithSSBRxfromULTRP-R19]

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 |

]

---End Change #4-------------------------

# ---Start Change #5 R4-2508404-------------------------

7.5.7	Minimum requirements for multi-TRP

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative transmission timing difference between slot timing of a pair of TAGs configured on the same carrier as shown in tables 7.5.7-1, if the UE does not indicate support of rxTimingDiff-r18.

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative transmission timing difference between slot timing of the pair of TAGs configured on the same carrier as shown in table 7.5.7-2, provided that the UE indicates that it is capable of rxTimingDiff-r18.

[A UE supporting [singleDCI multi TRP with 2 TA] shall be capable of handling at least a relative transmission timing difference between slot timing of a pair of TAGs configured on the same carrier with single DCI as shown in Table 7.5.7-3, if the UE does not indicate support of [FFS feature name], and in Table 7.5.7-4, if the UE indicates that it is capable of [FFS feature name].]

Table 7.5.7-1: Maximum uplink transmission timing difference requirement for multi-TRP for UE not supporting rxTimingDiff-r18

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | CP lengthnote 1+1.6us |
| FR2-1 | CP lengthnote 1+0.5us |
| NOTE 1: CP length of the maximum SCS on the carrier. NOTE 2: This requirement applies to the UE capable of [STxMP]. |  |

Table 7.5.7-2: Maximum uplink transmission timing difference requirement for multi-TRP for UE supporting rxTimingDiff-r18

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 |

[

Table 7.5.7-3: Maximum uplink transmission timing difference requirement for multi-TRP with single DCI for UE not supporting [FFS feature name]

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | CP lengthnote 1+1.6us |
| FR2-1 | CP lengthnote 1+0.5us |
| NOTE 1: CP length of the maximum SCS on the carrier. |  |

Table 7.5.7-4: Maximum uplink transmission timing difference requirement for multi-TRP with single DCI for UE supporting [FFS feature name]

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 |

]

---End Change #5-------------------------