3GPP TSG-RAN WG4 Meeting #115	                                                      R4-2508607

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.14.4.2

Source:	Xiaomi

Title:	Channel model validation results for Band 3

Document for:	Discussion

1	Introduction

In the Rel-19 TRP/TRS and MIMO OTA WID [1], bands n3, n7, n8, n77 are listed as first priority for FR1 MIMO OTA performance requirements. In this paper, we provide the channel model validation results for band n3.

2	Discussion

The channel model validation results are provided in Figs. 1-3 and Table 2-3. The measurement setup, test procedures, and instrument settings are according to TS 38.151. The measurement setup is composed by GTS5000 16-probe MIMO OTA chamber and Spirent Vertex channel emulator.

The following parameters of CDL-C UMi channel model have been validated:

Table 1. The validated CDL-C UMi channel model parameters

| Frequency | Band | Parameter |
| --- | --- | --- |
| 1880MHz | n3 | Temporal correlation |
|  |  | PDP |
|  |  | Spatial correlation |
|  |  | Cross-polarization |
|  |  | Power Validation |

Figure 1: Temporal correlation between measurement results and reference curves for CDL-C UMa for band n3

Figure 2: PDP measurement results and reference values for CDL-C UMa for band n3 Beam1

Figure 3: PDP measurement results and reference values for CDL-C UMa for band n3 Beam2

Figure 4: Spatial correlation measurement results and reference curves for CDL-C UMa, Band n3

Table 2. Cross-polarization verification results with CDL-C Uma for bands n3

| Frequency | Beam | Reference | Measurement result | Pass/fail limit |
| --- | --- | --- | --- | --- |
| fc = 1880 MHz | Beam 1 | Input 1+2: V/H = 0 dB | 0.52 dB | ±1dB |
| fc = 1880 MHz | Beam 2 | Input 3+4: V/H = 0 dB | 0.60 dB | ±1dB |

Table 3. Power verification results for CDL-C UMa bands n3

| Frequency | Measured Power | Target Power | Delta | Pass/fail limit |
| --- | --- | --- | --- | --- |
| fc = 1880 MHz | -47.94 | -48.06 | -0.12 dB | ±1.5dB |

Observation 1: The channel model validation for Bands n3 has been validated including TCF, SCF, PDP, Cross-polarization. It can be observed that the measurement results match well with the reference ones, meeting the pass/fail limits requirements.

3	Conclusion

In this paper, we provide part of channel model validation results for band n3.

Observation 1: The channel model validation for Bands n3 has been validated including TCF, SCF, PDP, Cross-polarization. It can be observed that the measurement results match well with the reference ones, meeting the pass/fail limits requirements.

References

- RP-250197,	 WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, 	vivo, CAICT.