3GPP TSG-RAN WG4 Meeting #115	                                                   R4-2508617

Malta, May 19 - 23 2025

Source:	China Telecom

Title:		Ad-hoc meeting minutes for BS MMSE-IRC requirements

Agenda Item:	7.17.1

Document for:	Information

# BS MMSE-IRC requirements

Issue 1-1-2: MMSE-IRC receiver assumption - estimation of interference covariance matrix

- Previous agreement in the WF R4-2413570:

The MMSE-IRC receiver weight matrix is usually defined as where and denote the estimated channel matrix and covariance matrix, respectively. To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations: , , where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs. The estimation of interference covariance matrix is performed at per PRB and per TTI basis.

- Proposals:

  - Option 1: No need to modify the agreed MMSE-IRC receiver assumption. (China Telecom, Ericsson, Nokia)

  - Option 2: Revise the baseline assumption for MMSE-IRC as the estimation of interference covariance matrix is performed based on DM-RS symbol per PRB and per TTI basis, i.e., the interference covariance matrix is the average of both DMRS RE and non DMRS RE in the DMRS symbol (Samsung, China Telecom, Huawei, ZTE)

- Online consensus:

  - The MMSE-IRC receiver assumption is not going to be captured in the RAN4 spec

- Companies ‘simulation result comparison between option2 and option 1:

Samsung:

| umber of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | INR Set | SNR with DMRS power boosting | SNR with DMRS power boosting |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | DMRS based estimation (dB) | DMRS + non DMRS RE based estimation (dB) |
|  |  | Tested signal(mcs16) | Interferer(16QAM) |  |  |  |
| 1 | 2 | TDLC300-100 Low | TDLC300-100 Low | HomNet set1: (-1.11. N/A) | 12.24 | 11.89 |
|  |  | TDLA30-10 Low |  | HetNet set2: (-0.43. NA/) | 12.9 | 12.38 |
|  | 4 | TDLC300-100 Low |  | HomNet set1: (-1.11. -10.91) | 8.86 | 8.31 |
|  |  | TDLA30-10 Low |  | HetNet set2: (-0.43. -13.78) | 8.5 | 7.92 |
|  | 8 | TDLC300-100 Low |  | HomNet set1: (-1.11. -10.91) | 6.9 | 4.34 |
|  |  | TDLA30-10 Low |  | HetNet set2: (-0.43. -13.78) | 5.9 | 3.59 |

CTC:

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | INR Set | SNR with DMRS power boosting DMRS based estimation (dB) | SNR w/o DMRS power boosting (dB) | SNR with DMRS power boosting DMRS + non DMRS RE based estimation (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 11.4 | 12.0 | 11.6 |
|  |  | TDLA30-10 Low |  | Set 2 | 12.5 | 12.8 | 13.0 |
|  | 4 | TDLC300-100 Low |  | Set 1* | 8.0 | 8.5 | 7.6 |
|  |  | TDLA30-10 Low |  | Set 2 | 8.5 | 8.9 | 8.5 |
|  | 8 | TDLC300-100 Low |  | Set 1* | 5.8 | 6.5 | 4.2 |
|  |  | TDLA30-10 Low |  | Set 2 | 6.0 | 6.3 | 4.0 |

Discussion:

Nokia: We understand Samsung’s proposal. We still prefer option 1. The DMRS based MMSE-IRC should be the baseline. It is an implementation issue.

Ericsson: Share same view with Nokia. We agreed all the parameters based on receiver assumption, so no need to modify.

Samsung: We do not against we will basically use DMRS for interference estimation. But the 3dB mismatch issue should be solved by also considering non-DMRS Res.

Nokia: Only considering DMRS should be the safest one.

ZTE: We will handle the 3dB mismatch to do new simulation. But detailed algorithm is for implementation.

Moderator: For this meeting results, are companies using option1 or opt2?

Samsung, HW：option2

ZTE， Nokia，Ericsson, CTC, Qualcomm：option 1.

Moderator would like to check with BS vendors: as per your implementation, can BS ensure there is no power level mismatch between the estimated covariance matrix and the data (not limited to the option 2 algorithm)?

Ericsson: We can deal with the mismatch.

Recommended WF

Ensure there is no power level mismatch between the estimated covariance matrix and the data

It is up to implementation how to deal with it.

CMCC: We are ok with option 2 and the recommended WF.

Nokia, Ericsson: We still against the recommended WF.

Status:

There is no consensus whether to modify the previously agreed MMSE-IRC receiver assumption.

Draft CR review

Issue 2-2-2: Manufacturer declarations

- Previous RAN4 agreements in the WF R4-2504768:

Agreement: - Introduce new manufactory declaration for BS supporting enhanced performance requirements.

- Proposals:

  - Proposal 1: (CATT)

Add manufacturer declarations for MMSE-IRC receiver for BS type 1-C and BS type 1-H conducted test requirements in Table 4.6-1 of TS 38.141-1 as below Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. [D.126]; MMSE-IRC receiver; Declaration of BS supporting MMSE-IRC receiver.; x; x. Add manufacturer declarations for MMSE-IRC receiver for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements in Table 4.6-1 of TS 38.141-2 as below: Declaration identifier; Declaration; Description; Applicability (Note 1). BS type 1-H (Note 2); BS type 1-O; BS type 2-O. [D.129]; MMSE-IRC receiver; Declaration of BS supporting MMSE-IRC receiver.; c; x; n/a.

  - Proposal 2: (Huawei)

| D.129 | Enhanced performance requirements type A | Declaration of support enhanced performance requirements type A | c | n/a | n/a |
| --- | --- | --- | --- | --- | --- |

Tentative Agreement:

Should be updated to:

Description: Declaration of BS supporting enhanced performance requirements.

Declaration: Enhanced performance requirements

Test clause title

- Proposals:

  - CTC:

8.2.X Requirements for PUSCH with enhanced performance requirements - B.X Interference model for PUSCH enhanced performance requirements - B.X.1 Interference to Noise ratio (INR) - B.X.2 Interference model for synchronous scenario

  - Samsung

- 8.2.xx Performance requirements for PUSCH with enhanced DM-RS - B.X Interference model for PUSCH requirement with intercell interference - B.x.1 Interference to Noise ratio (INR) - B.X.2 Interference model for PUSCH requirements

  - ZTE

- 8.2.X Performance requirements for MMSE-IRC for BS type 1-O - B.X Interference model - B.x.1 Dominant interferer proportion

Tentative agreement:

Update the clauses as:

- 8.2.xx	Performance requirements for PUSCH with inter-cell interference

- B.X	Interference model for PUSCH requirement with inter-cell interference

- B.x.1	Interference to Noise ratio (INR)

B.X.2	Interference model for synchronous scenario

INR definition

- Proposals:

  - CTC:

- B.X.1 Interference to Noise ratio (INR) Each inter-cell interferer involved in enhanced performance requirements is characterized by its associated interferer to noise ratio (INR) value: INRi=j=1NRXIor(i)(j)j=1NRXNoc(j) where Ior(i)(j) is the average received power spectral density from the i-th strongest interfering PUSCH involved in the requirement scenario on the j-th antenna connector and Noc is the average power spectral density of a white noise source consistent with definition provided in section 8.1.

  - Samsung

- B.x.1 Interference to Noise ratio (INR) Each interfering cell involved in PUSCH performance requirements with intercell interference is characterized by its associated interferer to noise ratio (INR) value: INRi=DIPi1-i=1MDIPi, i=1,…, M Where DIPi=Ior(i)N,, j=1,…, M where Ior(i) is the received energy from the i-th strongest inter-cell interferer involved in the requirement scenario and N'=j=1MIor(j)+N where N is the the energy of the white noise source consistent with the definition provided in clause xx and M is the total number of simultaneously transmitted inter-cell interferers involved in a given requirement scenario.

  - ZTE

- B.x.1 Dominant interferer proportion Each inter-cell interferer involved in enhanced performance requirements type A and type B is characterized by its associated dominant interferer proportion (DIP) value: ( = 1,…, ) where is the received energy from the i-th strongest inter-cell interferer involved in the requirement scenario and where is the the energy of the white noise source consistent with the definition provided in subclause 8.1 of TS 36.104 [2] and is the total number of simultaneously transmitted inter-cell interferers involved in a given requirement scenario.

Tentative agreement:

CTC’s version is agreed. No need to mention j because it is 1Tx for all cells.

Whether to consider additional applicability rule on how the test is performed

  - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped (Tejas, Samsung, CATT, Ericsson, Nokia, Huawei slightly prefer)

  - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes. (China Telecom, CMCC, Qualcomm, ZTE, Aligned with LTE approach)

Moderator Can proponents for option 2 compeimise to option 1?

CMCC: We do not see test burden issue as in option 1.

Qualcomm, ZTE: We can be ok with opt1.

CTC: For option 1, we would like to know the reason why first test HomNEt?

Nokia, HW: BS declared to support Wide Area BS, Medium Range BS more likely to be deployed in HomNet scenario.

CMCC, Technically, it is possible for WA BS to be deployed in HetNEt.