# 3GPP TSG-RAN WG4 Meeting #115	R4-2508634

# Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.13.1

Source:	Moderator (Huawei)

Title:	Ad-hoc meeting minutes for [115][303] NR_BS_RF_Part2_CLTA_SE

Document for:	Information

# Introduction

Focus of this thread is the co-location test antenna (CLTA) topic of the NR_BS_RF_req_evo work item, as covered in Topic #1.

Additionally, this thread covers discussion on the TX co-existence spurious emissions requirements re-evaluation in Topic #2.

Related objectives were extracted from the WID in RP-243270 are provided for reference:

| Core | OTA test enhancement - Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition. Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz. |
| --- | --- |
| Perf | OTA test enhancement - OTA testing scope reduction: Identify BS excessive OTA test scope cases and reduce it with the aim to shorten the test duration time for TX IM, RX out of band blocking, for NR FR1. - CLTA: Investigate whether the existing test approach can be improved, and if feasible, improve test methods for BS/IAB OTA co-location requirements and tests for AAS-based test specifications for FR1. - TRP simplification: Simplify BS TRP estimation methods (e.g., Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods. Transmitter co-existence spurious emission requirements - Conformance testing corresponding to co-existence spurious emission requirements, if needed. |

# Topic #1: CLTA

## Companies’ contributions summary

The following CLTA-related contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2506307 | Ericsson | Observation 1: Our results indicate angular regions of beam pointing directions with higher and lower coupling loss reflecting variations in the nearfield antenna pattern. Observation 2: For 3500 MHz to 3500 MHz, the A-to-SA coupling loss varies between 55 ± 15 dB. Observation 3: In this contribution identically sized AAS base stations have been studied. Previous studies [8] indicate such size matching is the worst-case coupling loss. Observation 4: For 7000 MHz to 7000 MHz, the A-to-A coupling loss varies between 77 ± 15 dB. Observation 5: For 7000 MHz to 7000 MHz, the A-to-SA coupling loss varies between 70 ± 15 dB. Observation 6: As expected, A-to-A and A-to-SA coupling losses are different. Observation 7: The non-uniform coupling loss with respect to subarray position in the Array-to-subarray and the impact for requirements derivation needs further study. Observation 8: For 3500 MHz to 7000 MHz, the A-to-A coupling loss varies between 54 ± 10 dB. Observation 9: The A-to-A coupling loss is lower for higher correlation. Observation 10: When considering AAS BS coupling loss characteristics for deriving requirement levels, choosing the minimum coupling loss represents an outlier case which would risk driving design complexity and cost. A more suitable percentile would be in the range 95 to 99%. Observation 11: For 3500 MHz to 7000 MHz, the A-to-SA coupling loss varies between 57 ± 10 dB. Observation 12: The spread in the coupling is lower in the A-to-SA compared to the A-to-A case. Observation 13: The simulation results indicates that the BS-to-BS coupling loss relevant for AAS BS operating at and above 3500 MHz is larger than current assumption of 30 dB. Proposal 1: For transmitter spurious emission and receiver out-of-band blocking co-location coupling loss between two systems operating at different bands should be considered, assuming relevant antenna aperture sizes for victim and aggressor. Proposal 2: For TDD OFF power and transmitter intermodulation co-location coupling loss between two systems operating at the same band should be considered. Proposal 3: Co-location of systems with different array size should be studied, e.g. non-AAS to AAS and AAS to AAS. Proposal 4: Study the impact on requirements of non-uniform coupling loss with respect to subarray position in the array-to-subarray case. Proposal 5: When deriving co-location requirements, RAN4 need to agree on what percentile to use in the coupling loss statistics. Proposal 6: Zero correlation of antenna weights can be used to derive worst case requirements for co-location between different bands well separated in frequency. |
| R4-2506308 | Ericsson | CR to TR 37.941: Addition of Rel-19 co-location requirement improvements in subclause 6.4 |
| R4-2507445 | Huawei, HiSilicon | Discussion on CLTA-related aspects |
| R4-2507447 | Huawei, HiSilicon | Draft CR to TR 37.941: Background and motivation for CLTA alternatives discussion |
| R4-2507468 | Huawei, HiSilicon | Big CR to TR 37.941: Rel-19 CLTA-related agreements |
| R4-2507511 | ZTE Corporation, Sanechips | Further discussion on OTA test enhancement |
| R4-2507512 | ZTE Corporation, Sanechips | Draft CR to TR 37.941: Impact of BS-to-BS isolation analysis on the legacy co-location requirements |
| R4-2507633 | Nokia | On OTA test enhancements |
| R4-2507634 | Nokia | DraftCR to TR 37.941 on CLTA alternatives - wideband horn antenna |

## Open issues summary

The following sub-topics were identified based on the formal Proposals, as capture in contributions listed in clase 1.1.

### Sub-topic 1-1: BS-to-BS isolation analysis

Discussion on BS-to-BS isolation analyses were provided in multiple contributions.

Proposals

  - Proposal 1: 3500 MHz BS-BS isolation measurement results to be updated in the summary, to reflect corrected value of 41.4 dB isolation (horizontal domain; no beam steering). (R4-2507445, Huawei)

  - Proposal 2: 4900 MHz BS-BS isolation measurement results to be included in the summary, taking -56.4 dB isolation value (horizontal domain; no beam steering, A-to-SA) as the worst case. (R4-2507445, Huawei)

  - Proposal 3: Consider the following BS-BS isolation data summary as the baseline for the conclusions’ derivation during RAN4#115 meeting (R4-2507445, Huawei):

BS-to-BS isolation data (simulations, or measurements) to be collected on top of the data already collected in the following table formats:

Table 7: BS-BS isolation measurement results (no beam steering; same band; same BS type; A-to-A)

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz |
| --- | --- | --- | --- |
| Isolation in vertical domain (dB) | -54 | -52 | -56 |
| Isolation in horizontal domain (dB) | -31 | -34 | -46 |

Table 8: BS-BS isolation measurement results (beam steering; same band; same BS type)

|  | 3.5 GHz | 7 GHz |
| --- | --- | --- |
| Isolation in horizontal domain (dB) | A-to-A: -55±20 | A-to-A: -77±25 A-to-SA: -70±20 |

Moderator: WF in R4-2502279 has prioritized the following BS-to-BS isolation scenarios:

- Horizontal domain prioritized (vertical BS stacking analyses not precluded);

- Same-band analysis (inter-band analysis not precluded),

- Array-to-array case (array to sub-array not precluded);

- Same BS type for aggressor and victim (mixed cases of various BS types not precluded).

For the derivation of the conclusion, [95-99th] percentile of CDF is expected to be used for simulations, or the worst-case results from measurements.

Recommended WF

- BS-to-BS isolation data collection: Update the set of collected values as in tables below, taking worst-case isolation as baseline for conclusions derivation.

Double-check for any missing data. Companies are asked to fill in all any missing data based on the template agreed in WF (R4-2502279).

- No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | ZTE (M, R4-2504358): -54 | ZTE (M, R4-2504358): -52 | ZTE (M, R4-2504358): -56 |  |
| Isolation in horizontal domain (dB) | ZTE (M, R4-2504358): -31 | ZTE (M, R4-2504358): -34 Huawei (R4-2504594, M): -46 | ZTE (M, R4-2504358): -46 Huawei (M, R4-2507445): -50.6 (A-to-SA) |  |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | S: Ericsson (R4-2418398): A-to-SA: -55±20 dB |  | S: Ericsson (R4-2502160): A-to-A: -77±25; A-to-SA: -70±20 |

Adhoc agreement:

The above tables will be further updated during this week to include more results, including horn antenna results and near field simulation results.

After the adhoc, additional BS-BS isolation data was collected and captured in the following tables:

- No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | ZTE (M, R4-2504358): -54 | ZTE (M, R4-2504358): -52 | ZTE (M, R4-2504358): -56 |  |
| Isolation in horizontal domain (dB) | ZTE (M, R4-2504358): -31 | ZTE (M, R4-2504358): -34 Huawei (R4-2504594, M): -46 Nokia (M, R4-2507633): -35 (A-to-SA) | ZTE (M, R4-2504358): -46 Huawei (M, R4-2507445): -50.6 (A-to-SA) | Nokia (M, R4-2507633): -55 (A-to-SA) |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; same band; same BS type:

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | S: Ericsson (R4-2506307): A-to-SA: -55±15 dB |  | S: Ericsson (R4-2506307): A-to-A: -77±15; A-to-SA: -70±15 |

- Beam steering consideration; different bands:

|  | 3.5 GHz to 7 GHz |
| --- | --- |
| Isolation in horizontal domain (dB) | S: Ericsson (R4-2506307): A-to-A: -54±10 dB A-to-SA: -57±10 dB |

### Sub-topic 1-2: Impacted RF requirements

Open issues and candidate options before meeting:

Proposals

- Proposal 1: Remove OTA TX IMD requirement for BS type 1-O in FR1 bands above 5 GHz, reuse the existing approach implemented for FR2 requirement. (R4-2507445, Huawei)

- Proposal 2: if RAN4 agrees to focus on the MCL analysis within the same band in Rel-19, propose to consider Tx OFF power and Tx IMD requirement only. (R4-2507511, ZTE)

- Proposal 3: Arrange work-split for the implementation of the radiated TX IMD requirement optimization for high frequency bands in FR1; Final CRs to be provided for August RAN4 meeting. (R4-2507445, Huawei)

| Specifications | Responsible company |
| --- | --- |
| TS 37.105 | [Huawei] |
| TS 37.145-1 | Not applicable |
| TS 37.145-2 | [Huawei] |
| TS 38.104 | Company#2 |
| TS 38.141-1 | Not applicable |
| TS 38.141-2 | Company#2 |
| TS 38.174 | Company#3 |
| TS 38.176-1 | Not applicable |
| TS 38.176-2 | Company#3 |
| TS 38.106 | Company#4 |
| TS 38.115-1 | Not applicable |
| TS 38.115-2 | Company#4 |

Recommended WF

- Check online if removal of OTA TX IMD requirement for BS type 1-O in FR1 bands above 5 GHz is agreeable.

    - If yes, update the work-split offline.

- Further discuss on the expected Tx OFF power requirement impact.

Adhoc discussion:

ZTE: In general, we are OK with Proposal 1.

For Proposal 2: inter-band was not addressed. Focus on intra-band cases.

Ericsson: proposals 1/2/3 are rushing too much. Background needed for each requirement needs clarifications.

Nokia: we need to agree on coupling loss first.

### Sub-topic 1-3: CLTA alternatives: Horn antenna

Discussion on horn antenna perfomance comparison with CLTA was provided (R4-2507633, Nokia), with multiple observations.

Proposals: no formal proposals were captured in the above referred tdoc.

Moderator: As horn antenna was agreed last meeting to be included in conclusions of this work, it is advised to check if companies have technical questions related to the horn antenna evaluations in R4-2507633.

Recommended WF: Discuss online. As there was no formal proposal captured, companies are encouraged to provide (online or offline) feedback on the horn antenna evaluations.

### Sub-topic 1-4: Near field modelling simulations

Open issues and candidate options before meeting:

Proposals (R4-2506307, Ericsson)

- Proposal 1: For transmitter spurious emission and receiver out-of-band blocking co-location coupling loss between two systems operating at different bands should be considered, assuming relevant antenna aperture sizes for victim and aggressor.

- Proposal 2: For TDD OFF power and transmitter intermodulation co-location coupling loss between two systems operating at the same band should be considered.

- Proposal 3: Co-location of systems with different array size should be studied, e.g. non-AAS to AAS and AAS to AAS.

- Proposal 4: Study the impact on requirements of non-uniform coupling loss with respect to subarray position in the array-to-subarray case.

- Proposal 5: When deriving co-location requirements, RAN4 need to agree on what percentile to use in the coupling loss statistics.

- Proposal 6: Zero correlation of antenna weights can be used to derive worst case requirements for co-location between different bands well separated in frequency.

Moderator:

- During RAN4#114bis meeting, multiple companies have expressed concerns on the consideration of near field simulation methodology.

- While WID captures objectives on CLTA definition updates and current work includes investigation on the CLTA antenna alternatives, the above proposals propose new way of OTA co-location requirements testing (i.e., no CLTA), which is not within the scope of Rel-19 WI.

Recommended WF: Discuss online. Additionally, collect feedback on the following aspects:

- Whether consideration of near field modelling and related simulation methodology is considered to be within the scope of this WI.

- Whether presented results shall be considered in the summary, as the underlying simulation assumptions are not following previous arrangements.

- Whether and how to potentially address near field modelling in future evaluations.

Adhoc discussion:

Ericsson: Comments were received last meeting that NF is hard to simulate. We have updated our paper accordingly. Results show that the coupling is far from 30 dB legacy assumption. Some simplifications were captured.

ZTE: Methodology is contrubution driven – we can consider those results as well. We can consider both simulations and measurements.

Nokia: We can use simulations and measurements. Starting point is not very dfferent. We also have measurement results.

ZTE: We also have measurement results.

Adhoc agreement:

Aim for introduction of additional results (simulation and measurements) into the summary.

### Sub-topic 1-5: Draft CRs

Open issues and candidate options before meeting:

The following Draft CR were submitted:

  - R4-2506308	CR to TR 37.941: Addition of Rel-19 co-location requirement improvements in subclause 6.4 (Ericsson)

  - R4-2507447	Draft CR to TR 37.941: Background and motivation for CLTA alternatives discussion (Huawei, HiSilicon)

  - R4-2507512	Draft CR to TR 37.941: Impact of BS-to-BS isolation analysis on the legacy co-location requirements (ZTE Corporation, Sanechips)

  - R4-2507634	DraftCR to TR 37.941 on CLTA alternatives - wideband horn antenna (Nokia)

  - R4-2507468	Big CR to TR 37.941: Rel-19 CLTA-related agreements (Huawei, HiSilicon)

Recommended WF

- Review Draft CRs one by one, aiming to Endorse aggregable content this week.

- Merge agreeable content into the Big CR (possibly for e-mail approval, depending on the progress during the week).

# Topic #2: TX co-existence spurious emissions requirements re-evaluation

## Companies’ contributions summary

The following contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2506535 | Qualcomm Germany | Observation 1: For protection UE receiver, only by considering relevant FSPL and noise figure values for upper FR1 bands (i.e., n104), relaxation of [9.5 dB, 13.5 dB] could be realized depending on the underlying assumptions. Observation 2: With system level assumptions such as 100% grid shift, 38.901 propagation model and ISD = 500m, conservative assumptions to define emission requirements that were considered TR 25.942 would not be applicable anymore. Proposal 1: To account for AAS impacts, RAN4 to account for vertical and azimuthal offsets of the subarray antenna gain in the link budget analysis to derive the emission requirements. Proposal 2: For the protection of BS receiver, RAN4 to further discuss if the full AAS gain or only the subarray gain to be account for when evaluating the BS-to-BS coupling loss. Proposal 3: For the protection of BS receiver, RAN4 to further discuss if statistical approach (i.e., SLS-inspired approach) to be followed or the MCL approach in TR 25.942. If the former, RAN4 to discuss which percentile of the BS-to-BS coupling loss CDF to be utilized when evaluating the updated emissions. |
| R4-2507446 | Huawei, HiSilicon | BS-BS Non-AAS aggressor Observation 1: The out of band passive antenna has significantly less directional BS-BS antenna gain than the in-band case. Observation 2: considering the effects to the element and array patterns the BS-BS antenna gain will not be worse than in band case. BS-BS AAS aggressor Observation 3: worst case for a coherent AAS harmonic emission a coherent beam is steered towards the edge of the cell. Observation 4: The average power distribution will take the shape of the element pattern Observation 5: with absolute worst case assumptions the maximum allowable interferer to maintain BS-BS co-existence performance with an AAS aggressor is -46dBm BS-UE Observation 6: The worst case MCL for the 6.4GHz victim with a 3.5GHz aggressor is 53dB Observation 7: The MCL is the same as the existing requirement. Observation 8: The similar MCL values can be simply explained by the increase in FSPL being cancelled out by the increase in antenna gain. Observation 9: Due to the statistical nature of the AAS beam pattern direct comparison with non-AAS MCL requires some assumptions. Worst case assumptions used are potentially too strict. Observation 10: Analysis does not include losses due to increased loss and decreased radiation efficiency and as such show the existing requirement represents a safe worst case. Proposal 1: Maintain the existing co-existence in same geographical area emissions levels. |
| R4-2507513 | ZTE Corporation, Sanechips | Observation 1a: if the spurious emission signal is not the harmonic spurious signal, then beamforming gain of spurious emission cannot been assumed at 6.4GHz. Observation 1b: if the spurious emission signal is the harmonic spurious signal, RAN4 needs more evaluations whether beamforming gain could assumed or not. Observation 2: the BS2BS MCL under the different grid shift are summarized as following: Table 1. the summarized BS2BS MCL under the different grid shift MCL. Grid shift; 99%; 99.998%; Baseline. 100%; -95; -75.7; -67. 50%; -67; -67.3; -67. 20%; -78.07; -58.6; -67. 10%; -72.7; -51.01; -67. Observation 3: the BS2UE MCL values are summarized as following: Table 2. the summarized BS2UE MCL MCL. Grid shift; 99%; 99.965%; Baseline. 100%; -71; -65.7; -53. Observation 4: If the 99% tile of MCL is considered for both BS2BS and BS2UE, then around 18dBc relaxation could be allowed. If the 99.9% tile of MCL is considered for both BS2BS and BS2UE, then around 12dBc relaxation could be allowed. |
| R4-2507635 | Nokia | Observation: Over 99% of the coupling loss is above the current MCL assumption of 70 dB in all simulated macro-cell environments. Proposal: To discuss and agree on the system level simulation assumptions in micro-cell environment to re-evaluate the current MCL assumption of 53 dB using system level evaluations. |
| R4-2507670 | Ericsson | Observation 1: Assuming uncorrelated signals can be used as a baseline. Observation 2: When signals are uncorrelated (ρ = 0), the antenna array does not exhibit array factor beamforming gain. Observation 3 : Passive Antenna type A10 from TR 37.840 with 10 element single column antenna model has a narrower horizontal beamwidth and 0.9 lambda vertical spacing as compared to AAS antenna assumptions with antenna parameters of 90 degrees horizontal beamwidth and 0.7 lambda vertical spacing. Observation 4 : In Release-99 approach to evaluate the MCL, it was reasonable to assume non-AAS passive fixed beam antennas because of the predictable and stable nature of the beam’s directions and unavailability of AAS antennas. Observation 5 : Release-99 approach to evaluate the MCL for AAS antennas has large variations, due to adaptive beamforming adjusting the beam patterns towards multiple UEs and different UE positions to maximize the reception and transmission of signal. Observation 6 : Beamforming gain is not considered at BS Tx side as the unwanted emissions are modelled as uncorrelated signals, whereas the Beamforming gain is included at BS Rx side. Observation 7 : Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective. Observation 8 : It can be observed that the coupling loss lies at the tail of the CDF, where the curve tends to be very flat and is spread around 20 dB from 99%-ile to 100%-ile. Observation 9 : It can be observed that the coupling loss lies at the tail of the CDF, where the curve tends to be very flat and is spread around 20 dB from 99%-ile to 100%-ile. Proposal 1 : RAN4 should model unwanted emissions as uncorrelated signals to simplify the system level evaluation. Proposal 2 : The Macro Non-AAS antenna configuration listed in Table 2.1.4.3-1 can be used for simulation evaluation, as it is expected to produce realistic results similar to a real passive antenna. Table 2.1.4.3-1: Macro Non-AAS BS antenna Non- AAS Urban Macro. 2 GHz (Aggressor); 3.5 GHz (Aggressor). Parameter. Element gain (dBi) (Note 2); 6.4; 6.4. Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 65º for V; 90º for H 65º for V. Horizontal/vertical front-to-back ratio (dB); 30 for both H/V; 30 for both H/V. Antenna polarization; Linear ±45º polarized sub-array; Linear ±45º polarized sub-array. Antenna array configuration (Row × Column) (Note 4); 10x1; 10x1. Horizontal/Vertical radiating sub-array spacing; 0.5 of wavelength for H, 0.7 of wavelength for V; 0.5 of wavelength for H, 0.7 of wavelength for V. Number of element rows in sub-array; 1x1; 1x1. Vertical element separation in sub-array (dv,sub); 0.7 of wavelength of V; 0.7 of wavelength of V. Pre-set sub-array down-tilt (degrees); N/A; N/A. Array Ohmic loss (dB) (Note 2); 2; 2. Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3); 38; 38. Base station horizontal coverage range (degrees); +/-60; +/-60. Base station vertical coverage range (degrees) (Note 1); 90-100; 90-100. Mechanical down-tilt (degrees); 6; 6. Proposal 3 : Define a coupling loss approach, similar to the legacy MCL approach, where we discuss around 99th percentile of the coupling loss CDF, instead of the minimum value. |

## Open issues summary

The following sub-topics were identified based on the formal Proposals, as capture in contributions listed in clase 2.1.

### Sub-topic 2-5: Antenna / array gain considerations

Selected observations related to the discussion:

Observation 1: if the spurious emission signal is not the harmonic spurious signal, then beamforming gain of spurious emission cannot been assumed at 6.4GHz. (R4-2507513, ZTE)

Observation 2: if the spurious emission signal is the harmonic spurious signal, RAN4 needs more evaluations whether beamforming gain could assumed or not. (R4-2507513, ZTE)

Observation 3: Assuming uncorrelated signals can be used as a baseline. (R4-2507670, Ericsson)

Observation 4: When signals are uncorrelated (ρ = 0), the antenna array does not exhibit array factor beamforming gain. (R4-2507670, Ericsson)

Observation 5	: Beamforming gain is not considered at BS Tx side as the unwanted emissions are modelled as uncorrelated signals, whereas the Beamforming gain is included at BS Rx side. (R4-2507670, Ericsson)

Observation 6: (BS-BS Non-AAS aggressor) The out of band passive antenna has significantly less directional BS-BS antenna gain than the in-band case.

Observation 7: (BS-BS Non-AAS aggressor) considering the effects to the element and array patterns the BS-BS antenna gain will not be worse than in band case.

Observation 8: (BS-BS AAS aggressor) worst case for a coherent AAS harmonic emission a coherent beam is steered towards the edge of the cell.

Open issues and candidate options before meeting:

Proposals (non-exclusive)

  - Proposal 1: To account for AAS impacts, RAN4 to account for vertical and azimuthal offsets of the subarray antenna gain in the link budget analysis to derive the emission requirements. (R4-2506535, Qualcomm)

  - Proposal 2: For the protection of BS receiver, RAN4 to further discuss if the full AAS gain or only the subarray gain to be account for when evaluating the BS-to-BS coupling loss. (R4-2506535, Qualcomm)

  - Proposal 3: RAN4 should model unwanted emissions as uncorrelated signals to simplify the system level evaluation. (R4-2507670, Ericsson)

Recommended WF:

- Proposal 1: double-check online if agreeable.

- Proposal 2: Discuss online.

- Proposal 3: Discuss online if the gain from unwanted emissions should be modelled as uncorrelated signals.

Nokia: we need to decide how to derive requirement.

### Sub-topic 2-6: Performance metrics

Selected observations related to the discussion:

Observation 1: It can be observed that the coupling loss lies at the tail of the CDF, where the curve tends to be very flat and is spread around 20 dB from 99%-ile to 100%-ile. (R4-2507670, Ericsson)

Open issues and candidate options before meeting:

Proposals

- Proposal 1: For the protection of BS receiver, RAN4 to further discuss if statistical approach (i.e., SLS-inspired approach) to be followed or the MCL approach in TR 25.942. If the former, RAN4 to discuss which percentile of the BS-to-BS coupling loss CDF to be utilized when evaluating the updated emissions. (R4-2506535, Qualcomm)

- Proposal 2: Define a coupling loss approach, similar to the legacy MCL approach, where we discuss around 99th percentile of the coupling loss CDF, instead of the minimum value. (R4-2507670, Ericsson)

Recommended WF: Discuss online.

Qualcomm: DL: MCL approach. UL: directivity to account for.

ZTE: for directivity is it subarray, of full array?

Qualcomm: We can finetune assumptions. We need to split UL and DL.

Nokia: Even for MCL for Rel-99, we had different MCL. Statistics can be used for UL.

Qualcomm: worst case scenario shall be used.

Ericson: We need to separate those cases.

### Sub-topic 2-7: System-level simulation assumptions

Selected observations related to the discussion:

Observation 1: Over 99% of the coupling loss is above the current MCL assumption of 70 dB in all simulated macro-cell environments. (R4-2507635, Nokia)

Observation 2: Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective.

Proposals

- Proposal 1: To discuss and agree on the system level simulation assumptions in micro-cell environment to re-evaluate the current MCL assumption of 53 dB using system level evaluations (R4-2507635, Nokia)

- Proposal 2: The Macro Non-AAS antenna configuration listed in Table 2.1.4.3-1 can be used for simulation evaluation, as it is expected to produce realistic results similar to a real passive antenna. (R4-2507670, Ericsson)

|  | Non- AAS Urban Macro |  |
| --- | --- | --- |
|  | 2 GHz (Aggressor) | 3.5 GHz (Aggressor) |
| Parameter |  |  |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) (Note 4) | 10x1 | 10x1 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array | 1x1 | 1x1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | N/A |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 38 | 38 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 6 | 6 |

Recommended WF: Discuss online.

### Sub-topic 2-7: Tx spurious emissions co-ex requirement

Selected observations related to the discussion:

Observation 1: If the 99% tile of MCL is considered for both BS2BS and BS2UE, then around 18dBc relaxation could be allowed. (R4-2507513, ZTE)

Observation 2: If the 99.9% tile of MCL is considered for both BS2BS and BS2UE, then around 12dBc relaxation could be allowed. (R4-2507513, ZTE)

Observation 3: Due to the statistical nature of the AAS beam pattern direct comparison with non-AAS MCL requires some assumptions. Worst case assumptions used are potentially too strict. (R4-2507446, Huawei)

Observation 4: Analysis does not include losses due to increased loss and decreased radiation efficiency and as such show the existing requirement represents a safe worst case. (R4-2507446, Huawei)

Observation 5: (BS-BS AAS aggressor) worst case for a coherent AAS harmonic emission a coherent beam is steered towards the edge of the cell. (R4-2507446, Huawei)

Observation 6: (BS-BS AAS aggressor) The average power distribution will take the shape of the element pattern. (R4-2507446, Huawei)

Observation 7: (BS-BS AAS aggressor) with absolute worst case assumptions the maximum allowable interferer to maintain BS-BS co-existence performance with an AAS aggressor is -46dBm. (R4-2507446, Huawei)

Observation 8: (BS-UE) The worst case MCL for the 6.4GHz victim with a 3.5GHz aggressor is 53dB. (R4-2507446, Huawei)

Observation 9: (BS-UE) The MCL is the same as the existing requirement. (R4-2507446, Huawei)

Observation 10: (BS-UE) The similar MCL values can be simply explained by the increase in FSPL being cancelled out by the increase in antenna gain. (R4-2507446, Huawei)

Open issues and candidate options before meeting:

Proposals

- Proposal 1: Maintain the existing co-existence in same geographical area emissions levels. (R4-2507446, Huawei)

Recommended WF: Discuss online.