3GPP TSG-RAN WG4 Meeting #115														R4-2507845

Malta, MT, 19th – 23rd May 2025

Agenda item:				7.11.1

Source:						Moderator (Intelsat)

Title:							Topic summary for [115][313] NR_NTN_Ku_Band_Coexistence…

Document for:			Information

# Introduction

This document provides a summary of the contributions submitted to the RAN4 #115 working group on the Ku Band Coexistence simulations.

With the agreement reached at the last RAN4 #114-bis meeting in Wuhan, on the simulation assumptions, the work in converging on the following areas:

- VSAT Phase array antenna performances

- Initial ACIR Values for GEO and LEO at 90-degree and 35-degree elevations

- Simulation of Parabolic Antenna

Seven contributions were received for the RAN4 #115 meeting on this topic, and we greatly appreciate the time and the effort that companies have spent in preparing various contributions.

# R4-2505718, Intelsat, Ku Band Co-existence: Performance of 45x45 and 78x78 Element Phase Array Antennas

In this contribution, Intelsat have presented performance of 45x45 and 78x78 phase array VSAT antennas. The simulations were carried out at 11 and 14 GHz and at 90-degrre and 35-degree elevations.

In the simulations the radiation patterns were measured against FCC and CEPT for compliance.

The following figure illustrates the 45x45 element antenna at 90-0degree

Figure 5: 45x45 element @ 0 deg scan: Antenna gain and EIRP density.

And the following illustrates the performance of the 78x78 antenna at 90-degree

For the 35-degree elevation the performance of the 45x45 element antenna is as shown in the following figure:

Figure 6: 45x45 element @ 55 deg scan: Antenna gain and EIRP density.

And for the 78x78 element antenna at 35-degree elevation the performance is as follows:

Figure 11: 78x78 element @ 55 deg scan: Antenna gain and EIRP density.

Intelsat have proposed the following:

Proposal 1: For the 45x45 phase array element antenna, to proceed with 10 W output power at 90 degree and 35-degree elevation angles, as antenna performances are compliant with FCC and CEPT masks.

Proposal 2: We propose that for 78x78 element array, the EIRP be reduced by 4.4 dB at 90 degrees elevation to be FCC mask compliant with 10 MHz bandwidth.

Proposal 3: We propose that for 78x78 element array, the bandwidth be increased to 30 MHz at 90 degrees elevation to be FCC mask compliant with 50 W power.

Proposal 4: We propose that for 78x78 element array, the peak EIRP density be 41.6 dBW/MHz at 90 degrees elevation to be FCC mask compliant.

Proposal 5: We propose that for 78x78 element array, the EIRP be reduced by 7 dB at 35 degrees elevation to be FCC mask compliant with 10 MHz bandwidth.

Proposal 6: We propose that for 78x78 element array, the EIRP be reduced by 4 dB at 35 degrees elevation to be CEPT mask compliant with 10 MHz bandwidth.

Proposal 7: We propose that for 78x78 element array, the bandwidth be increased to 50 MHz at 35 degrees elevation to be FCC mask compliant with 50 W power.

Proposal 8: We propose that for 78x78 element array, the bandwidth be increased to 25 MHz at 35 degrees elevation to be CEPT mask compliant with 50 W power.

Proposal 9: We propose that for 78x78 element array, the peak EIRP density be 35.5 dBW/MHz at 35 degrees elevation to be FCC mask compliant.

Proposal 10: We propose that for 78x78 element array, the peak EIRP density be 68.5 dBW/MHz at 35 degrees elevation to be CEPT mask compliant.

Moderator’s observations:

- The 45x45 element phase array antenna is compliant with the CEPT and FCC masks

- The 78x78 element antenna is not compliant with the FCC mask at 90-degree with 50W but it is compliant if the output power was reduced to 18W

- The 78x78 element phase array antenna with increase bandwidth of 30 MHz and 50W is compliant to the FCC mask

- The 78x78 element antenna at 35-degree elevation is compliant with the FCC mask with either reduced output power or with increase bandwidth of 50 MHz

Moderator’s note: For the co-existence simulation, the channel bandwidth is 100MHz.  As a result, the 78x78 element antenna will not exceed the FCC or CEPT mask.  However, it is important to bring forward to the group the FCC and CEPT mask non-compliance at lower channel bandwidth (i.e., 10 MHz).

# R4-2505747, Qualcomm, Coexistence simulation results between TN and NTN for Ku band

In this contribution Qualcomm have presented co-existence simulation results between Terrestrial and No-Terrestrial networks. Qualcomm’s uses the simulation assumptions agreed in the previous RAN4 #114-bis as the basis for their analysis

Four agreed scenarios were used for this simulation as shown in the following table:

Table 1: Simulated coexistence scenarios

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | 14 GHz | ACLR NTN UE to be varied/defined. ACS TN gNB fixed |
| 2 | TN with NTN | NTN DL | TN DL | 11 GHz | ACLR NTN SAN to be varied/defined. ACS TN UE fixed |
| 3 | TN with NTN | TN DL | NTN DL | 11 GHz | ACLR TN gNB fixed. ACS NTN UE to be varied/defined |
| 4 | TN with NTN | TN DL | NTN UL | 14 GHz | ACLR TN gNB fixed. ACS NTN SAN to be varied/defined |

The worst-case scenario, out of the four identified, was Scenario 1 and the following table illustrates the ACIR value calculated from the simulations:

Table 3: Required ACIR for Scenario 1 considering VSAT Parabolic antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 5 | 10 | 15 | 20 | 25 |
| GEO @90 deg | Average THP-Loss [%] | 2.4 | 1.5 | 0.9 | 0.5 | 0.33 | 0.18 |
|  | Cell edge THP-Loss [%] | 16.9 | 12.1 | 6.3 | 3.1 | 1.1 | 0.7 |
| GEO @35 deg | Average THP-Loss [%] | 5.1 | 3.4 | 2.2 | 1.4 | 0.9 | 0.5 |
|  | Cell edge THP-Loss [%] | 38 | 24.3 | 15.7 | 9.2 | 5.5 | 3 |
| LEO 600 @90 deg | Average THP-Loss [%] | 0.19 | 0.09 | 0.03 | 0.01 | 0.004 | 0.001 |
|  | Cell edge THP-Loss [%] | 0.88 | 0.57 | 0.08 | 0 | 0 | 0 |
| LEO 600 @35 deg | Average THP-Loss [%] | 3.2 | 2.2 | 1.5 | 0.97 | 0.6 | 0.35 |
|  | Cell edge THP-Loss [%] | 22.6 | 15.7 | 11.4 | 6.6 | 3.1 | 1.1 |

Qualcomm have made the following observations

Observation 1: The ACIR values for scenario 1 for VSAT under worst case assumption with 35 degrees of elevation can be 20 dB and the BS ACS of 30 dB can be respected. Hence the ACLR of the NTN UE VSAT can be defined with 20 dB with no problem for the Parabolic antenna case.

Observation 2: The ACIR values for scenario 1 for VSAT with phased array antenna under worst case assumption with 35 degrees of elevation can be 25 dB and the BS ACS of 30 dB can still be respected.

Observation 3: The ACIR values for scenario 2 are 0 dB for all the satellite altitudes and at different elevation angles. The TN UE ACS of 24 dB can be respected. Hence the ACLR of the NTN Satellite can be defined with less value than 24 dB with no problem or based on other scenarios.

Observation 4: The ACIR value for scenario 3 is 40 dB considering different elevation angles for VSAT using parabolic antenna which is exceeding the TN BS ACLR of 37 dB. Hence this scenario will be problematic without using a coordination distance between the BS and the VSAT or using a frequency guard band between the NTN and TN operation. That will need further discussions. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised either by increasing the satellite TX power or decreasing the RBs to avoid the out of coverage.

Observation 5: The ACIR value for scenario 3 is 45 dB considering different elevation angles for VSAT using phased array antenna which is exceeding the TN BS ACLR of 37 dB. Similarly, to VSAT with parabolic antenna, either coordination distance needed or a frequency guard band between the NTN and TN operation bands needed. That will need further discussions. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised either by increasing the satellite TX power or decreasing the RBs to avoid the out of coverage.

Observation 6: The ACIR values for scenario 4 are between 15 to 30 dB for different satellite altitudes and different elevation angles. The TN BS ACLR of 30 dB can be respected. Hence the ACS of the satellite can be defined with 30 dB with no problem. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised to avoid the out of coverage for UL with GEO.

Qualcomm have proposed the following:

Proposal 1: Based on the observations 1, and 2, the phased array antenna case should consider stringent requirements compared to parabolic antenna case.

Proposal 2: Based on the observations 1, and 2, the ACLR for NTN UE VSAT is 20 dB and 25 dB for parabolic and phased array antenna respectively.

Proposal 3: Based on observation 3, the ACLR of the satellite can be defined with value less than 24 dB.

Proposal 4: Based on observation 4, in case that cell edge throughput is “NaN” due to the out of coverage, RAN4 can conclude based on average throughput loss.

Proposal 5: Based on observation 4 and 5, the ACS of NTN UE for both types of VSAT antennas is difficult to be defined because the ACIR values are between 40 and 45 dB. RAN4 needs to study how to ensure coexistence between TN and NTN.

Proposal 6: The system parameters for lower elevation angles at 35 degrees need to be revised either by increasing the satellite or NTN UEs TX power or decreasing the number of RBs because most of NTN UEs in DL or UL will be out of coverage even without adjacent interference from TN.

Proposal 7: Based on observation 6, the ACS of the satellite can be defined with 30 dB.

# R4-2505762, Xiaomi, Temporary calibration results and simulation results for the Ku-band NTN coexistence study

In this contribution, Xiaomi have presented the initial calibration results of the Ku band co-existence simulation. Xiaomi have used the assumption values agreed in the previous RAN4 #114-bis as their basis for the analysis.

Xiaomi have based their analysis on the agreed 60 cm Parabolic antenna and 45x45 element phased array VSAT.

Xiaomi have presented the Coupling Loss and SINR (Signal to Interference Noise Ratio) of both Terrestrial and Non-Terrestrial Networks in Downlink and Uplink directions.

The following table illustrate the calibration summaries:

Table 1: Calibration summary for NTN DL

| Calibration metrics | DL Coupling Loss |  |  | DL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CDF percentile | @5% | @50% | @95% | @5% | @50% | @95% |
| LEO-600(90deg Parabolic) | 93.17 | 101.62 | 109.98 | 29.20 | 37.61 | 46.05 |
| GEO(90deg Parabolic) | 125.38 | 133.59 | 142.87 | 27.86 | 37.15 | 45.33 |
| LEO-600(35deg Parabolic) | 97.48 | 108.57 | 147.91 | -8.67 | 30.63 | 41.74 |
| GEO(35deg Parabolic) | 125.17 | 135.82 | 174.60 | -3.88 | 34.92 | 45.57 |
| LEO-600(90deg 45*45PhasedArray) | 91.49 | 99.12 | 107.84 | 31.36 | 40.13 | 47.75 |
| GEO(90deg 45*45PhasedArray) | 122.75 | 131.21 | 140.45 | 30.27 | 39.53 | 47.99 |
| LEO-600(35deg 45*45PhasedArray) | 100.01 | 110.26 | 149.48 | -10.33 | 28.95 | 39.22 |
| GEO(35deg 45*45PhasedArray) | 127.04 | 137.74 | 177.08 | -6.46 | 32.99 | 43.65 |

Table 2: Calibration summary for NTN UL

| Calibration metrics | UL Coupling Loss |  |  | UL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CDF percentile | @5% | @50% | @95% | @5% | @50% | @95% |
| LEO-600(90deg Parabolic) | 95.13 | 104.81 | 118.30 | 14.82 | 14.82 | 14.82 |
| GEO(90deg Parabolic) | 125.96 | 137.09 | 157.11 | -16.16 | 3.85 | 14.82 |
| LEO-600(35deg Parabolic) | 98.50 | 109.58 | 147.94 | -7.07 | 14.82 | 14.82 |
| GEO(35deg Parabolic) | 125.93 | 137.71 | 176.49 | -35.56 | 3.20 | 14.82 |
| LEO-600(90deg 45*45PhasedArray) | 94.46 | 104.24 | 118.36 | 14.82 | 14.82 | 14.82 |
| GEO(90deg 45*45PhasedArray) | 126.33 | 137.86 | 158.88 | -17.99 | 3.04 | 14.57 |
| LEO-600(35deg 45*45PhasedArray) | 102.24 | 113.27 | 152.04 | -11.13 | 14.82 | 14.82 |
| GEO(35deg 45*45PhasedArray) | 130.12 | 141.13 | 180.22 | -39.35 | -0.24 | 10.78 |

Table 3: Calibration summary for TN DL

| Calibration metrics | DL Coupling Loss |  |  | DL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CDF percentile | @5% | @50% | @95% | @5% | @50% | @95% |
| Macro Urban | 50.63 | 75.61 | 95.54 | 12.41 | 32.77 | 55.26 |

Table 4: Calibration summary for TN UL

| Calibration metrics | UL Coupling Loss |  |  | UL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CDF percentile | @5% | @50% | @95% | @5% | @50% | @95% |
| Macro Urban | 50.72 | 75.82 | 95.44 | 10.20 | 14.75 | 14.81 |

Moderator’s note: Calibration for phase array antenna has not been performed to date.

Xiaomi have proposed the following summary table for the ACIR value:

to achieve the 5% throughput loss criteria.

|  | SAN GEO | SAN LEO-600 | VSAT - 45 x 45 Phased Array | VSAT - 60 cm Parabolic |
| --- | --- | --- | --- | --- |
| ACIR requirements to determine ACLR | 0 | 0 | 3.6 | 9.6 |
| ACIR requirements to determine ACS | 10.3 | 39.5 | 19.3 | 26.2 |

Xiaomi have made the following Observations:

Observation 1: Basic on the simulation results, the ACIR requirement for case 1 the GEO 60 cm Parabolic should be 9.6 dB.

Observation 2: Basic on the simulation results, the ACIR requirement for case 2 the GEO 60 cm Parabolic should be 9.9 dB.

Observation 3: Basic on the simulation results, the ACIR requirement for case 3 the GEO 60 cm Parabolic should be 25 dB.

Observation 4: Basic on the simulation results, the ACIR requirement for case 4 the GEO 60 cm Parabolic should be 0 dB.

Observation 5: Basic on the simulation results, the ACIR requirement for case 1 the LEO-600 60 cm Parabolic should be 2.1 dB.

Observation 6: Basic on the simulation results, the ACIR requirement for case 2 the LEO-600 60 cm Parabolic should be 37.7 dB

Observation 7: Basic on the simulation results, the ACIR requirement for case 3 the LEO-600 60 cm Parabolic should be 26.2 dB.

Observation 8: Basic on the simulation results, the ACIR requirement for case 4 the LEO-600 60 cm Parabolic should be 0 dB.

Observation 9: Basic on the simulation results, the ACIR requirement for case 1 the GEO 45 x 45 Phased Array case should be 3.6 dB.

Observation 10: Basic on the simulation results, the ACIR requirement for case 2 the GEO 45 x 45 Phased Array case should be 10.3dB.

Observation 11: Basic on the simulation results, the ACIR requirement for case 3 the GEO 45 x 45 Phased Array case should be 18.3dB.

Observation 12: Basic on the simulation results, the ACIR requirement for case 4 the GEO 45 x 45 Phased Array case should be 0 dB.

Observation 13: Basic on the simulation results, the ACIR requirement for case 1 the LEO-600 45 x 45 Phased Array case should be 0 dB.

Observation 14: Basic on the simulation results, the ACIR requirement for case 2 the LEO-600 45 x 45 Phased Array case should be 39.5dB.

Observation 15: Basic on the simulation results, the ACIR requirement for case 3 the LEO-600 45 x 45 Phased Array case should be 19.3 dB.

Observation 16: Basic on the simulation results, the ACIR requirement for case 4 the LEO-600 45 x 45 Phased Array case should be 0 dB.

Xiaomi have proposed the following:

Proposal 1: RAN4 can discuss the necessity of the calibration and consider the calibration work for the Ku-band NTN.

Proposal 2: RAN4 can consider the simulation results as a starting point of ACIR requirement discussion.

# R4-2506020, CATT, Further discussion on coexistence evaluations for Ku-band for NR NTN

CATT in this contribution, CATT are re-examining the previously agreed simulation parameters for the GEO Satellite.

CATT are proposing to use the following values:

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 51.4 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 3.14 m | 0.714 m |
| G/T |  |  | 23.8 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 51.4 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

CATT’s proposal is

- to increase the GEO Satellite’s gain to 51.4 dBi

- decrease the Equivalent satellite antenna aperture  3.14 m

- Increase the G/T value to 23.8 dB K-1

- Increase the GEO Satellite RX max Gain to 51.4 dBi

Moderator’s note: The GEO and LEO satellites’ parameters were agreed in the last meeting following a lengthy simulation and as many companies have already based their simulation based on the agreed parameter values, changing the simulation parameters at this stage would be counterproductive.

# R4-2506258, ZTE, Discussion on coexistence study for NR NTN Ku band

In this contribution, ZTE have shared their analysis of the interference between the terrestrial and the non-terrestrial Ku Band systems. .Their analysis include the GEO and LEO at 90 degree and 35-degree elevations.

In this extensive contribution, ZTE have used the 60 cm Parabolic antenna and 45x45 element phase array VSAT antennas for this co-existence analysis.

Following  table illustrates ACIR results of Scenario 1 with GEO Satellite using 60cm Parabolic and 45x45 element phase array antennas

Table 2.1.1.1-1: Simulation results for scenario 1

| Elevation angle | Required ACIR [dB] |  | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 90° | Throughput Loss | Average | 9.89 | 7.44 | 5.69 | 4.33 | 3.29 | 2.51 | 1.94 | 1.48 | 1.14 | 0.87 | 0.69 | 0.56 | 0.44 | 0.35 | 0.27 | 0.21 |
|  |  | 5%-tile | 74.45 | 65.29 | 55.38 | 45.48 | 34.95 | 25.57 | 18.74 | 13.22 | 8.77 | 6.67 | 4.79 | 3.93 | 2.96 | 1.7 | 1.2 | 0.71 |
| 35° | Throughput Loss | Average | 11.44 | 8.67 | 6.56 | 5 | 3.79 | 2.92 | 2.25 | 1.71 | 1.29 | 0.98 | 0.77 | 0.59 | 0.46 | 0.37 | 0.3 | 0.22 |
|  |  | 5%-tile | 78.29 | 69.92 | 60.86 | 50.11 | 40.32 | 32.05 | 23.95 | 17.44 | 12.67 | 8.3 | 6.1 | 3.96 | 2.18 | 1.71 | 1.44 | 0.58 |

Table 2.1.1.1-2: Interpolated ACIR values for Scenario 1 to meet the 5% throughput loss criteria

| Elevation angle | Throughput Loss | Interpolated ACIR[dB] |
| --- | --- | --- |
| 90° | Average | 5.01 |
|  | 5%-tile | 19.78 |
| 35° | Average | 6 |
|  | 5%-tile | 21.03 |

#### 2.1.1.2 45*45 phased array antenna pattern

Table 2.1.1.2-1: Simulation results for scenario 1

| Elevation angle | Required ACIR [dB] |  | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 90° | Throughput Loss | Average | 1.19 | 0.95 | 0.75 | 0.6 | 0.47 | 0.37 | 0.3 | 0.24 | 0.2 | 0.16 | 0.12 | 0.1 | 0.07 | 0.06 | 0.05 | 0.04 |
|  |  | 5%-tile | 25.3 | 20.59 | 16.98 | 13.48 | 10.82 | 8.43 | 7.27 | 5.7 | 4.54 | 3.48 | 2.31 | 1.57 | 1.37 | 0.71 | 0.38 | 0.19 |
| 35° | Throughput Loss | Average | 0.16 | 0.14 | 0.11 | 0.09 | 0.08 | 0.06 | 0.05 | 0.04 | 0.03 | 0.02 | 0.02 | 0.02 | 0.01 | 0.01 | 0.01 | 0 |
|  |  | 5%-tile | 3.19 | 1.78 | 1.37 | 1.13 | 0.93 | 0.72 | 0.5 | 0.29 | 0.27 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.14 | 0.06 |

Table 2.1.1.2-2: Interpolated ACIR values for Scenario 1 to meet the 5% throughput loss criteria

| Elevation angle | Throughput Loss | Interpolated ACIR[dB] |
| --- | --- | --- |
| 90° | Average | 0 |
|  | 5%-tile | 15.21 |
| 35° | Average | 0 |
|  | 5%-tile | 0 |

ZTE have made the following observation

Observation 1: The required ACIR for NTN Ku band coexistence is as follows:

| Scenario |  | 1 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| Frequency |  | 14GHz | 14GHz | 11GHz | 11GHz |
| Scenario case |  | Aggressor NTN UL to Victim TN UL | Aggressor TN DL to Victim NTN UL | Aggressor TN DL to Victim NTN DL | Aggressor NTN DL to Victim TN DL |
| Required ACIR[dB] | GEO | 21.03 | 28.65 | 38.58 | 0 |
|  | LEO600 | 0 | 38.46 | >50 | 0 |

# R4-2507169, Samsung, Discussion on Ku-band coex. study

In this contribution, Samsung have provided 3D simulation plots of the 45x45 and 78x78 element phase array antennas.

Samsung have provided the normalised antenna radiation and the EIRP distribution of both 45x45 and 78x78 antennas at 90- degree and 35-degree elevation.

Figure 5. The e.i.r.p. distribution of a 78x78 phased array with its normal direction to 90-degree in elevation

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

Figure 2. The e.i.r.p. distribution of a 45x45 phased array with its normal direction to 90-degree in elevation

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

Furthermore, Samsung have illustrated that for a 78x78 phase array antenna, the performance in 90-degree exceeds the CEPT masks.

Phased array off-axis e.i.r.p.:

Figure 6. 78x78 phased array: compliance to CEPT off-axis e.i.r.p limits in meas. BW 40kHz

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

Samsung have made the following observations:

Observation 1: The above calculation has slightly difference to the ‘Max EIRP value’ agreed in [1], my calculation shows max eirp is around 74.6 dBm, while the agreed Max EIRP value in [1] is 76.6 dBm, there’s 2dB difference which equals to the ohmic loss.

Observation 2: The max e.i.r.p. will drop around 5dB when purely utilizing electrical beam steering with a 55-degree in elevation domain.

Observation 3: The 45x45 phased array, when operating in 5MHz as minimum supported chbw in this release, when electrically steered to 35 degrees elevation angle probably don’t need any power reduction for compliance to off-axis eirp limits to CEPT.

Observation 4: The compliance of this 45x45 phased array antenna to FCC and other regional/country-wide off-axis eirp limits need further study.

Observation 5: The above calculation has slightly difference to the ‘Max EIRP value’ agreed in [1], my calculation shows max eirp is around 86.2 dBm, while the agreed Max EIRP value in [1] is 88.2 dBm, there’s 2dB difference which equals to the ohmic loss.

Observation 6: The max e.i.r.p. will drop around 5dB when purely utilizing electrical beam steering with a 55-degree in elevation domain, similar as the results from 45x45 antenna.

Observation 7: The NTN VSAT with 78x78x1 phased array parameters agreed in [1] would result in exceedance to the CEPT off-axis eirp limit given in [3], even without any electrical beam steering, when transmitting in 5MHz channel bandwidth.

Observation 8: Given the phased array is not really considered in previous NTN studies, and these parameters are newly proposed. We believe a short calibration phase before collecting co-ex results can be helpful.

Samsung have proposed the following:

Proposal 1: Discuss and if agreed, to update the ‘Max EIRP value’ for 45x45 phased array antenna in [1] to 74.6 dBm by applying ohmic loss, which seems was not applied.

Proposal 2: Discuss and if agreed, to update the ‘Max EIRP value’ for 78x78 phased array antenna in [1] to 86.2 dBm by applying ohmic loss, which seems was not applied.

Proposal 3: To have the proponent of this 78x78x1 phased array antenna to double check if the parameters needs any update and/or if they target on larger channel bandwidth than 5MHz, considering our Observation 7.

Proposal 4: No need to consider the power adjustment in co-existence study assuming 100MHz, and the UE RF requirement should reflect the reduction may be required for certain cases at least similar to Ka-band VSAT.

Moderator’s note: Samsung uses 5MHz channel bandwidth for the analysis.  The minimum channel bandwidth for NTN is 10MHz.

# R4-2507672, Ericsson, On co-existence simulation assumptions for NR NTN Ku band

In this contribution, Ericsson have provided calibration assumption for the Ku Band co-existence analysis.

For the calibration assumptions, Ericsson are proposing the following:

Proposal 1	For the NTN calibration assumptions, only 90 degrees elevation angle and Urban scenario can be considered for both GEO and LEO to minimize the simulation efforts.

Proposal 2	For the VSAT calibration assumptions, Table 6a.2.2.2-1 which states VSAT antenna characteristics in the Annex below can be adopted as baseline with VSAT NF of 6 dB.

Proposal 3	For the calibration metrics, the calibration results should include SINR and coupling loss distributions in DL, and SINR, coupling loss and transmit power distributions in UL at 5%, 50% and 95%iles on the CDF.

Ericsson have also provided results of the co-existence analysis with a 60cm Parabolic antenna for both GEO and LEO cases.

Table 2.2-1 Required ACIR at 5%-ile

|  |  |  |  |  | ACIR (at 5% tile) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | GEO |  | LEO600 |  |
| Prioritized Scenarios | Aggressor | Victim | Contributing | Frequency | 90 degrees | 35 degrees | 90 degrees | 35 degrees |
| 1 | NTN GEO UL | TN UL | NTN UE ACLR | 14 GHz | 2 | 3 | ~0 | 8 |
| 4 | TN DL | NTN GEO UL | NTN SAN GEO ACS | 14 GHz | ~0 | 16 (avg) | 9 | 22 |
| 5 | TN DL | NTN GEO DL | NTN UE ACS | 11 GHz | 35 | 30 (avg) | 35 | 30 (avg) |
| 6 | NTN GEO DL | TN DL | NTN SAN GEO ACLR | 11 GHz | ~0 | ~0 | ~0 | ~0 |

# R4-2507673, Ericsson, On co-existence simulation results for NR NTN Ku band

In this contribution Ericsson provides initial co-existence analysis results using a 60 cm Parabolic antenna for both GEO and LEO satellites.

The following table illustrated the GWO simulation results:

|  |  |  |  |  | ACIR at 5% tile |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 90 degrees | 35 degrees |
|  |  |  |  |  | Mobile VSAT - Parabolic | Mobile VSAT - Parabolic |
| Scenario | Aggressor | Victim | Contributing | Frequency | 2.5 dB NF | 2.5 dB NF |
| 1 | NTN GEO UL | TN UL | NTN UE ACLR | 14 GHz | 2 | 3 |
| 2 | TN UL | NTN GEO UL | NTN SAN GEO ACS | 14 GHz | 9 | 10 (avg) |
| 3 | NTN GEO UL | TN DL | NTN UE ACLR | 14 GHz | ~0 | ~0 |
| 4 | TN DL | NTN GEO UL | NTN SAN GEO ACS | 14 GHz | 0 | 16 (avg) |
| 5 | TN DL | NTN GEO DL | NTN UE ACS | 11 GHz | 35 | 30 (avg) |
| 6 | NTN GEO DL | TN DL | NTN SAN GEO ACLR | 11 GHz | ~0 | ~0 |
| 7 | NTN GEO DL | TN UL | NTN SAN GEO ACLR | 11 GHz | ~0 | ~0 |
| 8 | TN UL | NTN GEO DL | NTN UE ACS | 11 GHz | 26 | 18 (avg) |

And the following table illustrates the results for the LEO case

|  |  |  |  |  | ACIR at 5% tile |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 90 degrees | 35 degrees |
|  |  |  |  |  | Mobile VSAT - Parabolic | Mobile VSAT - Parabolic |
| Scenario | Aggressor | Victim | Contributing | Frequency | 2.5 dB NF | 2.5 dB NF |
| 1 | NTN LEO600 UL | TN UL | NTN UE ACLR | 14 GHz | ~0 | 8 |
| 2 | TN UL | NTN LEO600UL | NTN SAN LEO600 ACS | 14 GHz | 16 | 14 |
| 3 | NTN LEO600 UL | TN DL | NTN UE ACLR | 14 GHz | ~0 | ~0 |
| 4 | TN DL | NTN LEO600 UL | NTN SAN LEO600 ACS | 14 GHz | 9 | 22 |
| 5 | TN DL | NTN LEO600 DL | NTN UE ACS | 11 GHz | 35 | 30 (avg) |
| 6 | NTN LEO600 DL | TN DL | NTN SAN LEO600 ACLR | 11 GHz | ~0 | ~0 |
| 7 | NTN LEO600 DL | TN UL | NTN SAN LEO600 ACLR | 11 GHz | ~0 | ~0 |
| 8 | TN UL | NTN LEO600 DL | NTN UE ACS | 11 GHz | 26 | 20 (avg) |

The scenarios highlighted in green are the scenarios that were agreed to be studied as they provide the worst interference cases.

# Moderator’s Summary

10.1 VSAT Phase Array Antennas

- Majority of ACIR calculations are performed for 60 cm Parabolic antenna and 45x45 element phase array antenna

- Companies are uncertain of the 78x78 element phase array antenna: this would require discussion and further clarification.

- On the 78x78 element Antenna at 90-degree and 35-degree elevation; to comply with the FCC and CEPT mask we can assume 100MHz channel bandwidth for uplink an downlink.

10.2 Parabolic Antenna

- Should the UL bandwidth of the Parabolic antenna to be reduced to 10 MHz to close the UL Link Budget for the GEO case?

10.3 Throughput Performance

- The Cell Edge Data Throughput results / losses are below expectations; should we instead use the Average Cell Throughput Results

10.4 Clarification is needed on the following topics/ contributions:

- What is the guard band between the terrestrial and the non-terrestrial Ku band for all contributions?

- Only one company has provided Calibration data. Is this set sufficient for future calibration or should we ask other companies to supply Calibration data for comparison?

10.5 Practical Considerations

The aim of this series of calculations are to extract the ACLR and ACS odthe Ku Band Non-Terrestrial system in close proximity of a fictitious terrestrial system operting in the Ku band.

Whilst useful data has been generated by many companies for the ACIR, we must also considert the practical implications of specifying ACLR and ACS characterisitcs which are impractical or very costly to implement.

Therefore input is required at a later stage to better understand the impementation limitations of various ACLR and ACS values that are being considered.

10.6 FCC and CEPT Mask cCmpiance

For the co-existence simulation, the channel bandwidth is 100MHz.  As a result, the 78x78 element antenna will not exceed the FCC or CEPT mask.  However, it is important to bring forward to the group the FCC and CEPT mask non-compliance at lower channel bandwidth (i.e., 10 MHz).

10.7 Summarize of simulation results.

| ACIR（worst case btw 35/90 deg） |  |  | Xiaomi R4-25057620 | Qualcomm R4-2505747 | ZTE R4-2506258 | Ericsson R4-2507672 |
| --- | --- | --- | --- | --- | --- | --- |
| scenario 1 | GEO | Phase array | 3.6 | 25/25 | 15 |  |
|  |  | Parabolic | 9.6 | 20/15 | 21 | 3 |
|  | LEO | Phase array | 0 | 25/0 | 0 |  |
|  |  | Parabolic | 2.1 | 20/0 | 0 | 8 |
| scenario 4 | GEO | Phase array | 10.3 | 20/15 | 28.21 |  |
|  |  | Parabolic | 9.9 | 20/15 | 28.7 | 16 |
|  | LEO | Phase array | 39.5 | 25/25 | 38.46 |  |
|  |  | Parabolic | 37.7 | 30/25 | 38.46 | 22 |
| scenario 5 | GEO | Phase array | 18.3 | 45/45 | 18.37 |  |
|  |  | Parabolic | 25 | 40/40 | 38.58 | 35 |
|  | LEO | Phase array | 19.3 | 45/45 | 24.36/>40 |  |
|  |  | Parabolic | 26.2 | 40/40 | 41.32 | 35 |
| scenario 6 | GEO | Phase array | 0 | 0 | 0 |  |
|  |  | Parabolic | 0 | 0 | 0 | 0 |
|  | LEO | Phase array | 0 | 0 | 0 |  |
|  |  | Parabolic | 0 | 0 | 0 | 0 |