# 3GPP TSG-RAN WG4 Meeting #115	R4-2508657

# Malta, MT, 19th – 23rd May, 2025

Title: 	Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE: CLTA

Agenda Item:	7.13.1

Source: 	Huawei, HiSilicon

Document for:	Approval

## Sub-topic 1-1: BS-to-BS isolation analysis

Way forward: The following data is considered baseline for the BS-BS isolation conclusions derivation:

- No beam steering consideration; intra-band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | ZTE (R4-2504358, M): -54 (SA-to-SA) | ZTE (R4-2504358, M): -52 (SA-to-SA) | ZTE (R4-2504358, M): -56 (SA-to-SA) |  |
| Isolation in horizontal domain (dB) | ZTE (R4-2504358, M): -31 (SA-to-SA) | ZTE (R4-2504358, M): -34 (SA-to-SA) Huawei (R4-2504594, M): -46 Nokia (R4-2507633, M): -35 (A-to-SA) | ZTE (R4-2504358, M): -46 (SA-to-SA) Huawei (R4-2507445, M): -50.6 (A-to-SA) | Nokia (R4-2507633, M): -55 (A-to-SA) |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling - SA-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; intra-band; same BS type:

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | Ericsson (R4-2506307, S): A-to-SA: -55±15 dB | ZTE (R4-2507511, M): -69 dB (A-to-SA) | Ericsson (R4-2506307, S): A-to-A: -77±15; A-to-SA: -70±15 |

- Beam steering consideration; inter-band:

|  | 3.5 GHz ÷ 7 GHz |
| --- | --- |
| Isolation in vertical domain (dB) |  |
| Isolation in horizontal domain (dB) | Ericsson (R4-2506307, S): A-to-A: -54±10 dB A-to-SA: -57±10 dB |

Based on date captured in bullets above, the following BS-BS isolation data summary is extracted:

Intra-band BS-BS isolation results

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | -54 | -52 | -56 |  |
| Isolation in horizontal domain (dB) | -31 | -34 | -46 | -55 |

As per previous agreement in R4-2504701 (RAN4#114bis), further BS-to-BS isolation data provision during RAN4#116 (August 2025) is not precluded.

## Sub-topic 1-2: Impacted RF requirements

Way forward:

- Based on the BS-to-BS isolation data, no RF (including core and conformance test) co-location requirements impact below 4.9 GHz was agreed.

- OTA TX IMD:

  - Impact on the OTA TX IMD requirement is FFS, considering the argument that this is a regulatory requirement.

  - Provide further inputs to address the testability issue.

  - Further investigate impact of higher than 43dBm BS output power.

## References

[1]	R4-2504701	Way Forward for [114bis][303] NR_BS_RF_Part2_CLTA_SE, Huawei