3GPP TSG-RAN4 Meeting #114-bis	R4-2508660

Malta , MT, 19th – 23st, May. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.870 | CR | 0027 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for split TRP/TRS measurement grids method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRP_TRS_MIMO_OTA_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of test methodology to allow devices to be tested without changing/charging the battery and multiple device re-positionings |  |  |  |  |  |  |  |  |
| Summary of change: |  | Split the measurement grid up into multiple coarser grids, perform each TRP/TRS measurement on the coarse grid without the need for a battery change/charge, and then average the different results. This approach will allow TRP/TRS to be measured with the original, classical measurement grid. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some devices may need to change the battery during one long time OTA test, which yields higher uncertainty of the measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.11.5, 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion paper was previously submitted in R4-2413195 and related data analysis is submitted in R4-2505850. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2507366 |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

### B.2.11.5	Positioning Uncertainty from using Phantoms

Some uncertainty also occurs from the positioning of the DUT against the SAM phantom, as the DUT cannot be attached exactly in the same way every time. This uncertainty depends on how much the DUT’s positioning against the SAM phantom and hand phantoms varies from the specified testing positions. It is noted that the uncertainty of the phone positioning depends on the phone holder and the measurement operator and is in fact difficult to distinguish from random uncertainty. Some uncertainty also occurs from the positioning of the DUT plugged into the Laptop Ground Plane phantom, as the DUT may not be plugged into the USB connector and positioned exactly in the same way every time. This uncertainty depends on how much the DUT’s position plugged into the Laptop Ground Plane phantom varies from the specified plug-in position. Therefore, the positioning uncertainty is included in random uncertainty.

For devices with limited battery capacity, TRP/TRS measurements might not be performed with all grid points assessed in a single session, i.e., without changing/charging the battery and multiple device repositionings. The repositioning MU for TRP/TRS measurement approaches that require multiple repositionings, e.g., classical grids with full power or split measurement grids, shall therefore take multiple device positionings into account in the MU calculation.

To estimate this uncertainty for the SAM phantom, it is suggested to perform at least five evaluations of TRP/TRS whereby the device shall be dismounted and newly positioned with a fully charged battery before each test. This measurement set has to be carried out in mid channel of lowest and highest frequency bands utilized by the testing lab, for at least three phones with different type of mechanical design. The values have to be normalized by the mean for each measurement set. As a result, the uncertainty contribution entered to uncertainty budget is the difference between the maximum and minimum normalized values.

With head and hand phantoms, random uncertainty evaluation may be done separately for each measurement configuration i.e. head only, browsing mode or speech mode. A speech mode random uncertainty evaluation, were both head and hand phantoms are used, can reasonably be considered to be the worst-case scenario and thus random uncertainties in other configurations to be less.

To estimate this uncertainty for the Laptop Ground Plane phantom, it is suggested to perform at least five evaluations of TRP/TRS for the plug-in position whereby the device shall be dismounted and newly positioned before each test. This measurement set has to be carried out in mid channel of lowest and highest frequency bands utilized by the testing lab, for at least three USBs with different type of mechanical design. The values have to be normalized by the mean for each measurement set. As a result, the uncertainty contribution entered to uncertainty budget is the difference between the maximum and minimum normalized value.

For the forearm phantom, the assessment of uncertainties for the phantoms defined in CTIA Certification OTA Test Plan 01.72 Section 2 [25], is further detailed in CTIA Certification OTA Test Plan 01.70 Section 2 [26]. Values are from the CTIA Certification Test Plan for Wireless Device Over-the-Air Performance © CTIA Certification. Reproduced with permission.

<<< Skip Unchanged Sections >>>

## 10.3 TRP/TRS OTA test based on averaging multiple split measurement grids

Editor’s note: The positioning uncertainty from using phantoms for limited battery capacity device is FFS. The final MTSU (Maximum Test System Uncertainty) limit may not be relaxed to accommodate the worst-case test method due to the impact on test tolerance. This decision will need to be made after the MU analysis.

The classical TRP measurement requires 266 (for the Clenshaw-Curtis quadrature) or 264 (for the sin(theta) quadrature) unique grid points to properly assess the radiated TX performance as outlined in Clause 5.1.1.2. Similarly, the classical TRS measurement requires 64 (Clenshaw-Curtis) or 62 (sin(theta)) unique grid points to properly assess the radiated RX performance. Some devices/device types with limited battery capacity might not support TRP/TRS testing performed using the full set of grid points without changing/charging the battery and multiple device re-positionings. Splitting the grid up into multiple coarser grids, performing each TRP/TRS measurement on the coarse grid without the need for a battery change/charge, and then averaging the different results would allow an accurate assessment of TRP/TRS performance without the need for device re-positionings during the TRP/TRS procedures.

A test grid with 266 test points, considered as the fine measurement grid (fine grid), can be split into four 64-grid points measurement grids (split into 2, 3 or other N numbers of sub-grids are also possisble, and N number of sub-grids still correspond to a fine measurement grid), considered here as split coarse measurement grid (split grid). The measurement grid points in each of split grid is a subset of that of the fine grid. Other than the grid points at the poles, the measurement grid points of one split grid are different from any measurement grid points in another split grid. However, the combination of the measurement points of all the split grid is still the same as the number of measurement points in the fine grid.

For such approximation, it is possible to split the one fine grid with 266 points into 4 coarse grids in different ways. The results of split grid should be that the measurement points are equally spreading around the sphere in the favour of the test procedure can be easily design for facilitate the OTA test.

One example of splitting 266 fine grids into 4x68 interpolated grids is illustrated in Figures 10.3-1 and 10.3-2.

Figure 10.3-1: 266 measurement grid point for TRP.

Figure 10.3-2: Split 4x68 measurement grids

To achieve the same accuracy of TRP measurement results based on the 4x68-points split grids compared with using 266-points fine grids, we first use each of the 68-points split grids in the OTA testing of TRP according to the procedure in Clause 7.4.2.2. Repeat the OTA test four times to get the TRP1, TRP2, TRP3, and TRP4.

Then, the TRP can be calculated as in the equation below:

TRP=10log10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104

The calculation of the TRP1/TRP2/TRP3/TRP4 based on EIRP measurements follows Clause 5.1.1.2 for TRP.

Similarly, using the same method to measure TRS by using four split grids in the OTA test which follows Clause 5.2.1 for TRS1 through TRS4. Then the TRS can be calculated as in the equation below:

TRS=10log4/110TRS110+110TRS210+110TRS310+110TRS410

<<< END OF CHANGES >>>