3GPP TSG-RAN WG4 Meeting #115	R4-2508704

Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0777 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PDSCH demodulation performance requirements and Reference Channel for Rel-18 8Rx CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2025-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The DMRS RE number for 8 layers is 48 without PDSCH multiplexing for 8Rx CA,however some test requirements are based on 24 REs, i.e., the reference channels for FDD 10MHz and TDD 40MHz should be corrected to the ones with 48 DMRS REs, and one Reference Channel Table should be corrected with 48 DMRS REs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - update Table 5.2A.4.1-3 and Table 5.2A.4.1-6 in Clause 5.2A.4.1 - update Table A.3.2.1.1-21 in Clause A.3.2.1.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDSCH Reference Channels for 8Rx CA in 5.2A.4.1-3, Table 5.2A.4.1-6 and Table A.3.2.1.1-21 are not aligned with the test parameters description in Table 5.2A-1, may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.4.1, A.3.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2506672 |  |  |  |  |  |  |  |  |

<Start of changes 1>

5.2A.4.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.4.1-7 or Table 5.2A.4.1-8 (dependent on Baseline or Simplified SU-MIMO 8Rx Receiver), the requirements are based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.4.1-1 ~ Table 5.2A.4.1-6, with the parameters in Table 5.2A-1 ~ Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.4.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-22.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 15 | R.PDSCH.1-22.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 20 | R.PDSCH.1-22.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 25 | R.PDSCH.1-22.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 30 | R.PDSCH.1-23.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 35 | R.PDSCH.1-23.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 40 | R.PDSCH.1-23.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 45 | R.PDSCH.1-23.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.9 |
| 50 | R.PDSCH.1-23.5 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 14.0 |

Table 5.2A.4.1-2: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx  Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-22.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 15 | R.PDSCH.1-22.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 20 | R.PDSCH.1-22.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 25 | R.PDSCH.1-22.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 30 | R.PDSCH.1-23.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.2 |
| 35 | R.PDSCH.1-23.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 40 | R.PDSCH.1-23.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.2 |
| 45 | R.PDSCH.1-23.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |
| 50 | R.PDSCH.1-23.5 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.4 |

Table 5.2A.4.1-3: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-20.1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.6 |
| 10 | R.PDSCH.1-3.8 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.7 |
| 15 | R.PDSCH.1-20.2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.9 |
| 20 | R.PDSCH.1-20.3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 25 | R.PDSCH.1-20.4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 30 | R.PDSCH.1-21.1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.4 |
| 35 | R.PDSCH.1-21.2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.6 |
| 40 | R.PDSCH.1-21.3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.9 |
| 45 | R.PDSCH.1-21.4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.7 |
| 50 | R.PDSCH.1-21.5 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.5 |

Table 5.2A.4.1-4: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-36.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.3 |
| 10 | R.PDSCH.2-36.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.5 |
| 15 | R.PDSCH.2-36.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 20 | R.PDSCH.2-36.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 25 | R.PDSCH.2-36.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 30 | R.PDSCH.2-36.6 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 50 | R.PDSCH.2-37.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 60 | R.PDSCH.2-37.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 80 | R.PDSCH.2-37.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 14.0 |
| 90 | R.PDSCH.2-37.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 100 | R.PDSCH.2- 37.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |

Table 5.2A.4.1-5: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-36.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 10 | R.PDSCH.2-36.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 15 | R.PDSCH.2-36.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 20 | R.PDSCH.2-36.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 25 | R.PDSCH.2-36.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 30 | R.PDSCH.2-36.6 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 50 | R.PDSCH.2-37.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 60 | R.PDSCH.2-37.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 80 | R.PDSCH.2-37.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |
| 90 | R.PDSCH.2-37.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.4 |
| 100 | R.PDSCH.2- 37.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |

Table 5.2A.4.1-6: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-34.1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.7 |
| 10 | R.PDSCH.2-34.2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.6 |
| 15 | R.PDSCH.2-34.3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.9 |
| 20 | R.PDSCH.2-34.4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.8 |
| 25 | R.PDSCH.2-34.5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.8 |
| 30 | R.PDSCH.2-34.6 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 40 | R.PDSCH.2-3.8 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.1 |
| 50 | R.PDSCH.2-35.1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.0 |
| 60 | R.PDSCH.2-35.2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.4 |
| 80 | R.PDSCH.2-35.3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.6 |
| 90 | R.PDSCH.2-35.4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.7 |
| 100 | R.PDSCH.2- 35.5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.8 |

<End of changes 1>

<Start of changes 2>

A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

<Unchanged sections omitted>

Table A.3.2.1.1-21: PDSCH Reference Channel for FDD CC and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-21.1 FDD | R.PDSCH.1-21.2 FDD | R.PDSCH.1-21.3 FDD | R.PDSCH.1-21.4 FDD | R.PDSCH.1-21.5 FDD |
| Channel bandwidth | MHz | 30 | 35 | 40 | 45 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 160 | 188 | 216 | 242 | 270 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 4 | 4 | 4 | 4 | 4 |
| Number of DMRS REs |  | 48 | 48 | 48 | 48 | 48 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 155776 | 184424 | 213176 | 237776 | 262376 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 19 | 22 | 26 | 29 | 32 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 345600 | 406080 | 466560 | 522720 | 583200 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 368640 | 433152 | 497664 | 557568 | 622080 |
| Max. Throughput averaged over 2 frames | Mbps | 147.987 | 175.203 | 202.517 | 225.887 | 249.257 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Two codewords and given per codeword |  |  |  |  |  |  |

<End of changes 2>