3GPP TSG-RAN WG4 Meeting #115                                                               R4-2508708

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.25.1

Source:	Samsung

Title:	Way Forward on UE-UE CLI problem statement and potential solutions

Background:

- Based on Rel-18 study, the following conclusion is provided for Case 1 (aggressor SBFD DU victim NR TDD DL) in the conclusion part of Chapter 11 in TR 38.858:

### 11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.1-1. Table 11.3.1-1: Case 1 co-existence conclusions Deployment Scenario (Aggressor -> Victim); Frequency range; Co-existence conclusion. Urban Macro -> Urban Macro Scenario 1 and Scenario 6; FR1 and FR2-1; No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx powers (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and different SBFD BS antenna configurations.. Urban Hotspot -> Urban Hotspot Scenario 2; FR1; DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid-shifts (5% to 100%) and BS Tx powers (46 dBm to 53 dBm).. Urban Macro -> Urban Micro Scenario 4; FR1; No DL throughput degradation for both average throughput and cell edge throughput is observed.. Indoor -> Indoor Scenario 3 and Scenario 9; FR1 and FR2-1. Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8; FR1 and FR2-1.

- In RAN4#114, the proposal is proposed to introduce new mechanism for limiting maximum UE transmit power in SBFD symbols/slots [R4-2500474].

- In RAN4#114bis, it is highlighted that the mechanism is for the single operator scenario in which a single operator owning both networks, SBFD and legacy TDD. In the corresponding proposal, it is stated that UE-to-UE CLI could be mitigated, by using DUD (40%, 20%, 40%), and by reducing PCMAX of the SBFD UE power by XdB [R4-2503235].

- In RAN4#115, the following proposals are provided in the Tdoc [R4-2505459]:

  - Proposal 1: Reduce pMax of the SBFD UE power by XdB.

  - Proposal 2: Send LS from RAN4 to RAN1 asking to approve our agreement

  - Proposal 3: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%).

Conclusion from RAN1#121:

- In RAN1#121, the following conclusion is provided on this topic:

Conclusion Separate configuration of Pcmax/p-max is a RAN4 issue and will not be discussed in RAN1. It is up to RAN4 make decision on this issue. As part of RAN1 study on this issue, one company submitted simulation results in R1-2504391.

Way Forward

- UE-to-UE CLI problem statement:

  - The scenario to be considered:

    - Intra-operator adjacent-channel inter-cell (i.e., both channels belong to a single operator)

      - Use Urban Hotspot -> Urban Hotspot Scenario 2 as reference

        - FFS non-located assumption is always valid or not

        - detailed description provided in TR38.858

  - The case to be considered:

    - SBFD-aware UE (aggressor) UL transmission to NR TDD UE (victim) DL reception

- WF-1: Companies are encouraged to provided analysis on the above UE-to-UE CLI problem:

  - The purpose is to identify whether the above-mentioned UE-to-UE CLI problem exists or not in the practical deployment

  - One evaluation method is to re-perform RAN4 Rel-18 co-existence study on the above scenario/case

    - Parameters agreed in Rel-18 will be reused unless difference identified

    - FFS impact from different grid shift values

    - FFS impact from ACLR model

- WF-2: Companies are encouraged to identify potential solution(s) to the above UE-to-UE CLI problem:

  - Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB.

    - the reduced p-max is configurable per-UE

    - FFS X dB is a single value or a configurable value or a configurable range

  - Solution-2: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel)

  - Solution-3: Avoid scheduling the aggressor UE, up to BS implementation.

  - Solution-4: Configure the aggressor UE fallback to TDD operation, up to BS implementation.

  - Other new solutions (including the combination of the above-mentioned solutions) are encouraged to be provided, if any.

- WF-3: Companies are encouraged to provide analysis on solutions, by at least considering the following aspects:

  - Benefit(s):

    - At least for Solution-1, one evaluation method is to perform RAN4 co-existence study on the above scenario/case, by using XdB lower maximum UE power for aggressor UE(s)

      - FFS how to determine one SBFD-aware UE is aggressor UE

      - The value of X can be chosen from the range of [1, 5] and other values may not be precluded.

      - How to determine the value of X or the range can be FFS

      - Baseline to be compared to is Rel-18 coexistence study re-performed in WF-1

      - Detailed parameters can be further discussed by conference call and/or email discussion before RAN4#116.

    - Other methods are not precluded

  - Limitation(s), e.g., decreased coverage for certain SBFD-aware UE, etc.

  - Expected RAN4 requirement impact, including

    - the impact to TS 38.104

    - the impact to TS 38.101 series, e.g., Pcmax, etc.

  - Expected RAN2 requirement impact:

    - whether new or changed RAN2 signaling(s) is required:

  - Expected RAN3 requirement impact:

    - whether new or changed RAN3 signaling(s) is required:

  - The following table can be used to summarize the above aspects in the future RAN4 meeting:

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution-1 |  |  |  |  |  |
| Solution-2 |  |  |  |  |  |
| Solution-3 |  |  |  |  |  |
| Solution-4 |  |  |  |  |  |
| New solution if any |  |  |  |  |  |