3GPP TSG-RAN WG4 Meeting # 115                                                 R4-2508709

Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | Draft | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TS 38.104 to introduce conducted and OTA out-of-band blocking for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conducted and OTA out-of-band blocking requirements for SBFD capable BS need to be introduced in TS 38.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For the conducted out-of-band blocking requirements of SBFD capable BS, - The general requirements are introduced in clause 12.3.5.1. - The minimum requirements are introduced in clause 12.3.5.2. - The minimum requirements for co-location are introduced in clause 12.3.5.3. - For the OTA out-of-band blocking requirements of SBFD capable BS, - The general requirements are introduced in clause 12.6.6.1. - The minimum requirement for BS type 1-O is introduced in clause12.6.6.2, including general minimum requirement specifed in clause 12.6.6.2.1 and minimum requirements for co-location specifed in clause 12.6.6.2.2. - The minimum requirements for BS type 2-O are introduced in clause12.6.6.3, including general minimum requirements specifed in clause 12.6.6.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conducted and OTA out-of-band blocking requirements for SBFD capable BS are missing in the TS 38.104. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.5, 12.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

### 12.3.5	Out-of-band blocking for SBFD capable BS

#### 12.3.5.1 General

The general requirement for SBFD capable BS is the same as general requirement specified in clause 7.5.1.

#### 12.3.5.2 Minimum requirement for SBFD capable BS type 1-H

The minimum requirements are the same as those for the BS type 1-H specified in clause 7.5.2, except for PREFSENS, and the PREFSENS depends on the SBFD UL subband as specified in clause 12.3.2.

#### 12.3.5.3 Co-location minimum requirements for SBFD capable BS type 1-H

The co-location minimum requirements are the same as those for the BS type 1-H specified in clause 7.5.3, except for PREFSENS, and the PREFSENS depends on the SBFD UL subband as specified in clause 12.3.2.

<End of Change>

<Start of Change>

### 12.6.6	OTA out-of-band blocking for SBFD capable BS

12.6.6.1 General

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

### 12.6.6.2	Minimum requirement for SBFD capable BS type 1-O

#### 12.6.6.2.1	General minimum requirement

The general minmum requirements are the same as those specified in clause 10.6.2.1, except for EISminSENS. The EISminSENS depends on the SBFD capable BS class and the SBFD UL subband, and it is specified in clause 12.6.2.

#### 12.6.6.2.2	Co-location minimum requirement

The co-locatioon minmum requirements are the same as those specified in clause 10.6.2.2, except for EISminSENS. The EISminSENS depends on the SBFD capable BS class and the SBFD UL subband, and it is specified in clause 12.6.2.

### 12.6.6.3	Minimum requirement for SBFD capable BS type 2-O

#### 12.6.6.3.1	General minimum requirement

The general minmum requirements are the same as those specified in clause 10.6.3.1, except for EISREFSENS. The EISREFSENS depends on the SBFD capable BS class and the SBFD UL subband, and it is specified in clause 12.6.3.

<End of Change>