3GPP TSG-RAN WG4 Meeting #115 	R4-2508714

Malta, 19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.104 on Reference sensitivity and dynamic range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of requirements for SBFD-capable BS to TS 38.104 for package 8 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes is made in 12.3.2 Reference sensitivity level for SBFD 12.3.3 Dynamic range for SBFD 12.6.2 OTA sensitivity for SBFD 12.6.3 OTA reference sensitivity level for SBFD 12.6.4 OTA dynamic range for SBFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The number of test cases are unnecessary large. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

12.3	Conducted receiver characteristics for SBFD-capable BS

12.3.1	General

12.3.2	Reference sensitivity level

12.3.2.1	General

The reference sensitivity power level is the minimum mean power received at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation at which a throughput requirement shall be met for a specified reference measurement channel.

12.3.2.1	Minimum requirements for BS type 1-H

The same requirement as clause 7.2.2 is applicable to reference sensitivity power level for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.3.3	Dynamic range

#### 12.3.3.1	General

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

#### 12.3.3.2	Minimum requirement for BS type 1-H

The same requirement as clause 7.3.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.6.2	OTA sensitivity for SBFD

12.6.2.1	BS type 1-H and BS type 1-O

12.6.2.1.1	General

The general description of OTA sensitivity requirement as clause 10.2.1.1 shall also be applied for SBFD operation.

12.6.2.1.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in clause 7.2 shall be met when the level of the arriving signal is equal to the minimum EIS level with 0.7dB degradation allowed in the respective declared set of EIS level and UL subband bandwidth, in SBFD symbols with the transmitted signals in SBFD DL subband(s) present.

12.6.3	OTA reference sensitivity level

12.6.3.1	General

The general description of OTA reference sensitivity requirement as clause 10.3.1 shall also be applied for SBFD operation.

12.6.3.2	Minimum requirement for BS type 1-O

The OTA reference sensitivity requirement as specified in clause 10.3.2 with 0.7dB degradation allowed is applicable for SBFD operation in SBFD symbols with the transmitted signals in SBFD DL subband(s) present, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.6.3.3	Minimum requirement for BS type 2-O

The OTA reference sensitivity requirement as specified in clause 10.3.3 with 0.7dB degradation allowed is applicable for SBFD operation in SBFD symbols with the transmitted signals in SBFD DL subband(s) present, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.6.4	OTA dynamic range

#### 12.6.4.1	General

The general description of OTA dynamic range requirement as clause 10.4.1 shall also be applied for SBFD operation.

#### 12.6.4.2	Minimum requirement for BS type 1-H

The same requirement as clause 10.4.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

<End of changes>