3GPP TSG-RAN4 Meeting #115	R4-2508716

# Malta, MT,  19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.104 on In-channel leakage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of the in-channel adjacent sub-band leakage for SBFD BS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the in-channel adjacent sub-band leakage requirement for SBFD BS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In-channel adjacent sub-band leakage requirement requirement for SBFD-capable BS is not well defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 12.2.9 and 12.5.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

12.2.9	In-channel adjacent subband leakage for SBFD

12.2.9.1 General

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of UL sub-band configuration within one NR carrier.

The requirement shall apply during the transmitter ON period.

12.2.9.2 Limits and Basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 12.2.9.2-1.

Table 12.2.9.2-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Category A Wide Area BS | TBD dBm/MHz |
| Category B Wide Area BS | TBD dBm/MHz |
| Medium Range BS | TBD dBm/MHz |
| Local Area BS | TBD dBm/MHz |

Table 12.2.9.2-2: Filter parameters for the UL sub-band

| RAT | Filter on the assigned UL sub-band and corresponding filter bandwidth |
| --- | --- |
| NR | NR of the closet BW with SCS that provides largest transmission bandwidth configuration compared with UL sub-band |

12.2.9.3 Minimum requirement for SBFD BS type 1-H

The ICASL requirements for SBFD BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clause 12.2.9.2, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the SBFD BS type 1-H spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the in-channel adjacent sub-band leakage power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The  in-channel adjacent sub-band leakage power at each TAB connector shall be less than or equal to the SBFD BS type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

12.5.10	OTA In-channel adjacent subband leakage for SBFD

12.5.10.1 General

OTA In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of UL sub-band configuration within one NR carrier. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON period.

12.5.10.2 Minimum requirement for SBFD BS type 1-O

For SBFD BS type 1-O, the ICASL shall less than or equal to a basic limit defined in table 12.2.9.2-1 + X (where X = 9 dB).

12.5.10.3 Minimum requirement for SBFD BS type 2-O

For SBFD BS type 2-O, the ICASL shall be less than or equal to the value specified in table 12.5.10.3-1.

Table 12.5.10.3-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Wide Area BS | TBD dBm/MHz |
| Medium Range BS | TBD dBm/MHz |
| Local Area BS | TBD dBm/MHz |

Table 12.5.10.3-2: Filter parameters for the UL sub-band

| RAT | Filter on the assigned UL sub-band and corresponding filter bandwidth |
| --- | --- |
| NR | NR of the closet BW with SCS that provides largest transmission bandwidth configuration compared with UL sub-band |

<End of change 1>