3GPP TSG-RAN WG4 Meeting #115	R4-2508724

# Malta, MT, 19th - 23rd May, 2025

Title: 	Way Forward for [115][328] NR_ATG_enh_demod

Agenda Item:	7.12.1

Source: 	CMCC

Document for:	Approval

<Topic 1> DL CA

Issue 1-1-1: CA Duplex mode

Agreement:

- TDD 30 kHz + TDD 30 kHz

- FDD 15 kHz + TDD 30 kHz

Issue 1-1-2: Test setups

Issue 1-1-2-1: TDD pattern

- Option 1: Use the legacy TDD pattern as the starting point

- Option 2: Define new TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirement

Issue 1-1-2-2: Rank

- Option 1: For CA requirement, use Rank 2 as the starting point

- Option 2: Only consider Rank 1 for DL CA demodulation requirements

Issue 1-1-2-3: MCS

- Option 1: Consider 64QAM MCS22 for ATG downlink CA requirements

- Other Options are not precluded

Issue 1-1-2-4: Bandwidth

Agreement:

- all bandwidth configuration for FDD 15kHz and TDD 30kHz

- Single carrier requirements for each of the considered bandwidth sizes in the bandwidth combination sets shall then be specified.

- Issue 1-1-3: CA configuration selection rules

- Option 1: Define the CA configuration selection rules as

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

- Issue 1-1-4: Test applicability rules

- Option 1: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

- Option 2: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

<Topic 2> DL MIMO

Issue 2-1-1: Antenna configuration

Agreement:

- XP-low model

Issue 2-1-2: TDD pattern

- Option 1: Use legacy TDD pattern 7DS2U as the starting point.

Issue 2-1-3: MCS

- Option 1: consider MCS 13 and MCS 22

- Option 2: Consider 64QAM

- Other Options are not precluded

Issue 2-2-2: RI

Agreement:

- Don’t introduce RI requirement for ATG MIMO

<Topic 3> UL MIMO

Issue 3-1-1: Whether to reuse existing requirements for ATG UL MIMO

Agreement:

- Only the applicable PUSCH test cases in Clause 8.2.1 with 2Tx 2 layer will be applied for ATG

Issue 3-1-2: Whether to introduce new PUSCH test cases for ATG UL MIMO

Agreement:

- Limited new test cases could be considered for ATG 2Tx PUSCH demodulation requirements

Issue 3-1-3: Whether to introduce new PUCCH and PRACH test cases for ATG UL MIMO

Agreement:

- No PUCCH and PRACH requirement for UL MIMO with 2Tx

Issue 3-1-4: Antenna configuration for incremental test

- Option 1: 2T2R

- Option 2: 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point

- Option 3: 2T2R with low correlation

Issue 3-1-5: Propagation condition for incremental test

Agreement:

- For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference

- Capture the static channel matrix in spec

Issue 3-1-6: Bandwidth & SCS for incremental test

Agreement:

- 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS

Issue 3-1-7: Mapping type for incremental test

Agreement:

- Mapping type A

Issue 3-1-8: DMRS configuration for incremental test

Agreement:

- Rel-15 DMRS type

Issue 3-1-9: Rank for incremental test

Agreement:

- Rank 2

Issue 3-1-10: MCS for incremental test

Agreement:

- 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and [256QAM Table 2 MCS 20] for initial simulation

- Only one MCS will be used

Issue 3-1-11: TPMI for incremental test

Agreement:

- TPMI index 0

Issue 3-1-12: Others

Agreement:

- Existing configuration as R15 BS PUSCH demodulation requirement with 2Tx