3GPP TSG-RAN4 Meeting #115 R4-2508779 # Malta, MT, 19th – 23rd May, 2025 CR-Form-v12.3. CHANGE REQUEST. 38.870; CR; draft; rev; -; Current version:; 19.0.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; Draft CR to 38.870 on NTN OTA performance metric and test method. Source to WG:; vivo, Samsung, Apple. Source to TSG:; R4. Work item code:; TRP_TRS_MIMO_OTA_Ph3-Core; Date:; 2025-05-05. Category:; B; Release:; Rel-19. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; This is further updated draft CR (based on endorsed R4-2504728 last meeting) to provide additional content to accommodate NTN test method and performance metric.. Summary of change:; Add sub-clause for NR-NTN and IoT-NTN radiated power performance metric, test procedure, positioning guideline, and MU assessment.. Consequences if not approved:; The spec is not appicable for NTN UE testing.. Clauses affected:; 4.2.1, 5.3.1, 5.4.1, 6.6, 7.4.1, 7.4.5, 7.4.6, 7.5.1, 7.5.5, 7.5.6, B.4.1, B.5.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

## <<< START OF CHANGES >>>

4.2.1	UE use scenarios for TRP TRS test

The following use scenarios are considered for TRP TRS test:

-	Talk mode using head & hand phantom for narrow phones between 56 mm and 72 mm and for wide phones with a width >72 mm and <92 mm.

-	Browsing mode using hand phantom for narrow and wide phones

-	Using forearm phantom for wrist-worn devices

-	Using XR Head phantom for headworn XR devices. FFS details

-	Free Space is used for devices not used in above-mentioned scenarios, other phantoms are not precluded for wearable devices

For TN testing of smartphones, both browsing mode and talk mode shall be covered. Free space (FS) testing for smartphone is low priority.

For NTN testing of smartphones, the use scenarios are categorized as following which are based on UE declaration:

- Usage scenario 1: both head+hand mode and hand only mode

- Usage scenario 2: hand only mode targeted for specific UE orientation usage

- Usage scenario 3: hand only mode targeted for arbitrary UE orientation usage

Note:	For each supported NR NTN band, declare only one supported scenario.

For wrist-worn (e)Redcap devices, forearm phantom is the first priority. FFS other (e)Redcap form factor devices.

For other device types, free space (FS) testing configuration is the first priority.

Note:	the UE positioning guideline described in Clause 6 and phantom definition described in Annex D are RATs agnostic.

<<< Skip Unchanged Sections >>>

5.3	Definition of the Partial sphere Radiated Power (PsRP)

5.3.1	Definition of the Partial sphere Radiated Power (PsRP) for AC

5.3.1.1	Applicability of PsRP metrics

The PsRP metrics are defined in Clauses 5.3.1.2 through 5.3.1.3, which can be applied to different devices, the applicability provided in Table 5.3.1.1-1 below.

Table 5.3.1.1-1: Applicability of PsRP metrics

| Clause | PsRP definition | Applicability |
| --- | --- | --- |
| 5.3.1.2 | UHRP= |  |
| 5.3.1.3 | PsRP= |  |

5.3.1.2	Upper Hemisphere Radiated Power

The UHRP metric is defined as total radiated power in three-dimensional upper hemisphere, which covers a partial surface with angular width of ±90°.

TBA

Figure 5.3.1.2-1: Visualization of UHRP

5.3.1.3	Partial sphere Radiated Power (PsRP)

The PsRP metric is defined as the total power radiated in the Theta and Phi range from partial surface with TBD angular width degrees.

TBA

Figure 5.3.1.3-1: Visualization of PsRP within TBD angular range

5.4	Definition of the Partial sphere Radiated Sensitivity (PsRS)

5.4.1	Definition of the Partial sphere Radiated Sensitivity (PsRS) for AC

5.4.1.1	Applicability of PsRS metrics

The PsRS metrics are defined in Clauses 5.4.1.2 through 5.4.1.3, which can be applied to different devices, with the applicability provided in Table 5.3.1.1-1 below.

Table 5.4.1.1-1: Applicability of PsRS metrics

| Clause | PsRS definition | Applicability |
| --- | --- | --- |
| 5.3.1.2 | UHRS= |  |
| 5.3.1.3 | PsRS= |  |

5.4.1.2	Upper Hemisphere Radiated Sensitivity

The UHRS metric is defined as total radiated Sensitivity in three-dimensional upper hemisphere, which covers a partial surface with angular width of ±90°.

TBA

Figure 5.3.1.2-1: Visualization of UHRS

5.4.1.3	Partial sphere Radiated Sensitivity (PsRS)

The PsRS metric is defined as the total Sensitivity radiated in the Theta and Phi range from partial surface with TBD angular width degrees.

TBA

Figure 5.3.1.3-1: Visualization of PsRS within TBD angular range

<<< Skip Unchanged Sections >>>

6.6	Hand phantom only for NTN scenario 2

The positioning specified in this clause is used for the hand only test case of NTN scenario 2. The characteristics of the Hand Phantom are specified in Annex D.

This mode is used to simulate user cases where the DUT is held in hand with specific direction, the positioning guideline in Clause 6.2 (traditional hand only browsing mode) can be reused as much as possible, the only difference is that the DUT shall be mounted in a suitable hand phantom and oriented such that the DUT’s main display is aligned with vertical. Elevation tilt is allowed based on vendor declaration. FFS the detailed elevation title angles.

TBA

Figure 6.6-1: Hand phantom only for NTN scenario 2 (vertical)

<<< Skip Unchanged Sections >>>

7.4.1	General

For TRP and TRS testing in SA or EN-DC mode, measurements should be only performed at NR carrier. The LTE link antenna in EN-DC mode is used to provide a stable LTE link to the DUT without precise path loss or polarization control.

The TRP of the DUT is measured by sampling the radiated transmit power of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with the sampling step for theta () and phi () axes defined in Table B.2.12-1, Annex B.2.12. For some test system that can not measure 180º EIRP, then the extrapolation approach can be adopted when generating the 3D antenna pattern. All of the measured power values will be integrated to TRP, as defined in Clause 5.1.1.2.

The Partial sphere Radiated Power (PsRP) of the DUT is measured by sampling the radiated transmit power of the DUT with partial sphere of three-dimensional scan at various locations surrounding the device, which is defined in Clause 5.3.1. The measurement is performed with the sampling step for theta () and phi () axes defined in Table B.2.12-1, Annex B.2.12. All of the measured power values will be integrated to PsRP, as defined in Clause 5.3.1. UHRP can be considered as a special case of PsRP.

For TRP and PsRP measurement, the evaluations shall be performed at maximum transmit power of DUT.

<<< Skip Unchanged Sections >>>

7.4.5	TRP and PsRP test procedure for NR NTN

7.4.5.1	UE configuration

NR-NTN TRP and PsRP radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

The NR NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and TRS test.

For NR NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 38.521-5 [31], section 6.2.1 (UE maximum output power) using the default settings specified in TS 38.521-5 [31] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-5 Clause 4.3.

7.4.5.2	Test procedure

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NR-NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-5 [31] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the TRP performance metric using the TRP integration approaches outlined in Clause 5.1.1.2.

For NR-NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-5 [31] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point [on the surface declared by the manufacturer] and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the PsRP performance metric using the approaches outlined in Clause 5.3.1

7.4.6	TRP and PsRP test procedure for IoT-NTN

7.4.6.1	UE configuration

IoT-NTN TRP and PsRP radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

The IoT-NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and TRS test. For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

For cat M1 NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [32], section 6.2.A.1 (UE maximum output power for category M1) using the default settings specified in TS 36.521-4 [32] and TS 36.508 [33] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-7 Clause 4.3.

For NB-IoT NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [32], section 6.2.B.1 (UE maximum output power for category NB1 and NB2) using the default settings specified in TS 36.521-4 [32] and TS 36.508 [33] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-9 Clause 4.3.

7.4.6.2	Test procedure for cat M1 NTN

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For cat M1 NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the TRP performance metric using the TRP integration approaches outlined in Clause 5.1.1.2.

For cat M1 NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point [on the surface declared by the manufacturer] and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the PsRP performance metric using the approaches outlined in Clause 5.3.1

7.4.6.3	Test procedure for cat NB-IoT NTN

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NB-IoT NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2B.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The performance metric is TRP, and TRP value is calculated using the TRP integration approaches outlined in Clause 5.1.1.2.

For NB-IoT NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2B.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point [on the surface declared by the manufacturer] and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the PsRP performance metric using the approaches outlined in Clause 5.3.1

<<< Skip Unchanged Sections >>>

7.5.1	General

For SA and EN-DC, the TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with the sampling step for theta () and phi () axes defined in Table B.2.12-1, Annex B.2.12, for TRS measurement.

EIS, or receiver sensitivity measurements, is defined as the minimum downlink signal power received at the UE antenna input required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel (RMC) (the maximum throughput is per Appendix A of TS 38.521-1 [5]).

The Partial sphere Radiated Sensitivity (PsRS) of the DUT is measured by sampling the radiated sensitivity power of the DUT with partial sphere of three-dimensional scan at various locations surrounding the device, which is defined in Clause 5.3.1. The measurement is performed with the sampling step for theta () and phi () axes defined in Table B.2.12-1, Annex B.2.12. All of the measured sensitivity power values will be integrated to PsRS, as defined in Clause 5.3.1.

For TRS and PsRS measurement, the evaluations shall be performed at maximum transmit power.

For TRS measurement, no specific setting is needed for Rx antennas. By default, the maximum number of Rx antennas supported at each band should be enabled during the TRS and PsRS test.

<<< Skip Unchanged Sections >>>

7.5.5	TRS and PsRS test procedure for NR NTN

7.5.5.1	UE configuration

NR-NTN TRS and PsRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

The NR NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the OTA test.

For NR NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 38.521-5 [31], section 7.3 (Reference sensitivity) using the default settings specified in TS 38.521-5 [31] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-6 Clause 4.3.

7.5.5.2	Test procedure

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NR-NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 7 in section 7.3.2.4.2, TS 38.521-5 [31], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR NTN sensitivity level.

4)	Calculate the TRS performance metric using the TRS integration approaches outlined in Clause 5.2.1.

For NR-NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 7 in section 7.3.2.4.2, TS 38.521-5 [31], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR NTN sensitivity level.

4)	Calculate the PsRS performance metric using the PsRS integration approaches outlined in Clause 5.4.1.

7.5.6	TRS and PsRS test procedure for IoT-NTN

7.5.6.1	UE configuration

IoT-NTN TRS and PsRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

The IoT-NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and TRS test. For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

For cat M1 NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [32], section 7.3.A (Reference sensitivity power level for UE category M1) using the default settings specified in TS 36.521-4 [32] and TS 36.508 [33] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-8 Clause 4.3.

For NB-IoT NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [32], section 7.3.B (Reference sensitivity power level for UE category NB1 and NB2) using the default settings specified in TS 36.521-4 [32] and TS 36.508 [33] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-10 Clause 4.3.

7.5.6.2	Test procedure for cat M1 NTN

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For cat M1 NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 4 in section 7.3A.4.2, TS 36.521-4 [32], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the M1 NTN sensitivity level.

4)	Calculate the TRS performance metric using the TRS integration approaches outlined in Clause 5.2.1.

For cat M1 NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 4 in section 7.3A.4.2, TS 36.521-4 [32], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the M1 NTN sensitivity level.

4)	Calculate the PsRS performance metric using the PsRS integration approaches outlined in Clause 5.4.1.

7.5.6.3	Test procedure for cat NB-IoT NTN

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NB-IoT NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 3 in section 7.3B.4.2, TS 36.521-4 [32], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NB-IoT NTN sensitivity level.

4)	Calculate the TRS performance metric using the TRS integration approaches outlined in Clause 5.2.1.

For NB-IoT NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 3 in section 7.3B.4.2, TS 36.521-4 [32], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NB-IoT NTN sensitivity level.

4)	Calculate the PsRS performance metric using the PsRS integration approaches outlined in Clause 5.4.1.

<<< Skip Unchanged Sections >>>

B.4.1	MU Assessment for TRP in Anechoic Chamber

<Editor’s note: The MU assessment for XR device will be further added. >

The uncertainty contributions related to TRP are listed in Table B.4.1-1.

The preliminary example uncertainty budgets for TRP are presented in this clause, and the corresponding applicability are summarized as following:

-  Hand only TRP MU is presented in Table B.4.1-2, which can be applied to at least handheld UE and NTN UE under scenario 1 and 3. FFS other UE types.

-  Head and hand TRP MU is presented in Table B.4.1-3, which can be applied to at least handheld UE and NTN UE under scenario 3. FFS other UE types.

-  Hand only TRP MU for Wrist-Worn device is presented in Table B.4.1-4.

-  Head and hand TRP MU for Wrist-Worn device is presented in Table B.4.1-5.

<<< Skip Unchanged Sections >>>

B.5.1	MU Assessment for TRS in Anechoic Chamber

<Editor’s note: The MU assessment for XR device will be further added. >

The uncertainty contributions related to TRS are listed in Table B.5.1-1.

The preliminary example uncertainty budgets for TRS are presented in this clause, and the corresponding applicability are summarized as following:

-  Hand only TRS MU is presented in Table B.5.1-2, which can be applied to at least handheld UE and NTN UE under scenario 1 and 3. FFS other UE types.

-  Head and hand TRS MU is presented in Table B.5.1-3, which can be applied to at least handheld UE and NTN UE under scenario 3. FFS other UE types.

-  Hand only TRS MU for Wrist-Worn device is presented in Table B.5.1-4.

-  Head and hand TRS MU for Wrist-Worn device is presented in Table B.5.1-5.

## <<< END OF CHANGES >>>