3GPP TSG-RAN WG4 Meeting # xxx											R4-2505575

Malta, MT, May 19th – May 23rd, 2025

Agenda item:			7.30.1

Source:	Moderator (Nokia)

Title:	Topic summary for [115][226] NR_XR_Ph3

Document for:	Information

# Introduction

This summary includes the proposals and issues for discussion related to Rel-19 enhancements for extended reality for NR Phase 3, under thread [115][226] NR_XR_Ph3.

This summary is based on the contributions submitted to the agenda items:

- 7.30	XR for NR Phase 3	[NR_XR_Ph3]

  - 7.30.1	Moderator summary and conclusions	[NR_XR_Ph3-Core]

  - 7.30.2	General aspects	[NR_XR_Ph3-Core]

  - 7.30.3	RRM core requirements	[NR_XR_Ph3-Core]

The issues for discussion are organized in topics which are as follows:

- Topic 1 Scenarios for measurement skipping

- Topic 2 Measurements with gaps

- Topic 3 XR skipping requirements impact

- Topic 4 UE assistance information

- Topic 5 Additional signalling aspects

- Topic 6 CR handling

- Topic 7 UE capabilities

The following issues are proposed to be prioritized for discussion

- Issue 6-1: Naming convention

- Issue 2-1: Types of measurement gaps

- Issue 2-2: Proposals for Type 2 gaps

- Issue 2-3: Proposals for NCSG

- Issue 2-4: Proposals for Pre-MG

- Issue 2-5: Proposals for Rel-18 concurrent gaps

- Issue 4-1: Additional aspects for UAI

- Issue 3-1: Principles for measurement delay calculation

- Issue 3-2: Formulas for measurement delay extension without DRX

- Issue 3-4: Conditions for measurement skipping

- Issue 7-1: UE capabilities for XR - general proposals

- Issue 7-2: UE capabilities for XR - Tables

- Issue 5-1: Semi-static schemes

- Issue 3-6: Impact on different frequency layers

- Issue 3-7: UL XR impact

List of latest agreements are collected by rapporteurs for information in this document

- R4-2507388, Rapporteur Inputs, Nokia, Qualcomm

# Agreed workplan (from R4-2414027 for information)

| RAN4 meeting | Tasks |
| --- | --- |
| RAN4 #112 - August 2024 | - Approval of workplan - Initial discussion on scenarios for measurement skipping based on RAN1 conclusions. |
| RAN4 #112bis - October2024 | - Conclusions on scenarios for measurement skipping based on RAN1 conclusions. - Discussion on the need for UE assistance information regarding measurements occasions needed. |
| RAN4 #113 - November 2024 | - Discussion on RRM requirements with measurement skipping. - Discussion on the necessary UE assistance information. |
| RAN4 #114 - February 2025 | - Conclusion on RRM requirements with measurement skipping. - Discussion on the need for UE assistance information regarding measurement skipping. - Agreement of CR work split. |
| RAN4 #114bis - April 2025 | - Conclusion on the necessary UE assistance information. - First Draft CRs for RRM core requirements. - Draft CRs for 38.133 expected. - First big Draft CR. |
| RAN4 #115 - May 2025 | - Discussion on remaining open issues for RRM core |
| RAN4 #116 - August 2025 | - Discussion on remaining open issues for RRM core - Revision of CRs for RRM core requirements. - Planning of RRM performance. - Initial discussion on RRM test cases. |

# Topic #1: Title

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

### Discussion papers

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2507388 | Nokia, Qualcomm | Rapporteur Inputs <moderator> document for information only |
| R4-2506457 | vivo | On UE capability for XR WI . Proposal 1: FG 64-1 is the prerequisite feature for UE capability of 3ms time offset and UAI of ratio indication. Proposal 2: For both UE capability of 3ms time offset and UAI of ratio indication, the reporting granularity is per-UE. Proposal 3: UE feature for Rel-19 XR WI is agreed as in Table 1. <Moderator> Table 1 is shown in Topic 7 |
| R4-2505389 | China Telecom | RRM requirements on XR enhancements Proposal 1: Use option 2 as baseline of the formula for measurement delay extension, and option 1 can be further discussed and adopted if consensus can be reached. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) ´ Max(MGRP , SMTC period)) ´ CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. Proposal 2: For the formulas for measurement delay extension with DRX, option 2 is also preferred. - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter Proposal 3: No conditions are to be defined for measurement skipping. |
| R4-2506456 | vivo | Further discussion on RRM requirements for XR Proposal 1: Pre-configure measurement gap may be considered. Proposal 2: Option 1 is used to define requirements for gap skipping. Proposal 3: Consider only one gap is skipped during one DRX cycle if there is indication for skipping. Proposal 4: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. Proposal 5: The UAI about ratio of gaps for skipping is per gap if per-FR gap is configured and is regardless of DRX. Proposal 6: Send an LS to RAN2 if proposal 5 is agreeable. Proposal 7: RAN4 agrees to introduce RRC signaling indication for gap skipping. Proposal 8: RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements. Observation 1: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied. Observation 2: It is up to UE implementation which gaps are not used for measurements under current measurement configuration. Observation 3: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance. Observation 4: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps. |
| R4-2506593 | CMCC | Discussion on XR (eXtended Reality) for NR Phase 3 Proposal 1: for types of measurement gaps, it is proposed that Type 1 MG, Type 2 MG, NCSG, Pre-MG, concurrent gaps are considered. - For NSCG, the measurement gap skipping is applied to VIL1 and VIL2 of a NCSG pattern - For Pre-MG, the measurement gap skipping is applied to the first activated Pre-MG which satisfy the time offset - For concurrent gap, the measurement gap skipping is applied to the kept gap after applying dropping rules based on priority. Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, the measurement delay extension due to measurement skipping is proposed as following: - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter Proposal 3: for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes. |
| R4-2506712 | Huawei, HiSilicon | Discussion on RRM requirements for Rel-19 XR Observation 1: Compared with DCI-based solution, semi-static approach has following benefits: - It is less complicated to UE implementation that UE can have a stable long-term measurement pattern. - Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impact on measurement delay. - A single RRC signal has less overhead than DCI which is per-occasion indication. Proposal 1: RAN4 to conclude whether to introduce semi-static indication by RAN4#115. Proposal 2: RAN4 to consider following approach to move forward: - The DCI-based indication shall be supported by default if UE support MG skipping. - RAN4 to introduce semi-static indication to skip measurement gap as additional UE capability. Observation 2: Artificially precluded certain type of gaps will cause unnecessary implementation complexity to both gNB and UE. Proposal 3: MG skipping applies to all type of MG except for MUSIM and Position which is precluded in last meeting. Proposal 4: For concurrent gap, existing dropping rules applies and the MG skipping only applies to the MG which is not dropped, and the MG skipping won’t change status of MG occasion(s) w.r.t dropped or not. Proposal 5: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended. - Option 2: - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - the delay shall be extended accordingly for all frequencies which are candidates to be measured using the gap. Proposal 6: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. |
| R4-2507121 | NTT DOCOMO, INC. | Discussions on RRM requirements for XR Observation 1: For measurement delay extension without DRX, the controversial point is whether the skipping factor should affect CSSFinter or not and to keep same number of measurement samples with skipping consideration, CSSFinter should be out of the effect of Nskip. Proposal 1: Without DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)) Proposal 2: With DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms ´ CSSFinter, Ceil(8 ´1.5) ´ Max(MGRP, SMTC period, DRX cycle) ´ CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 ´ DRX cycle ´ CSSFinter + Nskip ´ DRX cycle |
| R4-2507250 | Samsung | General discussion on XR RRM requirement Proposal 1: Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter Proposal 2: Do not consider semi-static solutions |
| R4-2507292 | ZTE Corporation, Sanechips | Discussion on RRM aspects of R19 XR Proposal 1: Gap skipping is applicable to Type-2 MG with the assumption of the skipping indication is valid to the remaining gap occasions after priority rule applied. Proposal 2: RAN4 to define the measurement delay extension for inter-frequency measurement with gap as: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) x Max(MGRP , SMTC period)) x CSSFinter - where Nskip is the number of skipped MG occasions overlaping with SMTC in the measurement period. - Use this formula as example and follow similar approach for the other cases. Proposal 3: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. Proposal 4: Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered. Proposal 5: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. Proposal 6: Support both dynamic and semi-static gap skipping indication as a packet, and NW decide which scheme is to be enabled. |
| R4-2507582 | MediaTek inc. | Discussion on RRM requirements for XR Proposal 1: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic. Proposal 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of skipped MG occasions in the measurement period. Proposal 3: RAN4 to define measurement delay extension with DRX cycle <= 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period, DRX cycle)) × CSSFinter’, where Nskip is the number of skipped MG occasions in the measurement period. Proposal 4: RAN4 to define measurement delay extension with DRX cycle > 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Ceil ((8 + Nskip) Kgap) DRX cycle CSSFinter’, where Nskip is the number of skipped MG occasions in the measurement period. Proposal 5: RAN4 shall discuss and define the max number of occasions cancelled during a measurement period. |
| R4-2507739 | Ericsson | On RRM requirements for XR - Proposal 1: The requirements with MG cancellation are not applicable for UEs configured with concurrent measurement gaps. - Proposal 2: For transmitting UL XR data during the cancelled MG occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled MG. - Proposal 3: For transmitting UL XR data during the cancelled MG occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled MG. - Proposal 4: When receiving a DCI scheduling UL XR data during a cancelled MG occasion, it is assumed that the UE will have the correct UL timing prior to the transmission of the UL XR data during the cancelled MG occasion. - Proposal 5: RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes. - Proposal 6: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: Condition; FR1: TPSS/SSS_sync_inter. No DRX; Max(600 ms, (8+XPSS/SSS,gaps) ´ Max(MGRP, SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(600 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter. Condition NOTE1; FR2-1: TPSS/SSS_sync_inter. No DRX; Max(600 ms, (Mpss/sss_sync_inter + XPSS/SSS,gaps) ´ Max(MGRP , SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(600 ms, Ceil(1.5 ´ (Mpss/sss_sync_inter + XPSS/SSS,gaps)) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (Mpss/sss_sync_inter+ XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter. - Proposal 7: The maximum numbers of extensions of inter-frequency cell detection period are for FR1: XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms. FR2: XPSS/SSS,gaps,max=1.5´Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, XPSS/SSS,gaps,max=0.5 ´ Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms. - Proposal 8: UE behaviour when the maximum number of extensions for inter-frequency cell detection is exceeded: Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement. - Proposal 9: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively: Condition NOTE1; FR1: TSSB_measurement_period_inter NOTE2. No DRX; Max(120 ms, (3+Xind,gaps)´ Max(MGRP , SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(120 ms, Ceil((3+Xind,gaps) ´ 1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (3+Xind,gaps)´ DRX cycle ´ CSSFinter. Condition; FR2-1: TSSB_measurement_period_inter. No DRX; Max(200 ms, (MSSB_index_inter + Xind,gaps) ´ Max(MGRP, SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(200 ms, Ceil(1.5 ´ (MSSB_index_inter + Xind,gaps)) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (MSSB_index_inter + Xind,gaps) ´ DRX cycle ´ CSSFinter. - Proposal 10: The maximum numbers of extensions of inter-frequency SSB time index detection period are for FR1: Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms. FR2: Xind,gaps,max=1.5 ´ MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and Xind,gaps,max=0.5 ´ MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms. - Proposal 11: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded: Upon exceeding Xind,gaps,max the UE shall restart the time index detection. - Proposal 12: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively: Condition NOTE1; FR1: TSSB_measurement_period_inter NOTE2. No DRX; Max(200 ms, (8 + Xmeas) ´ Max(MGRP , SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(200 ms, Ceil((8 + Xmeas) ´ 1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (8 + Xmeas) ´ DRX cycle ´ CSSFinter. Condition; FR2-1: TSSB_measurement_period_inter. No DRX; Max(400 ms, (Mmeas_period_inter + Xmeas) ´ Max(MGRP , SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(400 ms, Ceil(1.5 ´ (Mmeas_period_inter + Xmeas) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (Mmeas_period_inter + Xmeas) ´ DRX cycle ´ CSSFinter. - Proposal 13: The maximum numbers of extensions of inter-frequency cell detection period are for FR1: XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms. FR2: Xmeas=1.5 ´ Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, Xmeas=0.5 ´ Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms. - Proposal 14: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded: Upon exceeding Xmeas,max the UE shall restart the measurement. - Proposal 15: RAN4 to discuss MG configurations relevant for MG cancellation, e.g., - minimum MGL, e.g., MGL≥3 ms, - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays. - Proposal 16: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.: Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. - Proposal 17: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3. |
| R4-2507747 | Nokia | Discussion on XR ph3 RRM requirements Concurrent measurement gaps are subject to measurement gap cancellation and following conditions apply: - If concurrent measurement gaps are colliding as specified in clause 9.1.8.3, the dropping rule is applied before measurement gap cancellation. - A concurrent measurement gap with the gapAssociationPRS parameter set, is not subject to measurement gap cancellation. - The UE may provide the UAI related information on how many MG occasions may be skipped for each configured MG. Measurement gap cancellation applies to NCSG. - The skipping indication on a DCI applies for an entire NCSG occasion, where the NCSG occasion is composed of a combination of VIL1, ML and VIL2. - The DCI with skipping indication for a NCSG gap occasion must be received by the UE at least Toffset before the start of VIL1. - The DCI with skipping indication for a NCSG gap occasion received between VIL1 and VIL2 applies for the next NCSG gap occasion. Pre-configured measurement gaps are not subject to measurement gap cancellation. Rel-18 concurrent measurement gaps are not subject to measurement gap cancellation. - The measurement delay of option 2 is 44,4 % larger than the delay for Option 1 when CSSF=5 and Nskip=5. RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms ´ CSSFinter, 8 ´ Max(MGRP , SMTC period) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period)), where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. Alternative proposal - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - Tskip = Nskip ´ Max(MGRP , SMTC period) - Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. Alternative 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms ´ CSSFinter, ceil(8 ´ 1,5) ´ Max(MGRP , SMTC period, DRX cycle) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period, DRX cycle)) - DRX cycle > 320 ms - 8 ´ DRX cycle ´ CSSFinter + Nskip ´ DRX cycle - Use this formula as example and follow similar approach for the other cases. Alternative proposal - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority. It is possible to increase skipping ratio to up to 50% for 2 layers and 80% for 5 layers without impacting the measurement delay of 1 frequency layer. High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is - Nskip ≤8 ´ (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤ 5 ´ (CSSF - Nhigh) for intra-frequency measurements - where Nhigh is the number of high priority layers. Our simulation results show the benefit of using about 60% skipping ratio. In at least some scenarios skipping ratio can be increased to about 75% without significant mobility degradation. Measurement gaps can be configured by the network to receive reports on potential carrier aggregation candidates. Maximum number of skipping depends on the deployment, e.g. number of layers, measurement gap configuration, deployment inter-site distance. No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration. RAN1 already confirmed working assumption regarding Alt 1-1. The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL. RAN4 already agreed on 5 ms time offset for measurement skipping as a baseline. Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1) Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic. Semi-static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge). RAN1 already made down selection between DCI and RRC schemes Follow RAN1 agreement and consider DCI scheme only. RAN1 already defined UE feature for XR measurement gap skipping. RAN2 started discussion on UE feature for UAI reporting based on RAN4 LS. RAN4 to discuss whether RAN4 should define the UE capability for UAI. Define UE feature for support of UAI as optional with per UE capability. RAN4 to discuss the need for additional UE feature for XR on top of RAN1 and RAN2 discussed features. |
| R4-2507877 | Qualcomm Incorporated | RRM requirements for XR enhancements Observation 1: Network may not be aware whether a pre-configured MG is activated or deactivated if UE autonomously determines the status. Proposal 1: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17 Observation 2: NCSG do not cause big interruption on data and may not impact the XR QoS as much. Proposal 2: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17 Observation 3: Concurrent MGs may cause more frequent data interruptions at the UE. Proposal 3: MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - Note that the other MG is already dropped, so a cancelling DCI would effectively mean that both the colliding MGs are dropped. Proposal 4: The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. Proposal 5: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples. Proposal 6: Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc. Proposal 7: If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers. Proposal 8: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC or MAC-CE based signaling as well. |
| R4-2505394 | InterDigital Finland Oy | NR_XR_Ph3 gap skipping impact Observation 1: Gap pattern IDs 0, 4, 6, 10, 12, 16 result in considerable impact on XR traffic. Proposal 1: RAN4 to perform the selection of the applicable Type-2 gaps to be included in Rel-19 considering the impact on XR traffic when Gap pattern IDs 0, 4, 6, 10, 12, 16 are in the possible configuration. Observation 2: Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements. Proposal 2: Support Option 1: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. Observation 3: Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements. Proposal 3: Support Option 1: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms ´ CSSFinter, Ceil(8 ´1.5) ´ Max(MGRP, SMTC period, DRX cycle) ´ CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 ´ DRX cycle ´ CSSFinter + Nskip ´ DRX cycle Proposal 4: Support Option 1 - Option 1: Do not consider semi-static solutions. Proposal 5: Support Proposal 3 from the above list: - Proposal 3: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. |
| R4-2505713 | OPPO | RRM requirements for XR enhancements Proposal 1: For measurement delay extension, RAN4 to extend the L3 delays by adding the number of skipped MG occasions on top of the legacy number of measurement samples in the measurement period. Proposal 2: RAN4 to define maximum time separation between two non-skipped gaps for measurement. |
| R4-2505878 | Apple | Proposal 1: type-2 gap, Pre-MG, NCSG and concurrent gaps are considered this work item. - Type-2 gap: it can be handled in the same way as type-1 gap. - Pre-MG: it can be handled in the same way as type-1 gap when Pre-MG is ON. Otherwise when Pre-MG is OFF, no need to cancel it. - Concurrent gaps: handle gap collision first, then apply DCI for gap cancellation. In other words, gap cancellation won’t bring the gap, which is with lower priority and dropped due to gap collision, back. - NCSG: it can be handled in the same way as type-1 gap assuming a gap occasion is VIL1+ML+VIL2 Proposal 2: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. Proposal 3: max number of occasions cancelled during a measurement period shall be defined. Proposal 4: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) ´ Max(MGRP , SMTC period)) ´ CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. Proposal 5: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter Proposal 6: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported. Proposal 7: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. |
| R4-2507390 | Nokia | Discussion on Rel-19 RAN4 UE feature list for NR for XR_Ph3 <moderator> document submitted to AI 8. Proposal in table is captured in Topic 7 for completeness of the discussion. |

### Draft CRs

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2506455 | vivo | Draft CR on NR inter-frequency L1 measurement for XR |
| R4-2506713 | Huawei, HiSilicon | DraftCR on Inter-RAT measurements E-UTRAN FDD for Rel-19 XR |
| R4-2507304 | ZTE Corporation, Sanechips | Draft CR on measurement gap skipping in intra-frequency measurement with gaps for R19 XR |
| R4-2507389 | Nokia | Draft CR38.133 Introduction of measurement gap cancelation |
| R4-2507738 | Ericsson | draftCR38133 Inter-frequency measurement requirements with XR |
| R4-2505879 | Apple | Inter-RAT measurements E-UTRAN TDD measurements |

## Open issues summary

### Topic 1 Scenarios for measurement skipping

#### Issue 1-1: UE power class

- Proposals

  - Option 1 (Ericsson): The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

- Recommended WF

  - Discussion needed.

### Topic 2 Measurements with gaps

#### Issue 2-1: Types of measurement gaps

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Company proposals on the priorities for RAN4 to work are summarized using based on the following table

| Gap types | May be cancelled | Shall not be cancelled |
| --- | --- | --- |
| Type 2 | CMCC, Huawei, ZTE, Nokia, Qualcomm, Apple | Ericsson |
| NCSG | CMCC, Huawei, Nokia, Apple | Qualcomm |
| Rel 18 concurrent gap | CMCC, Huawei, Apple | Nokia, Ericsson |
| PreConfigured | Vivo, CMCC, Huawei, Apple | Nokia, Qualcomm |

- Recommended WF

  - Discussion on each individual gap type based on Issues 2-2, 2-3, 2-4, and 2-5.

#### Issue 2-2: Proposals for Type 2 gaps

- Proposals

  - Proposals not supporting skipping of Type 2 gaps

    - Proposal 1 (Ericsson): The requirements with MG cancellation are not applicable for UEs configured with concurrent measurement gaps.

  - Proposals allowing skipping of Type 2 gaps

    - Proposal 2 (CMCC): For concurrent gap, the measurement gap skipping is applied to the kept gap after applying dropping rules based on priority.

    - Proposal 3 (Huawei): For concurrent gap, existing dropping rules applies and the MG skipping only applies to the MG which is not dropped, and the MG skipping won’t change status of MG occasion(s) w.r.t dropped or not.

    - Proposal 4 (ZTE): Gap skipping is applicable to Type-2 MG with the assumption of the skipping indication is valid to the remaining gap occasions after priority rule applied.

    - Proposal 5 (Nokia):

      - If concurrent measurement gaps are colliding as specified in clause 9.1.8.3, the dropping rule is applied before measurement gap cancellation.

      - A concurrent measurement gap with the gapAssociationPRS parameter set, is not subject to measurement gap cancellation.

      - The UE may provide the UAI related information on how many MG occasions may be skipped for each configured MG.

    - Proposal 6 (Qualcomm): MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules.

      - Note that the other MG is already dropped, so a cancelling DCI would effectively mean that both the colliding MGs are dropped.

      - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules.

    - Proposal 7 (Apple): it can be handled in the same way as type-1 gap.

    - Proposal 8 (Apple): handle gap collision first, then apply DCI for gap cancellation. In other words, gap cancellation won’t bring the gap, which is with lower priority and dropped due to gap collision, back.

- Merged proposals by moderator

  - Option 1: Type 2 gaps cannot be skipped

  - Option 2: Type 2 gaps can be skipped.

    - Proposal 1: MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules.

    - Proposal 2: The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules.

    - Proposal 3: A concurrent measurement gap with the gapAssociationPRS parameter set, is not subject to measurement gap cancellation.

    - Proposal 4: The UE may provide the UAI related information on how many MG occasions may be skipped for each configured MG.

- Recommended WF

  - Discuss the proposals. For the companies supporting Type 2 gaps, please discuss the aspects of the proposals under Option 2.

#### Issue 2-3: Proposals for NCSG

- Proposals

  - Option 1 (Qualcomm): MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17

  - Option 2 (CMCC): For NSCG, the measurement gap skipping is applied to VIL1 and VIL2 of a NCSG pattern

  - Option 3 (Nokia):

    - The skipping indication on a DCI applies for an entire NCSG occasion, where the NCSG occasion is composed of a combination of VIL1, ML and VIL2.

    - The DCI with skipping indication for a NCSG gap occasion must be received by the UE at least Toffset before the start of VIL1.

    - The DCI with skipping indication for a NCSG gap occasion received between VIL1 and VIL2 applies for the next NCSG gap occasion.

  - Option 4 (Apple): it can be handled in the same way as type-1 gap assuming a gap occasion is VIL1+ML+VIL2

- Recommended WF

  - Discuss the proposals.

#### Issue 2-4: Proposals for Pre-MG

- Proposals

  - Option 1 (Qualcomm, Nokia): MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17

  - Option 2 (CMCC): For Pre-MG, the measurement gap skipping is applied to the first activated Pre-MG which satisfy the time offset.

  - Option 3 (vivo): Pre-configured measurement gap may be considered.

  - Option 4 (Apple): Pre-MG: it can be handled in the same way as type-1 gap when Pre-MG is ON. Otherwise when Pre-MG is OFF, no need to cancel it.

- Recommended WF

  - Discuss the proposals.

#### Issue 2-5: Proposals for Rel-18 concurrent gaps

- Proposals

  - Option 1 (Nokia, Ericsson): Rel-18 concurrent measurement gaps are not subject to measurement gap cancellation.

  - Option 2 (CMCC): For concurrent gap, the measurement gap skipping is applied to the kept gap after applying dropping rules based on priority.

  - Option 3 (Apple): handle gap collision first, then apply DCI for gap cancellation. In other words, gap cancellation won’t bring the gap, which is with lower priority and dropped due to gap collision, back.

- Recommended WF

  - Clarify if CMCC proposal is also applicable to Rel-18 concurrent gaps.

  - To be considered also are the different cases of Rel-18 concurrent gaps, i.e.

    - Concurrent Pre-MGs

    - Concurrent NCSGs

    - Type 2 + concurrent Pre-MG

    - Type 2 + concurrent NCSG

### Topic 3 XR skipping requirements impact

#### Issue 3-1: Principles for measurement delay calculation

- Proposals

  - Proposal 1 (MediaTek): RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

  - Proposal 2 (Qualcomm): For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

  - Proposal 3 (OPPO): For measurement delay extension, RAN4 to extend the L3 delays by adding the number of skipped MG occasions on top of the legacy number of measurement samples in the measurement period.

  - Proposal 4 (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

- Recommended WF

  - Discuss the proposals.

#### Issue 3-2: Formulas for measurement delay extension without DRX

- Company proposals

  - Option 1 (vivo, Nokia, NTT DOCOMO, Interdigital): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

  - Option 2 (China Telecom, Ericsson, Apple): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period)) ´ CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

  - Option 3 (CMCC, Huawei, ZTE, MediaTek): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

  - Option 4 (Samsung): Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap

    - FFS: The maximum number of skipped MG occasions in the measurement period to limit Nskip

    - FFS: CSSFinter

  - Option 5 (Nokia): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

    - Where

      - Tskip = Nskip ´ Max(MGRP , SMTC period)

      - Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

- Merged proposals (not considering Type 2 gaps, i.e. without Kgap):

  - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

    - Option 3:

      - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

      - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

      - where Tskip = Nskip ´ Max(MGRP , SMTC period)

  - where Nskip is the number of  skipped MG occasions in the measurement period.

  - FFS: CSSFinter

- Recommended WF

  - Please discuss first the case without Type 2 (i.e. Kgap not considered). If type 2 is agreed in Issue 2-1/2-2, discuss how to include Kgap in the formula.

#### Issue 3-3: Formulas for measurement delay extension with DRX

- Proposals

  - Option 1 (NTT DOCOMO, Nokia, Interdigital, vivo ?): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - DRX cycle ≤ 320 ms

      - Max(200 ms ´ CSSFinter, Ceil(8 ´1.5) ´ Max(MGRP, SMTC period, DRX cycle) ´ CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

    - DRX cycle > 320 ms

      - 8 ´ DRX cycle ´ CSSFinter +  Nskip ´ DRX cycle

  - Option 2 (China Telecom, Ericsson, Apple): Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

    - DRX cycle ≤ 320 ms

      - Max(200 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter

    - DRX cycle > 320 ms

      - Ceil(8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter

  - Option 3 (vivo) : Consider only one gap is skipped during one DRX cycle if there is indication for skipping.

  - Option 4 (MediaTek):

    - RAN4 to define measurement delay extension with DRX cycle <= 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period, DRX cycle)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

    - RAN4 to define measurement delay extension with DRX cycle > 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Ceil ((8 + Nskip) ´ Kgap) ´ DRX cycle ´ CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

  - Option 5 (Nokia): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

    - Where

      - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle)

      - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle

- Recommended WF

  - Discussion first on Issue 3-2.

#### Issue 3-4: Conditions for measurement skipping

- Proposals from companies

  - Proposals for no conditions

    - Proposal 1 (China Telecom): No conditions are to be defined for measurement skipping.

    - Proposal 2 (Nokia): No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration

    - Proposal 3 (ZTE): No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

    - Proposal 4 (Qualcomm): Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc.

  - Proposals on max number of occasions

    - Proposal 5 (MediaTek): RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

    - Proposal 6 (vivo): For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified

    - Proposal 7 (Apple): max number of occasions cancelled during a measurement period shall be defined.

    - Proposal 8 (Ericsson): The maximum number of extensions are

|  | PSSS/SSS detection | SSB index detection | Measurement period |
| --- | --- | --- | --- |
| No DRX or max(DRX cycle, SMTC period, MGRP)≤40 ms | FR1: 12 FR2: 1.5×Mpss/sss_sync | FR1: 5 FR2: 1.5×MSSB_index | FR1: 12 FR2: 1.5×Mmeas_period |
| DRX cycle ≤ 320 ms | FR1: 8 FR2: Mpss/sss_sync | FR1: 3 FR2: MSSB_index | FR1: 8 FR2: Mmeas_period |
| DRX cycle > 320 ms | FR1: 5 FR2: 0.5×Mpss/sss_sync | FR1: 2 FR2: 0.5× MSSB_index | FR1: 5 FR2: 0.5× Mmeas_period |

    - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

    - Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

    - Upon exceeding Xmeas,max the UE shall restart the measurement.

  - Proposals on maximum separation between two occasions shall be

    - Proposal 9 (Ericsson): Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

      - Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

    - Proposal 10 (OPPO): RAN4 to define maximum time separation between two non-skipped gaps for measurement.

- Proposals merged by the Moderator

  - Option 1: No conditions are defined

  - Option 2: Define the maximum number of gaps skipped during the measurement period.

  - Option 3: Define a maximum separation between two non-skipped gaps for measurement

- Recommended WF

  - Discussion on the conditions is needed. If the group agrees that conditions are necessary, we can further discuss the limits taking Proposal 8 as an example.

#### Issue 3-5: Measurement gap configurations relevant to skipping

- Options

  - Option 1 (Ericsson): RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

    - minimum MGL, e.g., MGL≥3 ms,

    - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

  - Option 2 (Interdigital): RAN4 to perform the selection of the applicable Type-2 gaps to be included in Rel-19 considering the impact on XR traffic when Gap pattern IDs 0, 4, 6, 10, 12, 16 are in the possible configuration.

- Recommended WF

  - Please discuss if there is need for limiting which MG configurations can be skipped.

#### Issue 3-6: Impact on different frequency layers

- Proposals

  - Proposal 1 (Nokia): RAN4 to define requirements such that the network can configure which frequency layers are measured with higher priority

  - Proposal 2 (Nokia): High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

    - Nskip ≤ 8 × (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤  5 × (CSSF - Nhigh) for intra-frequency measurements

    - where Nhigh is the number of high priority layers.

- Recommended WF

  - Discuss the proposals.

#### Issue 3-7: UL XR impact

- Proposals

  - Proposal 1 (Ericsson): For transmitting UL XR data during the cancelled MG occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled MG.

  - Proposal 2 (Ericsson): For transmitting UL XR data during the cancelled MG occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled MG.

  - Proposal 3 (Ericsson): When receiving a DCI scheduling UL XR data during a cancelled MG occasion, it is assumed that the UE will have the correct UL timing prior to the transmission of the UL XR data during the cancelled MG occasion.

- Recommended WF:

  - Discuss the proposals.

### Topic 4 UE assistance information

Sub-topic description

RAN1 has sent an LS (R1-2405736) to RAN4, requesting RAN4 to decide whether to introduce any UE assistance information related to measurement occasions. This sub-topic includes issues relating to the need and feasibility of such UE assistance information.

Open issues and candidate options before meeting:

#### Issue 4-1: Additional aspects for UAI

- Agreement from RAN4 #114

Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

- Agreement from RAN4 #114bis

## Issue 5-1: General on UAI <Agreement>: Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting. - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

- Proposals

  - Proposal 1 (vivo): The UAI about ratio of gaps for skipping is per gap if per-FR gap is configured and is regardless of DRX.

  - Proposal 2 (Nokia): The UE may provide the UAI related information on how many MG occasions may be skipped for each configured MG.

- Recommended WF

  - Please discuss proposals.

  - If proposal 1 and/or 2 is agreed, LS is needed to RAN2.

### Topic 5 Additional signalling aspects

#### Issue 5-1: Semi-static schemes

- Proposals from companies

  - General proposals

    - Proposal 1 (Huawei) RAN4 to conclude whether to introduce semi-static indication by RAN4#115.

  - Proposals not supporting semi-static schemes

    - Proposal 2 (Nokia): Follow RAN1 working assumption and consider DCI only.

    - Proposal 3 (Samsung): Do not consider semi-static solutions.

    - Proposal 4 (Ericsson): RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes.

    - Proposal 5 (Interdigital): Do not consider semi-static solutions

  - Proposals supporting semi-static schemes

    - Proposal 6 (Qualcomm): In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well.

    - Proposal 7 (vivo): RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported.

    - Proposal 8 (ZTE): Support both dynamic and semi-static gap skipping indication as a packet, and NW decide which scheme is to be enabled.

    - Proposal 9 (Huawei): RAN4 to consider following approach to move forward:

      - The DCI-based indication shall be supported by default if UE support MG skipping.

      - RAN4 to introduce semi-static indication to skip measurement gap as additional UE capability.

    - Proposal 10 (CMCC): for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes.

    - Proposal 11 (Apple): In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported.

- Proposals merged by the Moderator

  - Further discussion based on recommended options

    - Option 1: Do not consider semi-static solutions

    - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution.

      - Suboptions regarding type of support

        - Option 2a: RRC pattern-based

        - Option 2b: MAC-CE based

      - Suboptions regarding UE support

        - Option 2c:  Support both dynamic and semi-static gap skipping indication as a packet, and NW decides which scheme is to be enabled.

        - Option 2d: The DCI-based indication shall be supported by default if UE supports MG skipping.

          - RAN4 to introduce semi-static indication to skip measurement gap as additional UE capability.

- Recommended WF

  - Discuss the proposals.

#### Issue 5-2: Skipping indication on frequency domain

- Background: RAN1 agreement related to this issue in RAN1 #120

Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

- Proposals

  - Option 1 (vivo): RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements.

  - Option 2: Further RAN4 discussion needed

    - Option 2a (Qualcomm) If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers.

    - Option 2b (ZTE): For the gap/restriction skipping impact in frequency domain,

      - For the gap skipping, depend on whether the UE capable of per-FR gap

        - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

        - Otherwise, no need to distinguish different FR.

      - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

    - Option 2c (Huawei, Interdigital, Apple): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

- Recommended WF

  - Issue has been agreed in RAN1. Please discuss if Option 1 is enough or if further discussion is needed.

#### Issue 5-3: Partial measurement skipping

- Proposals

  - Proposal 1 (ZTE): Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered.

- Recommended WF

  - Please discuss the proposal.

### Topic 6 CR handling

#### Issue 6-1: Naming convention

- Options regarding cancellation/skipping

  - Option 1: Measurement gap cancelation

  - Option 2: Measurement gap skipping

- Variable in formulas for number of canceled/skipped occasions

  - Option 1: XPSS/SSS,gaps

  - Option 2: Nskip

  - Option 3: Lskipping

- Indication of which formula/table to use

  - Option 1: for UE supporting [Enabling TX/RX during measurement gap],

  - Option 2: For UE with at least one measurement gap occasion cancelled due to [the UE XR operation]

  - Option 3: For the UE supporting [64-1] under the conditions in clause 9.1.y.4 (new)

  - Option 4: For UE capable of [measurement gap skipping indication] and if [measurement gap skipping enabled] is configured by NW

  - Option 5: “… for UE configured with MG cancellation due to its XR operation”

- Recommended WF:

  - It is recommended to align as much as possible with existing terminology agreed in RAN1.

### Topic 7 UE capabilities

#### Issue 7-1: UE capabilities for XR – general proposals

- Proposals from companies

  - Proposal 1 (vivo): FG 64-1 is the prerequisite feature for UE capability of 3ms time offset and UAI of ratio indication.

  - Proposal 2 (vivo): For both UE capability of 3ms time offset and UAI of ratio indication, the reporting granularity is per-UE.

  - Proposal 3 (Nokia): RAN4 to discuss whether RAN4 should define the UE capability for UAI.

  - Proposal 4 (Nokia): Define UE feature for support of UAI as optional with per UE capability.

  - Proposal 5 (Nokia): RAN4 to discuss the need for additional UE feature for XR on top of RAN1 and RAN2 discussed features.

- Options for discussion

  - Is a new capability for 3 ms signalling needed?

    - Option 1; yes, as per UE capability

    - Option 2: no

  - Should RAN4 define the UE feature for UAI reporting

    - Option 1: yes, as per UE capability

    - Option 2: no

- Recommended WF

  - Discussion on UE features needed in RAN4. The following points are to be considered:

    - RAN1 agreed feature already includes 3/5 ms option. Please check if additional capability is needed

#### Issue 7-2: UE capabilities for XR - tables

- Background information: RAN1 agreed feature for XR in R1-2502977

| - Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 64. NR_XR_Ph3 | 64-1 | Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1] | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. 4. The minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction FFS: other component (which may be separate FG) | FFS | YES | n/a | UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1] | [Per UE or Per band] |  |  |  | Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms} | Optional with capability signalling |

- Company proposals

  - Proposal 1 (vivo):

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57.NR_XR_Ph3 | 57-1 | Time offset for enabling Tx/Rx during measurement gap | Support of 3ms time offset for enabling Tx/Rx during measurement gap indicated by DCI | 64-1 | Yes | N/A | UE does not support 3ms time offset for enabling Tx/Rx during measurement gap indicated by DCI | Per UE | N/A | No |  | Candidate value: [3ms] | Optional with capability signalling |
| 57.NR_XR_Ph3 | 57-2 | Supports indication of a ratio of gap occasions for enabling Tx/Rx | Supports of indication of a ratio of gap occasions during a time period of 1 s for enabling Tx/Rx. | 64-1 | Yes | N/A | UE cannot recommend ratio of gap occasions for enabling Tx/Rx | Per UE | N/A | No |  | Candidate value: [0, 20%, 40%, 60%] | Optional with capability signalling |

  - Proposal 2 (Nokia):

| - Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57.NR_XR_Ph3 | 57-1 | Recommended ratio of gap occasions for cancelation report. | Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation. | 64-1 | yes | n/a | The UE does not support reporting recommended ratio for gap cancelation | Per UE | no | no |  |  | Optional with capability signalling |

- Recommended WF:

  - Discussion needed based on RAN1 feature and the outcome of issue 7-1