3GPP TSG-RAN WG4 Meeting #115	R4-2505310

St Julian’s, Malta, 19th – 25th May 2025

3GPP TSG RAN WG1 #120bis		                                        R1-2503108

Wuhan, China, April 7th – 11th, 2025

Title:	LS to RAN4 on time instance B for on-demand SSB SCell operation

Release:	Release 19

Work Item:	Netw_Energy_NR_enh-Core

Source:	TSG RAN WG1

To:	TSG RAN WG4

Cc:

Contact person:	Seonwook Kim

sseonwook.kim@lgepartner.com

# 1	Overall description

RAN1 has made the following agreement for MAC-CE based deactivation of on-demand SSB (OD-SSB) transmission on SCell.

Agreement

- Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

  - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

    - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

      - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

    - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - (Working assumption) T=T_min

    - RAN4 to confirm the above two working assumptions

    - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

RAN1 respectfully asks RAN4 if the two working assumptions can be confirmed to agreement.

# 2	Actions

To TSG RAN4

ACTION: 	RAN1 respectfully asks RAN4 if the two working assumptions can be confirmed to agreement.

# 3	Dates of next TSG RAN WG1 meetings

TSG-RAN WG1 Meeting #121	19h – 23st May 2025				Malta, MT

TSG-RAN WG1 Meeting #122	25th – 29th August 2025			Bengaluru, IN