3GPP TSG-RAN WG4 Meeting #115	R4-2505308

St Julian’s, Malta, 19th – 25th May 2025

3GPP TSG RAN WG1 Meeting #120bis                                                                             R1-2503091

Wuhan, China, 07th – 11th April 2025

Title:	Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP

Response to:	          R1-2501699 (R4-2502667)

Release:	Release 19

Work Item:	          NR_MIMO_Ph5

Source:	RAN1

To:	RAN4

Cc:

Contact Person:

#### Name: Li Guo

Tel. Number:

E-mail Address:	guoli@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 would like to thank RAN4’s LS on DL reference timing to support two TA enhancement for asymmetric DL sTRP/UL mTRP. With regard to the RAN4’s question, RAN1 provides the corresponding answer as below:

Question: Based on RAN1 LS (R1-2410870), RAN4 discussed timing requirement for two TA enhancement for asymmetric DL sTRP/ UL mTRP. RAN4 would like to check with RAN1 whether RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s).

Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s).

And RAN1 made the following new agreement for further clarification:

Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI

2. Actions:

To RAN4

ACTION: RAN1 respectfully asks RAN4 to take the above replied information into consideration for their future work.

3. Date of Next TSG-WG1 Meetings:

RAN1#121	 19 - 23 May, 2025		St Julian’s, Malta

RAN1#122	 25 - 29 August, 2025		Bengaluru, India