3GPP TSG- WG4 Meeting # 115	R4-2505323

# St Julian’s, Malta, 19th – 25th May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | Draft | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to 38.101-1: Time mask for LB-LB switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Core |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Rel-19 time mask requirements for low band CA via switching are not specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Rel-19 time mask requirements for low band CA via switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE RF requirements of Rel-19 NR_LBCA_Sw-Core WI are absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

>> Start of Changes <<

6.3A.3.3.7	Time mask for FDD-SDL carrier aggregation via low band switching

The switching time mask requirements specified in this sub-clause are applicable for FDD-SDL carrier aggregation via low band switching when the capability [supportedLowBandSwitching-r19] is present, and are only applicable for CA via low band switching specified in clause [6.x.x] of TS38.214[10].

The low band switching is performed between FDD band and SDL band. The switching time masks in Figure 6.3A.3.3.7-1 and [Figure 6.3A.3.3.7-2a/2b] are applicable. For low band CA via switching from SDL carrier to FDD carrier, the switching periods described in Figure 6.3A.3.3.7-1 are located in SDL carrier. [For low band CA via switching from FDD carrier to SDL carrier, the switching periods described in Figure 6.3A.3.3.7-2a/2b are located in SDL/FDD carrier.] For each band pair of FDD and SDL, the length of uplink switching period with X OFDM symbols is indicated by RRC signalling [switchingPeriodConfigForLBSwitching]. The gap for switching period would be reserved without transmissions in the RRC configured switching pattern.

Figure 6.3A.3.3.7-1: Time mask for low band CA via switching from SDL carrier to FDD carrier, where the switching period is located in SDL carrier

[

Figure 6.3A.3.3.7-2a Time mask for low band CA via switching from FDD carrier to SDL carrier, where the switching period is located in FDD carrier

Figure 6.3A.3.3.7-2b Time mask for low band CA via switching from FDD carrier to SDL carrier, where the switching period is located in SDL carrier]

>> End of Changes <<