3GPP TSG-RAN WG4 Meeting #115		R4-2505344

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.7.2

Source: 	TELUS

Title: 	UE features for low band CA via switching

# Introduction

This contribution presents our perspectives on UE capabilities for low band CA via switching.

# Discussion

In RAN4#114bis, the following agreements [1] were achieved.

RAN4#114bis Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded Issue 5: Test methodology - Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

Under these agreements, we can introduce a RAN4 feature group that defines switching period capability per band pair per band combination.

Proposal 1: For Low NR band carrier aggregation via switching, a FG 59-1 is introduced with the following:

- Component:

UE to indicate support of LB-LB carrier aggregation via switching between an FDD band and an SDL band.

switchingPeriodForFDD-SDL indicates the length the switching time between an FDD band and a SDL band: 35us represents 35 us of switching time, 140us represents 140us of switching time, as specified in TS 38.101-1.

For a complete view of the new FG, please refer to the table in the Appendix.

Proposal 2: The table in the Appendix is a starting point for further UE feature discussion on Rel-19 low band CA via switching.

# Conclusions

In this contribution, the following are proposed:

Proposal 1: For Low NR band carrier aggregation via switching, a FG 59-1 is introduced with the following:

- Component:

UE to indicate support of LB-LB carrier aggregation via switching between an FDD band and an SDL band.

switchingPeriodForFDD-SDL indicates the length the switching time between an FDD band and a SDL band: 35us represents 35 us of switching time, 140us represents 140us of switching time, as specified in TS 38.101-1.

Proposal 2: The table in the Appendix is a starting point for further UE feature discussion on Rel-19 low band CA via switching.

# References

- R4-2505223, WF on UE RF requirements for LB CA via switching, RAN4#114bis

# Appendix: UE features for low band CA via switching

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_LBCA_Sw | 59-1 | Switching period for switching between a LB FDD band and an SDL band | UE to indicate support of LB-LB carrier aggregation via switching between an FDD band an SDL band. switchingPeriodForFDD-SDL indicates the length the switching time between an FDD band and a SDL band: 35us represents 35 us of switching time, 140us represents 140us of switching time, as specified in TS 38.101-1. | N/A | Yes | N/A | Operation of Carrier aggregation between FDD and SDL via switching is not supported. | Per band pair per band combination |  | Applicable to FR1 only |  |  | Optional with capability signalling |