3GPP TSG-RAN WG4 Meeting #115	R4-2505373

Malta, May 2025

Source	:	Qualcomm Incorporated

Title	:	On test mode references in core requirements

Agenda item	:	4.2.1

Document for	:	Discussion

# 1.	Introduction

During the Rel-15 work phase, there were many unknowns in measurement methods, especially for OTA requirements in FR2. RAN4 and RAN5 have since developed their understanding and defined detailed test procedures to verify core requirements. With 6G looming, it is now useful to look back on the requirement and better demarcate responsibilities between RAN4 and RAN5.

# 2. 	Discussion

As a general philosophy, RAN4 must define UE RF core requirements assuming behavior only defined in RAN1 and RAN2.

In FR2, the specific issue is reference to beam-lock (BL) functionality. Now, BL is not relevant for field operation. The need for BL is in fact intimately related to the test setup and method. A simple example to illustrate this dependence is TRP measurement: because the test system must measure all spatial points sequentially, it is necessary to ensure that the UE does not change its UL settings for the duration of the test. Consider another test set that has as many spatial antennas as grid points in the current method, and a simultaneous capture can be made across all of these antennas. In this method, a BL is un-necessary, because the duration of the test would not exceed the basic measurement interval of EIRP.

Observation 1: Beam lock and other test modes are instituted based on a certain assumed verification test setup and procedure.

The test set up and method are RAN5’s jurisdiction, and for this reason, definition and application of any related test modes must be left to them. The RAN4 UE RF core requirement should instead only focus on requirement as relevant to field performance, without implicit or explicit reliance on any special behavior not defined in RAN1 or RAN2.

Proposal 1: Remove all references, implicit and explicit, to test modes (for example: beam lock) from the RAN4 UE RF core requirements. Test mode application is left to RAN5, in conjunction with test set up and procedure design.

Unfortunately, this view is applied unevenly in 38.101-2, with many Tx tests referencing BL, while most Rx tests not. Looking ahead, it would be beneficial to streamline this aspect in preparation for 6G.

We conclude with some examples of how to implement the proposal above. A CR can be formulated once the general concept is agreed.