3GPP TSG-RAN WG4 Meeting #115                                   R4-2505374

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.1.2.1.1

Source: 	Murata Manufacturing Co Ltd.

Title: 	Further discussion on HPUE MSD simplification LUTs for 1 UL and 2 UL

Document for:	Discussion

Introduction

In the RAN4#114-bis meeting, the general MSD simplification rule for HPUE was discussed and WF [1] was approved. In this WF, it was agreed that RAN4 specifies LUTs for 1 UL and 2 UL with companion table which maps a maximum MSD.

For 1 UL, RAN4 reached an agreement of three maximum MSD, e.g. 3dB/6dB/9dB depending on MSD type (UL harmonic, Harmonic mixing and Cross-band isolation), power class and number of Tx even though there are some cases where LUTs may represent a slight tightening or relaxation for some band combinations.

However, 2 UL case was not discussed enough due to the diverse opinions from interested companies and lack of time.

Moreover, the tabulated values in HPUE simplification MSD LUTs for 1 UL as well as 2 UL should also be finalized.

In this contribution we first analyze the 2 UL HPUE MSD due to IMD to determine the maximum MSD for PC2 and PC1.5 2Tx/3Tx scenarios. Then the final LUTs for both 1 UL and 2 UL are proposed.

Discussion

Two UL HPUE MSD calculation (IMD)

In the previous meetings, there have not been discussions about two UL HPUE MSD calculation (IMD) due to diverse inputs and lack of time. Moreover, unlike one UL HPUE MSD calculation, two UL HPUE MSD calculation due to IMD from two aggressors is not simple and needs more analysis. This is because there are many power class combinations for one single band, different number of Tx chains, different IMD order with different index and different linearity between PC3 PA and PC2 PA.

Table 1 summarizes five categories of power class and number of Tx – PC3 2Tx (baseline), PC2 2Tx, PC2 3Tx, PC1.5 2Tx and PC1.5 3Tx. The third and the fourth column mean power combination to achieve 2 UL CA power class and PA power capability for each band respectively. The fifth column denotes the case number.

It should be noted that some cases (5, 6, 7, 8, 10) have not been specified until Rel-18 and could be introduced from Rel-19 or later release. And we do not yet have specified MSD for those cases until Rel-18.

We investigated two UL HPUE MSD rules for each category based on PC3 MSD (case #1) with different IMD orders.

Table 1 Two UL MSD categories of PC3, PC2 and PC1.5 with power class combinations

|  | Combination | Band A | Band B |  | Case # | Rel |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 2Tx | Power | 20 | 20 |  |  |  |
|  | PA | PC3 PA | PC3 PA |  | 1 | R15 |
| PC2 2Tx | Power | 23 | 23 |  |  |  |
|  | PA | PC3 PA | PC3 PA |  | 2 | R17 |
|  |  | PC3 PA | PC2 PA |  | 3 | R17 |
|  |  | PC2 PA | PC2 PA |  | 5 | R19 |
| PC2 3Tx | Power | 23 | 23 |  |  |  |
|  |  |  | 20 | 20 |  |  |
|  | PA | PC3 PA | PC3 PA | PC3 PA | 4 | R18 |
|  |  | PC2 PA | PC3 PA | PC3 PA | 6 | R19 |
| PC1.5 2Tx | Power | 26 | 26 |  |  |  |
|  | PA | PC2 PA | PC2 PA |  | 7 | R19 |
| PC1.5 3Tx | Power | 23 | 27.8 |  |  |  |
|  |  |  | 24.8 | 24.8 |  |  |
|  | PA | PC3 PA | PC2 PA | PC2 PA | 9 | R18 |
|  | Power | 26 | 26 |  |  |  |
|  |  |  | 23 | 23 |  |  |
|  | PA | PC2 PA | PC3 PA | PC3 PA | 8 | R19 |
|  |  | PC2 PA | PC2 PA | PC2 PA | 10 | R19 |

PC2 2Tx

There are three PA combinations in Table 1 for PC2 2Tx which are PC3 PA + PC3 PA (case 2), PC3 PA + PC2 PA (case 3) and PC2 PA + PC2 PA (case 5). It is well known that PC2 PA IPn is 3dB higher than PC3 PA IPn, so the interferer increase for case 5 is the smallest. However, it is not reasonable to specify three PC2 2Tx MSD requirements in the specification document. Moreover, the IMDn from other components (e.g. LNA) except PC2 PA can be dominant or comparable for some cases such as triple beat scenario or when the victim DL band is 3rd band. Considering MSD is the minimum requirement and to simplify the MSD rules as much as possible, the worst-case interferer increase (PC3 PA + PC3 PA, case 2) should be referred to calculation. In conclusion, the interferer increase for IMDn should be 3*n dB.

Proposal 1: For PC2 2Tx, RAN4 to use the maximum MSD of 3*n dB, where n is IMD order.

PC2 3Tx

First, 2 band UL-2 band DL band combinations when 1Tx is on FDD specified in Rel-18 would be considered.

During the Rel-18 discussion, it was agreed that the requirements specified based on 1Tx-1Tx UL configuration are applicable to 1Tx-2Tx UL configuration. There is no consideration of MRC imbalance change between 2Tx and 3Tx, higher headroom for PC2 TDD PA versus a PC3 TDD PA, or different coupling between PAs in a 2Tx and 3Tx architecture. Therefore, there is no discrepancy between PC2 2Tx and PC2 3Tx in the spec.

Following the same principle, PC2 2Tx MSD calculation can be applicable to PC2 3Tx MSD calculation.

Proposal 2: For PC2 3Tx with 2 band UL-2 band DL when 1Tx is on FDD, RAN4 to use the maximum MSD of 3*n dB, where n is IMD order.

Second case is 2 band UL-2 band DL band combinations when 2Tx is on FDD which has not been analysed yet. Figure 1 shows two RF FE architecture options. Figure 1(a) is when antenna is shared between two bands (e.g. LB+MB) and Figure 1(b) is when separate antenna is used between two bands (e.g. LB+UHB).

(a)

(b)

Figure 1. Example RF FE architecture for 3Tx with 2 band UL-2 Band DL

If it is assumed that PA forward IMD due to PCB isolation is dominant in Figure 1 (a) and (b), or PA reverse IMD due to antenna isolation is dominant in Figure 1(b), both primary FDD Rx path and diversity FDD Rx path of Band B would see the same IMD interferer, and MRC gain is not achieved as in 1UL 2Tx HPUE MSD simplification analysis. Therefore, the maximum MSD could be 3*n + 3 dB if the same difference (i.e. 3dB) between 1UL 1Tx HPUE MSD and 2Tx HPUE MSD is applied.

Proposal 3: For PC2 3Tx with 2 band UL-2 band DL when 2Tx is on FDD, RAN4 to use the maximum MSD of 3*n + 3 dB, where n is IMD order.

Third, 2 band UL-3 band DL band combinations which are not included until Rel-18 would be considered as in Figure 2. In this case, LNA IMD might be a dominant contributor for MSD calculation. In other words, IMD from LNA in primary Rx path and IMD from LNA in diversity Rx path should be calculated separately for PC3 MSD calculation through MRC between them.

Figure 2. Example RF FE architecture for 3Tx with 2 band UL-3 Band DL

There are two paths for LNAs’ IMD generation:

- PCB isolation from PA output to LNA input

- Filter attenuation (+ Antenna isolation for diversity Rx LNA) from PA output to LNA input

If we assume PCB isolation is identical and dominant, the IMD increase (IMDn) of PC2 is calculated as Eq. (1).

∆IMDn=a*23-20+b*23-20-a+b-1*∆IIPn= 3*a+b=3*n   (1)

where IIPn =0 since LNA IIPn is same regardless of power class.

Note that this IMD increase is equally applied to both primary Rx LNA and diversity Rx LNA, which means there is no imbalance change between 2Tx and 3Tx. Therefore, we can use maximum MSD of 3*n dB as Proposal 2.

However, if filter attenuation	 is not enough for suppression of Tx aggressors for some band combinations, the IMD increase for primary Rx LNA and diversity Rx LNA would be different.

In this case, the 3rd band (Band C) LNA of diversity path would see higher Tx aggressor signal of Band B from the additional PA in red dotted circle. In other words, the 2Tx MSD is calculated by MRC gain such that the LNA IMD in diversity path might be lower than LNA IMD in primary path thanks to the antenna isolation for Band B Tx aggressor. However, there would be no such an antenna isolation in 3Tx architecture. This would result in higher IMD increase in diversity path LNA and degrade the MSD more compared to PC2 2Tx MSD as below equations assuming 10dB antenna isolation.

∆IMDn PRx=a*23-20+b*20-20-a+b-1*∆IIPn= 3*a     (2)

∆IMDn DRx=a*23-20+b*20-20+AntennaIso-a+b-1*∆IIPn

=3*a+10*b                                               (3)

Table 2 shows the IMDn increase for each LNAs and resulting Max MSD through MRC assuming 6dB interferer imbalance for PC3 MSD. It should be noted that Max MSD is larger than 3*n dB by 3dB because the IMD at DRx LNA is significantly increased as in Eq. (3).

Table 2 IMDn increase for PRx and DRx LNAs and Max MSD for PC2 3Tx for 2 band UL-3 band DL band combinations

| Order | a | b | IMD increase at LNA_P (dB) | IMD increase at LNA_D (dB) | Max ΔMSD (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 3 | 13 | 8.76 → 9 | 9 |
| 3 | 1 | 2 | 3 | 23 | 8.66 → 9 | 12 |
|  | 2 | 1 | 6 | 16 | 11.76 → 12 |  |
| 4 | 1 | 3 | 3 | 33 | 7.91 → 9 | 15 |
|  | 2 | 2 | 6 | 26 | 11.66 → 12 |  |
|  | 3 | 1 | 9 | 19 | 14.76 → 15 |  |
| 5 | 1 | 4 | 3 | 43 | 7.59 → 9 | 18 |
|  | 2 | 3 | 6 | 36 | 10.91 → 12 |  |
|  | 3 | 2 | 9 | 29 | 14.66 → 15 |  |
|  | 4 | 1 | 12 | 22 | 17.76 → 18 |  |

The final Max MSD can be mapped to 9/12/15/18dB for IMD2/3/4/5 for simplicity of LUTs.

Proposal 4: For PC2 3Tx with 2 band UL-3 band DL, RAN4 to use the maximum MSD of 9/12/15/18 dB (3*n+3 dB) for IMD2/3/4/5.

PC1.5 2Tx

For this case, two PC2 PAs are used which means that the IMDn increase is 3+3*n dB from PC3 PAs. For 2UL-2DL band combinations, PA IMD contribution is dominant so 3+3*n dB IMDn increase could be used. However, for 2 band UL-3 band DL band combinations, LNA or other component IMD contribution is dominant, so 6*n dB IMDn increase could be expected. To make the LUT simple as much as possible, the worst-case scenario which is 6*n dB IMDn increase might be used for PC1.5 2Tx.

Proposal 5: For PC1.5 2Tx, RAN4 to use the maximum MSD of 6*n dB, where n is IMD order.

PC1.5 3Tx

For PC1.5 3Tx, there are two power combinations: 23dBm + 2*24.8dBm (case 9, Rel-18) and 26dBm + 2*23dBm (case 8, 10, Rel-19). So, we will analyse separate calculations in this contribution.

23dBm + 2*24.8dBm

This power combinations for PC1.5 3Tx has been specified in Rel-18 with 2 band UL-2 band DL band combinations with the 1Tx being on PC3 FDD. Unlike the other cases where the power of two bands is same, the power of two bands is different. Therefore, IMDn increase should be examined for every IMD order index (a, b) at PC3 PA at Band A as in Table 3 since PC3 PA at Band A experiences the greatest IMDn increase. As a result, we can see that the IMDn increase is not constant for a given IMD order but depends on the specified IMD order index (a, b). To simplify the companion table as much as possible, the IMD increase for each IMD order and index could me unified as 6*n dB even though there is one case (IMD5 (1,4)) which is tightened by 4.2dB.

Table 3 IMDn increase at PC3 PA for 23dBm + 2*24.8dBm

| IMD order | Band A | Band B | Delta at PC3 PA_(Band A) vs PC3 2Tx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | a | b | Pin_A | Pin_B | IPn | IMDn |
| 2 | 1 | 1 | 3 | 7.8 | 0 | 10.8 → 12 |
| 3 | 1 | 2 | 3 | 7.8 | 0 | 18.6 → 18 |
|  | 2 | 1 | 3 | 7.8 | 0 | 13.8 → 18 |
| 4 | 1 | 3 | 3 | 7.8 | 0 | 26.4 → 24 |
|  | 2 | 2 | 3 | 7.8 | 0 | 21.6 → 24 |
|  | 3 | 1 | 3 | 7.8 | 0 | 16.8 → 24 |
| 5 | 1 | 4 | 3 | 7.8 | 0 | 34.2 → 30 |
|  | 2 | 3 | 3 | 7.8 | 0 | 29.4 → 30 |
|  | 3 | 2 | 3 | 7.8 | 0 | 24.6 → 30 |
|  | 4 | 1 | 3 | 7.8 | 0 | 19.8 → 30 |

From Rel-19, new combination using PC2 FDD PA for Band A might be introduced. For those cases the IPn is increased by 3dB compared to PC3 FDD PA for Band A resulting in less IMDn increase. However, it is proposed to use the IMDn increase in Table 3 since this is the worst-case scenario.

For 2 band UL-3 band DL, same approach as PC2 3Tx can be used with following IMDn increase equations for PRx and DRx paths.

∆IMDn PRx=a*23-20+b*24.8-20-a+b-1*∆IIPn= 3*a+4.8*b  (4)

∆IMDn DRx=a*23-20+b*24.8-20+AntennaIso-a+b-1*∆IIPn

=3*a+14.8*b                                              (5)

Table 4 IMDn increase for PRx and DRx LNAs and Max MSD for PC1.5 3Tx for 2 band UL-3 band DL band combinations

| Order | a | b | IMD increase at LNA_P (dB) | IMD increase at LNA_D (dB) | Max ΔMSD (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 7.8 | 17.8 | 15.36 → 15 | 15 |
| 3 | 1 | 2 | 12.6 | 32.6 | 18.26 → 21 | 21 |
|  | 2 | 1 | 10.8 | 20.8 | 16.56 → 18 |  |
| 4 | 1 | 3 | 17.4 | 47.4 | 22.31 → 24 | 24 |
|  | 2 | 2 | 15.6 | 35.6 | 21.26 → 24 |  |
|  | 3 | 1 | 13.8 | 23.8 | 19.56 → 21 |  |
| 5 | 1 | 4 | 22.2 | 62.2 | 26.79 → 27 | 27 |
|  | 2 | 3 | 20.4 | 50.4 | 25.31 → 27 |  |
|  | 3 | 2 | 18.6 | 38.6 | 24.26 → 27 |  |
|  | 4 | 1 | 16.8 | 26.8 | 22.56 → 24 |  |

26dBm + 2*23dBm

This combination has not been introduced until Rel-18. It would be recommended that the same principle as PC2 3Tx is applied, which uses the same maximum MSD (i.e. 6*n dB) of PC1.5 2Tx for PC1.5 3Tx with 2 band UL-2 band DL when 1Tx is on FDD.

For 2 band UL-2 band DL when 2Tx is on FDD, both primary FDD Rx path and diversity FDD Rx path of Band B would see the same IMD interferer, and MRC gain is not achieved as in 1UL 2Tx HPUE MSD simplification analysis considering the RF FE architecture options in Figure 1. Therefore, the maximum MSD could be 6*n + 3 dB if the same difference (i.e. 3dB) between 1UL 1Tx HPUE MSD and 2Tx HPUE MSD is applied.

For 2 band UL-3 band DL, same approach as PC2 3Tx can be used with following IMDn increase equations for PRx and DRx paths.

∆IMDn PRx=a*26-20+b*23-20-a+b-1*∆IIPn= 6*a+3*b     (6)

∆IMDn DRx=a*26-20+b*23-20+AntennaIso-a+b-1*∆IIPn

=6*a+13*b                                               (7)

Table 5 IMDn increase for PRx and DRx LNAs and Max MSD for PC1.5 3Tx for 2 band UL-3 band DL band combinations

| Order | a | b | IMD increase at LNA_P (dB) | IMD increase at LNA_D (dB) | Max ΔMSD (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 9 | 19 | 14.76 → 15 | 15 |
| 3 | 1 | 2 | 12 | 32 | 17.66 → 18 | 21 |
|  | 2 | 1 | 15 | 25 | 20.76 → 21 |  |
| 4 | 1 | 3 | 18 | 45 | 19.91 → 21 | 27 |
|  | 2 | 2 | 18 | 38 | 23.66 → 24 |  |
|  | 3 | 1 | 21 | 31 | 26.76 → 27 |  |
| 5 | 1 | 4 | 18 | 58 | 22.59 → 24 | 33 |
|  | 2 | 3 | 21 | 51 | 25.91 → 27 |  |
|  | 3 | 2 | 24 | 44 | 29.66 → 30 |  |
|  | 4 | 1 | 27 | 37 | 32.76 → 33 |  |

The final Max MSD can be mapped to 15/21/27/33 dB (6*n+3 dB) for IMD2/3/4/5 for simplicity of LUTs when Table 4 and Table 2 are compared.

Proposal 6: For PC1.5 3Tx with 2 band UL-2 band DL when 1Tx is on FDD, RAN4 to use the maximum MSD of 6*n dB, where n is IMD order.

Proposal 7: For PC1.5 3Tx with 2 band UL-2 band DL when 2Tx is on FDD, RAN4 to use the maximum MSD of 6*n + 3 dB, where n is IMD order.

Proposal 8: For PC2 3Tx with 2 band UL-3 band DL, RAN4 to use the maximum MSD of 15/21/27/33 dB (6*n+3 dB) for IMD2/3/4/5.

HPUE MSD LUTs with companion table for both 1 UL and 2 UL

To converge 1 UL and 2 UL MSD look up tables, we propose to use simple equation as below though this assumes 0dB interferer imbalance between primary and secondary path with higher MSD in lower PC3 MSD region.

MSD（x）=10*log10(1+10x10(10MSD10-1))

Table 6 shows the LUT for each maximum MSD and companion table It should be noted that some PC3 MSD ranges are merged for 3/6/9dB maximum MSD since the difference is small. On the other hand, lower PC3 MSD region would be practically used for higher maximum MSD since PC3 MSD due to IMD4/IMD5 would be smaller than other cases.

Proposal 9: RAN4 to use Table 6 for HPUE MSD LUTs and companion table

Table 6 HPUE MSD LUTs and companion table

| PC31Tx MSD (dB) | DMSDmax (dB) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 0.1≤ ≤0.3 | 1.4 | 2 | 2.8 | 3 | 4.9 | 7.2 | 9.8 | 12.5 | 15.4 | 18.4 | 21.3 |
| 0.4≤ ≤0.6 |  |  |  | 4.7 | 7 | 9.6 | 12.4 | 15.3 | 18.2 | 21.2 | 24.2 |
| 0.7≤ ≤0.9 |  |  | 3.7 | 5.8 | 8.3 | 11.1 | 13.9 | 16.8 | 19.8 | 22.8 | 25.8 |
| 1.0≤ ≤1.9 |  | 3.2 | 5.4 | 8 | 10.8 | 13.7 | 16.6 | 19.6 | 22.6 | 25.6 | 28.5 |
| 2.0≤ ≤2.9 | 1.8 | 3.9 | 6.5 | 9.2 | 12.1 | 15 | 18 | 20.9 | 23.9 | 26.9 | 29.9 |
| 3.0≤ ≤3.9 | 2.1 | 4.5 | 7.1 | 10 | 12.9 | 15.8 | 18.8 | 21.8 | 24.8 | 27.8 | 30.8 |
| 4.0≤ - ≤4.9 | 2.3 | 4.8 | 7.6 | 10.5 | 13.4 | 16.4 | 19.4 | 22.4 | 25.4 | 28.4 | 31.4 |
| 5.0≤ - ≤6.9 | 2.6 | 5.3 | 8.2 | 11.1 | 14.1 | 17.1 | 20.1 | 23.1 | 26.1 | 29.1 | 32.1 |
| 7.0≤ - ≤9.0 | 2.8 | 5.6 | 8.5 | 11.5 | 14.5 | 17.5 | 20.5 | 23.5 | 26.5 | 29.5 | 32.5 |
| ≥10 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |

| MSD type |  | DMSDmax (dB) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |  |  |  |  |  |  |  |  |
|  | Rx mixing | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |  |  |  |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |  |  |  |
| 2UL | IMD2 |  | PC2 | PC21,2 | PC1.5 | PC1.53,4 |  |  |  |  |  |  |
|  | IMD3 |  |  | PC2 | PC21,2 |  | PC1.5 | PC1.53,4 |  |  |  |  |
|  | IMD4 |  |  |  | PC2 | PC21,2 |  |  | PC1.5 | PC1.53,4 |  |  |
|  | IMD5 |  |  |  |  | PC2 | PC21,2 |  |  |  | PC1.5 | PC1.53,4 |
| Note 1: For PC2 3Tx with 2 band UL-3 band DL |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 2: For PC2 3Tx with 2 band UL-2 band DL when 2Tx is on FDD |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3: For PC1.5 3Tx with 2 band UL-3 band DL |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 4: For PC1.5 3Tx with 2 band UL-2 band DL when 2Tx is on FDD |  |  |  |  |  |  |  |  |  |  |  |  |

Conclusion

In this contribution, we analyzed and provided our view on the HPUE MSD derivation rules based on the existing PC3 MSD. With this analysis result, we make the following proposals.

Proposal 1: For PC2 2Tx, RAN4 to use the maximum MSD of 3*n dB, where n is IMD order.

Proposal 2: For PC2 3Tx with 2 band UL-2 band DL when 1Tx is on FDD, RAN4 to use the maximum MSD of 3*n dB, where n is IMD order.

Proposal 3: For PC2 3Tx with 2 band UL-2 band DL when 2Tx is on FDD, RAN4 to use the maximum MSD of 3*n + 3 dB, where n is IMD order.

Proposal 4: For PC2 3Tx with 2 band UL-3 band DL, RAN4 to use the maximum MSD of 9/12/15/18 dB (3*n+3 dB) for IMD2/3/4/5.

Proposal 5: For PC1.5 2Tx, RAN4 to use the maximum MSD of 6*n dB, where n is IMD order.

Proposal 6: For PC1.5 3Tx with 2 band UL-2 band DL when 1Tx is on FDD, RAN4 to use the maximum MSD of 6*n dB, where n is IMD order.

Proposal 7: For PC1.5 3Tx with 2 band UL-2 band DL when 2Tx is on FDD, RAN4 to use the maximum MSD of 6*n + 3 dB, where n is IMD order.

Proposal 8: For PC2 3Tx with 2 band UL-3 band DL, RAN4 to use the maximum MSD of 15/21/27/33 dB (6*n+3 dB) for IMD2/3/4/5.

Proposal 9: RAN4 to use Table 6 for HPUE MSD LUTs and companion table

Table 6 HPUE MSD LUTs and companion table

| PC31Tx MSD (dB) | DMSDmax (dB) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 0.1≤ ≤0.3 | 1.4 | 2 | 2.8 | 3 | 4.9 | 7.2 | 9.8 | 12.5 | 15.4 | 18.4 | 21.3 |
| 0.4≤ ≤0.6 |  |  |  | 4.7 | 7 | 9.6 | 12.4 | 15.3 | 18.2 | 21.2 | 24.2 |
| 0.7≤ ≤0.9 |  |  | 3.7 | 5.8 | 8.3 | 11.1 | 13.9 | 16.8 | 19.8 | 22.8 | 25.8 |
| 1.0≤ ≤1.9 |  | 3.2 | 5.4 | 8 | 10.8 | 13.7 | 16.6 | 19.6 | 22.6 | 25.6 | 28.5 |
| 2.0≤ ≤2.9 | 1.8 | 3.9 | 6.5 | 9.2 | 12.1 | 15 | 18 | 20.9 | 23.9 | 26.9 | 29.9 |
| 3.0≤ ≤3.9 | 2.1 | 4.5 | 7.1 | 10 | 12.9 | 15.8 | 18.8 | 21.8 | 24.8 | 27.8 | 30.8 |
| 4.0≤ - ≤4.9 | 2.3 | 4.8 | 7.6 | 10.5 | 13.4 | 16.4 | 19.4 | 22.4 | 25.4 | 28.4 | 31.4 |
| 5.0≤ - ≤6.9 | 2.6 | 5.3 | 8.2 | 11.1 | 14.1 | 17.1 | 20.1 | 23.1 | 26.1 | 29.1 | 32.1 |
| 7.0≤ - ≤9.0 | 2.8 | 5.6 | 8.5 | 11.5 | 14.5 | 17.5 | 20.5 | 23.5 | 26.5 | 29.5 | 32.5 |
| ≥10 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |

| MSD type |  | DMSDmax (dB) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |  |  |  |  |  |  |  |  |
|  | Rx mixing | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |  |  |  |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |  |  |  |
| 2UL | IMD2 |  | PC2 | PC21,2 | PC1.5 | PC1.53,4 |  |  |  |  |  |  |
|  | IMD3 |  |  | PC2 | PC21,2 |  | PC1.5 | PC1.53,4 |  |  |  |  |
|  | IMD4 |  |  |  | PC2 | PC21,2 |  |  | PC1.5 | PC1.53,4 |  |  |
|  | IMD5 |  |  |  |  | PC2 | PC21,2 |  |  |  | PC1.5 | PC1.53,4 |
| Note 1: For PC2 3Tx with 2 band UL-3 band DL |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 2: For PC2 3Tx with 2 band UL-2 band DL when 2Tx is on FDD |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3: For PC1.5 3Tx with 2 band UL-3 band DL |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 4: For PC1.5 3Tx with 2 band UL-2 band DL when 2Tx is on FDD |  |  |  |  |  |  |  |  |  |  |  |  |

References

[1] R4-2505143, “WF on HPUE MSD simplification”, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE, 3GPP TSG RAN WG4 Meeting #114-bis, Wuhan, China, April 7th – 11th, 2025

[2] R4-2500117, “Further discussion on HPUE MSD rules”, Murata Manufacturing Co. Ltd, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, November 17th – 21st, 2025