3GPP TSG-RAN WG4 Meeting #115                      R4-2505380

Malta, May 19 - 23 2025

Source:	China Telecom

Title:	Simulation results on BS demodulation performance requirements for MMSE-IRC

Agenda Item:	7.17.4

Document for:	discussion

Introduction

This tdoc provides the initial simulation results on BS demodulation performance requirements for MMSE-IRC according to the approved WF [1-2].

Note that PUSCH DMRS power boosting is enabled for both target and interfering PUSCH.

Simulation results

Table 1: Simulation results for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Additional DM-RS position | Ideal SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | pos1 | 11.4 | 13.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 12.5 | 15.0 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  | 8.0 | 10.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 8.5 | 11.0 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  | 5.8 | 8.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 6.0 | 8.5 |

Table 2: Simulation results for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Additional DM-RS position | Ideal SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | pos1 | 11.5 | 14.0 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 13.3 | 15.8 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  | 7.7 | 10.2 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 9.0 | 11.5 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  | 6.1 | 8.6 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 5.7 | 8.2 |

Table 3: Simulation results for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Additional DM-RS position | Ideal SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | pos1 | 11.2 | 13.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 12.6 | 15.1 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  | 8.2 | 10.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 8.6 | 11.1 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  | 6.4 | 8.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 6.2 | 8.7 |

Table 4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Additional DM-RS position | Ideal SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | pos1 | 12.0 | 14.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 12.7 | 15.2 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  | 8.6 | 11.1 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  |

Table 5: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Additional DM-RS position | Ideal SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | pos1 | 10.8 | 13.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 13.4 | 15.9 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  | 8.2 | 10.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 8.9 | 11.4 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  | 5.9 | 8.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 5.7 | 8.2 |

Table 6: Simulation results for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Additional DM-RS position | Ideal SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | pos1 | 11.8 | 14.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 12.9 | 15.4 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  | 8.2 | 10.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 9.0 | 11.7 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  | 5.9 | 8.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 5.6 | 8.1 |

Table 7: Simulation results for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Additional DM-RS position | Ideal SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | pos1 | 11.6 | 14.1 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 13.1 | 15.6 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  | 8.3 | 10.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 8.5 | 11.0 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  | 6.5 | 9.0 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 5.7 | 8.2 |

Table 8: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Additional DM-RS position | Ideal SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | pos1 | 12.1 | 14.6 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  | 12.3 | 14.8 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  |

Reference

R4-2419780, Way Forward for [113][321] NR_demod_Ph5_Part1_General_BS, China Telecom, MediaTek, RAN4#113, Nov 2024.

R4-2502272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, China Telecom. RAN4#114, February, 2025

R4-2504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, China Telecom. RAN4#114bis, April, 2025