3GPP TSG-RAN WG4 Meeting #115                      R4-2505386

Malta, May 19 - 23 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for NR channel bandwidth less than 5MHz

Agenda Item:	7.3.5

Document for:	discussion

Introduction

RAN4#114bis is the first meeting for the UE demodulation performance part for WI: NR channel BW less than 5MHz for FR1 Phase 2. The Core part has been completed in RAN#107, which has specified RF requirements for CA/DC with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band [1]:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz or 5MHz in band n100 and 5MHz or 10MHz in band n101 NOTE: Consideration of 5MHz channel bandwidth in the above “3MHz or 5MHz” is specific to band n100 and does not apply to other bands. - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

After RAN4#114bis the WF is approved in [2].

In this contribution, our views on the remaining open issues are given.

Discussion

Whether to define PDSCH CA demodulation requirements for 3MHz CHBW

Candidate options: - Option 1: Define 3MHz channel bandwidth single carrier requirements in normal PDSCH CA demodulation requirements - Option 2: Do not introduce new PDSCH requirements for <5MHz channels Way Forward: - Keep open and this issue will be concluded in the next meeting. - In parallel with this discussion, companies are encouraged to bring simulation results in the next meeting.

As per TS38.101-1, there is at least one band (n106) that only supports 3MHz channel bandwidth. Therefore, if there is a possibility that a CA configuration including n106 is selected for demodulation testing, there will be no requirement for the test.

We think the 3GPP specification should accept all NR UE implementations thus we propose to define 3MHz channel bandwidth single carrier requirements in normal PDSCH CA demodulation requirements.

Observation 1: As per TS38.101-1, there is at least one band (n106) that only supports 3MHz channel bandwidth.

Proposal 1: Define 3MHz channel bandwidth single carrier requirements in normal PDSCH CA demodulation requirements

Test applicability

In this Perf part, a new CC with 3MHz channel bandwidth is introduced. We think the existing test applicability rule in 5.1.1.7 in TS38.101-4 can be reused.

Proposal 2: The existing test applicability rule for PDSCH CA requirements in 5.1.1.7 in TS38.101-4 can be reused.

Whether to define SDR requirements for 3MHz CHBW

Candidate options: - Option 1: RAN4 to discuss whether there are SDR requirement impact for UE supporting 3MHz channel bandwidth in CA. - Option 2: Do not define new SDR requirements for <5MHz channels.

Based on observation 1, there is also a possibility for a CA configuration including n106 for SDR testing. However, the legacy SDR testing requirement is defined agnostic to the selected channel bandwidth. Therefore, RAN4 only need to update the test parameters for the 3MHz CC (e.g., PDCCH AL) and no additional simulation is needed.

Proposal 3: RAN4 need to update the SDR test parameters for the 3MHz CC (e.g., PDCCH AL) and no additional simulation is needed.

Whether to define CA CQI reporting requirements for 3MHz CHBW

Candidate options: - Option 1: The CA CQI test parameters need to be updated for 3MHz CC - Other options not precluded.

Same as SDR test, the CA CQI test requirement is defined agnostic to the selected channel bandwidth. Thus the same approach with SDR test should be used.

Proposal 4: RAN4 need to update the CA CQI test parameters for the 3MHz CC (e.g., PDCCH AL) and no additional simulation is needed.

Simulation result

Based on the following agreed simulation assumption for PDSCH CA test for 3MHz CC in [2], our Ideal and impairment simulation result is provided.

Test parameters and simulation assumptions for 3MHz CC (if introduced) <Agreement>: - Number of PRBs: 15PRBs - PDCCH aggregation level: If the 3MHz CC is PCell, reuse R18 PDCCH test configuration for 3MHz, i.e., AL4 (CCE 4 to 7) with 3 symbols. If it is SCell, - Receiver assumption: MMSE-IRC - Rank: 2 - Propagation condition: TDLA30-10 - MCS 13 - Cover both 2Tx2Rx and 2Tx4Rx with ULA Low correlation - For the other parameters: Follow the Common test parameters for CA in Table 5.2A-1 in TS38.101-4

Table. Simulation result for PDSCH CA test for 3MHz CC (FDD 15kHz SCS)

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Simulation result |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | Ideal SNR (dB) | Impairment SNR (dB) |
| 3 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.8 | 13.8 |
|  |  |  | 2x4, ULA Low | 70 | 5.3 | 7.3 |

Conclusion

Observation 1: As per TS38.101-1, there is at least one band (n106) that only supports 3MHz channel bandwidth.

Proposal 1: Define 3MHz channel bandwidth single carrier requirements in normal PDSCH CA demodulation requirements

Proposal 2: The existing test applicability rule for PDSCH CA requirements in 5.1.1.7 in TS38.101-4 can be reused.

Proposal 3: RAN4 need to update the SDR test parameters for the 3MHz CC (e.g., PDCCH AL) and no additional simulation is needed

Proposal 4: RAN4 need to update the CA CQI test parameters for the 3MHz CC (e.g., PDCCH AL) and no additional simulation is needed.

Reference

RP-250781, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation RAN#107, March 2025

R4-2504754, Way Forward for [114bis][326] NR_less_than_5MHz_Ph2_demod, China Telecom, 3GPP TSG-RAN WG4 Meeting #114bis