3GPP TSG RAN WG4 #115															     R4-2505394

Saint Julian’s, Malta, May 19th – 23rd 2025

Agenda Item: 		7.30.3

Source:			 	InterDigital Finland Oy

Title:                      NR_XR_Ph3 gap skipping impact

Document for: 		Discussion and decision

# Background

In the last RAN4 meeting #114bis, the discussions on this topic progressed. As a result, a WF has been agreed in the meeting in [1].

# Discussion

In this contribution, we share our analysis on gap skipping measurements occasions for NR XR case and the NW scheduling mechanism.

In RAN4 meeting #114bis, the agreed WF in [1] treats a variety of topics and a certain number of issues still need to be sorted out. We will address them in the following sections.

# types of measurements with gaps

In the last meeting, the following agreements were made:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG <Agreement>: - Gaps which shall not be cancelled - MUSIM - Positioning - For other gap types: - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19 - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

In this meeting an analysis related to Type-2 gaps skipping is supposed to take place. In our view, along with the specification impact, the gap skipping impact analysis on XR traffic shall be considered.

In one of our previous contributions on this WI we presented an analysis that we would like to re-emphasize.

In Table 1 below the gaps with their respective MGL and MGRP are shown for convenience:

Table 1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

From the perspective of NR XR case, we performed preliminary evaluations to identify the impact of configuring different types of gaps patterns (for measurements) when performing DL transmissions of XR traffic. Such impact is evaluated in terms of the amount of overlap in time domain when performing DL transmissions in a single carrier, for NR-SA, of different types of XR traffic, including augmented reality (AR), virtual reality (VR) and cloud gaming (CG) [3], accounting for Rate (typical data rate), PDP (Packet Delay Budget), and Jitter. The evaluated impact of the gap patterns is provided In Table 2 below:

Table 2: Impact of the gap patterns on XR traffic

| Gap Pattern Configurations |  | AR traffic | VR traffic | CG traffic |
| --- | --- | --- | --- | --- |
|  |  | Rate: 45Mbps, PDB: 10ms, Periodicity: 16.66ms, Jitter: [-4,4]ms | Rate: 30Mbps, PDB: 10ms, Periodicity: 16.66ms, Jitter: [-4,4]ms | Rate: 8Mbps, PDB: 15ms, Periodicity: 16.66ms, Jitter: [-4,4]ms |
| MG-0 | MGL: 6ms, MGRP: 40ms | 9.45% - 10.45% | 9.25% - 9.85% | 7.15% - 7.35% |
| MG-1 | MGL: 6ms, MGRP: 80ms | 4.6% - 5.1% | 4.6% - 4.9% | 3.6% - 3.65% |
| MG-2 | MGL: 3ms, MGRP: 40ms | 5.85% - 6.55% | 5.85% - 6.4% | 3.75% - 3.8% |
| MG-3 | MGL: 3ms, MGRP: 80ms | 2.85% - 3.3% | 2.85% - 3.15% | 1.85% - 1.9% |
| MG-4 | MGL: 6ms, MGRP: 20ms | 18.8% - 20.2% | 18.5% - 19.5% | 14.35% - 14.5% |
| MG-5 | MGL: 6ms, MGRP: 160ms | 2.35% - 2.5% | 2.25% - 2.4% | 1.75% - 1.85% |
| MG-6 | MGL: 4ms, MGRP: 20ms | 14% - 15.45% | 14.35% - 15.35% | 10.15% - 10.5% |
| MG-7 | MGL: 4ms, MGRP: 40ms | 7% - 7.7% | 7.15% - 7.75% | 5.1% - 5.2% |
| MG-8 | MGL: 4ms, MGRP: 80ms | 3.45% - 3.85% | 3.5% - 3.85% | 2.5% - 2.6% |
| MG-9 | MGL: 4ms, MGRP: 160ms | 1.7% - 1.85% | 1.65% - 1.85% | 1.15% - 1.2% |
| MG-10 | MGL: 3ms, MGRP: 20ms | 11.65% - 13.05% | 11.6% - 12.6% | 7.4% - 7.5% |
| MG-11 | MGL: 3ms, MGRP: 160ms | 1.3% - 1.55% | 1.35% - 1.5% | 0.85% - 0.95% |
| MG-12 | MGL: 5.5ms, MGRP: 20ms | 17% - 18.45% | 17.4% - 18.4% | 13.25% - 13.75% |
| MG-13 | MGL: 5.5ms, MGRP: 40ms | 8.55% - 9.3% | 8.75% - 9.35% | 6.65% - 7.25% |
| MG-14 | MGL: 5.5ms, MGRP: 80ms | 4.2% - 4.65% | 4.35% - 4.65% | 3.35% - 3.55% |
| MG-15 | MGL: 5.5ms, MGRP: 160ms | 2.05% - 2.2% | 2.1% - 2.25% | 1.6% - 1.675% |
| MG-16 | MGL: 3.5ms, MGRP: 20ms | 13.1% - 14.5% | 13.25% - 14.25% | 9.1% - 9.4% |
| MG-17 | MGL: 3.5ms, MGRP: 40ms | 6.55% - 7.3% | 6.65% - 7.25% | 4.6% - 4.85% |
| MG-18 | MGL: 3.5ms, MGRP: 80ms | 3.2% - 3.65% | 3.25% - 3.6% | 2.25% - 2.3% |
| MG-19 | MGL: 3.5ms, MGRP: 160ms | 1.65% - 1.75% | 1.6% - 1.75% | 1.05% - 1.15% |
| MG-20 | MGL: 1.5ms, MGRP: 20ms | 7.2% - 8.65% | 7.75% - 8.75% | 3.6% - 3.7% |
| MG-21 | MGL: 1.5ms, MGRP: 40ms | 3.55% - 4.3% | 3.85% - 4.4% | 1.75% - 1.85% |
| MG-22 | MGL: 1.5ms, MGRP: 80ms | 1.7% - 2.15% | 1.8% - 2.15% | 0.85% - 0.9% |
| MG-23 | MGL: 1.5ms, MGRP: 160ms | 0.85% - 1% | 0.85% - 1% | 0.35% - 0.355% |
| MG-24 | MGL: 10ms, MGRP: 80ms | 7.35% - 7.8% | 7.4% - 7.75% | 6.4% - 6.5% |
| MG-25 | MGL: 20ms, MGRP: 160ms | 6.7% - 6.85% | 6.65% - 6.85% | 6.15% - 6.2% |

From the evaluation results above, several gap patterns (e.g. gap pattern IDs 0, 4, 6, 10, 12, 16) result in considerable impact on XR traffic. When configured with such gap patterns for measurements, scheduling and data transmissions are unavailable up to 20% of the time. The gap patterns that have a high impact on XR traffic are typically the ones with relatively long MGL duration (e.g. > 6ms) and/or short MGRP (e.g. 20ms).

On the other hand, even when skipping measurement occasions in certain gap patterns (e.g. pattern 24, 25), the improvement achievable for XR is not significant (e.g. 6.2 – 7.8%).

Observation 1: Gap pattern IDs 0, 4, 6, 10, 12, 16 result in considerable impact on XR traffic.

Proposal 1: RAN4 to perform the selection of the applicable Type-2 gaps to be included in Rel-19 considering the impact on XR traffic when Gap pattern IDs 0, 4, 6, 10, 12, 16 are in the possible configuration.

# XR skipping requirements impact

RAN4 agreed to follow the NR-U model for measurement delay calculation.

<Agreement>: - Follow NR-U-like approach for designing the formulas for measurement delay extension.

The follow up was to have some equations for the delay calculation and three options have been retained:

<Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter

In our opinion, without having a strong position, Option 1 shows a more intuitive and direct way of computing the delay as it adds up the skipped measurements.

Observation 2: Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements.

Proposal 2: Support Option 1:

  - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

      - where Nskip is the number of skipped MG occasions in the measurement period.

      - Use this formula as example and follow similar approach for the other cases.

Another issue that needs to be resolved is about the DRX cases for measurement delay extension:

<Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter

We don’t have a strong opinion, but it seems that Option 1 provides a more intuitive reading for the requirement and its purpose.

Observation 3: Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements.

Proposal 3: Support Option 1:

  - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - DRX cycle ≤ 320 ms

      - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

    - DRX cycle > 320 ms

      - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

# additional signaling aspects

In the last meeting, in addition to the dynamic schemes, semi-static schemes were brought back into discussion even though RAN1 already discussed and excluded them one by one.

Here are the options on semi-static schemes retained in the WF:

<Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

We are reiterating our support for the DCI based dynamic scheme and suggest not considering semi-static schemes in this WI.

Proposal 4: Support Option 1

  - Option 1: Do not consider semi-static solutions.

# Skipping indication on frequency domain

In the last meeting, the following proposals were retained for further discussion:

<Way forward>: - Proposals - Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

After seeing the draft CR regarding the DCI gap skipping indication introduction from RAN1(see the Annex of this document for reference), we believe that Proposal 3 is in line with RAN1 understanding.

Proposal 5: Support Proposal 3 from the above list:

Proposal 3: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# Conclusions

This contribution discussed our analysis related to measurements occasions skipping for NR XR case. Based on the discussion, the following observations and proposals are made:

Observation 1: Gap pattern IDs 0, 4, 6, 10, 12, 16 result in considerable impact on XR traffic.

Proposal 1: RAN4 to perform the selection of the applicable Type-2 gaps to be included in Rel-19 considering the impact on XR traffic when Gap pattern IDs 0, 4, 6, 10, 12, 16 are in the possible configuration.

Observation 2: Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements.

Proposal 2: Support Option 1:

  - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

      - where Nskip is the number of skipped MG occasions in the measurement period.

      - Use this formula as example and follow similar approach for the other cases.

Observation 3: Option 1 offers a more intuitive and direct way of computing the delay as it adds up the skipped measurements.

Proposal 3: Support Option 1:

  - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - DRX cycle ≤ 320 ms

      - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

    - DRX cycle > 320 ms

      - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

Proposal 4: Support Option 1

  - Option 1: Do not consider semi-static solutions.

Proposal 5: Support Proposal 3 from the above list:

  - Proposal 3: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# References

- R4-2504911 “WF on RRM requirements for XR_Ph3, RAN4 Meeting #114bis, Wuhan, China, April 2025

- TS 38.133, Requirements for support of radio resource management, Rel-18

- TR 38.838, Study on XR (Extended Reality) evaluations for NR, v17.0.0, Apr 2022

# Annex

38.213 CR: Introduction of XR (eXtended Reality) for NR Phase 3

## *** Unchanged parts are omitted ***

## 10.6	Indication for cancelation of RRM measurement gaps

A UE can be indicated by a value ‘1’ of an XYZ field in a DCI format [5, TS 38.212] provided by a PDCCH reception to cancel an earliest RRM measurement gap occasion where the UE would perform RRM measurements on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]. The earliest RRM measurement gap occasion is at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception.

If a UE indicates “Value 1” for FG-ABC, the UE ignores a value ‘0’ of the XYZ field in a DCI format.

If a UE indicates “Value 2” for FG-ABC, a value ‘0’ of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [10, TS 38.133]. When the UE detects more than one DCI formats that include the XYZ field and are associated with a same earliest RRM measurement gap occasion where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the more than one DCI formats, the UE applies the indication provided by the XYZ field of a last DCI format, from the more than one DCI formats, that is in a PDCCH reception that ends at least 3 msec or 5 msec, as determined by a reported UE capability, before the start of that earliest RRM measurement gap occasion.

## *** Unchanged parts are omitted ***