3GPP TSG-RAN WG4 Meeting #115												R4-2505395

Malta, 19 – 23 May, 2025

Agenda item:			7.26.5

Source:	Huawei, HiSilicon

Title:	on Ambient IoT device OTA tests

Document for:	Approval

# Introduction

The WF [1] from RAN4 #114bis lists the following issues and this further discusses them.

## Topic 4-2: Test setup

Issue 4-2-1: testing antenna and CW antenna relationship

Agreement:

Whole sphere scan is supported

The position of CW antenna is a set of fixed position(s) as baseline (depending on core requirements discussion).

- FFS on relative positions between reader antenna and CW

Issue 4-2-2: how to ensure the isolation between CW node and measurement antenna

Agreement:

- Further discuss power difference between backscatter signal and the CW signal leakage, e.g. 6dB, 12dB, etc.

- Encourage TE vendors provide the feasibility of TE extracting D2R power when CW source is transmitting.

# Discussion

Regarding issue 4-2-1, the relative position between reader antenna and CW antenna is not critical as long as sufficient isolation can be achieved between them. It would be advantageous for reader and CW antennas on the same plane for the same supporting structure such as a boom to be shared between them in the cases where DUT can only be rotated in azimuth. Otherwise, two supporting structures would be needed. However, keeping reader and CW antennas on the same plane does not offer any advantage in the measurement setups where DUT can be rotated in both elevation and azimuth.

Regarding how to determine the position of CW antenna, following aspects should also be considered. Based on the discussion in last meeting, the criteria for a success test rely on correct response for both sensitivity and backscatter testing. The coupling between CW2D and D2R should be avoided. Otherwise, if a device fails a test as no correct response is received, it is not clear whether the failure is because the R2D message is not decoded or the CW is not backscattered properly. One example is that, the reader is pointing to the maximum gain direction and CW is pointing to the minimum gain direction (considering the radiation pattern of dipole antenna). Thus, following two options can be considered:

Option 1: CW antenna is fixed relative to reader with relatively small angular offset.

Option 2: CW antenna is fixed relative to DUT (e.g. always pointing to the maximum gain direction of DUT).

For Option 1, it is much easier for TE implementations. Taking RFID test spec [2] as reference, the angular offset is defined as 25 degree with 50dB isolation, which is shown as follows:

For option 2, based on current test OTA test system, the CW nodes need to be rotated together with DUT.

Based on analysis above, whether the CW position is fixed relative to DUT or Reader depends on the following scenarios:

- Scenario 1: If backscatter power is averaged over a given solid angle, the CW position should then be fixed relative to DUT. Otherwise the performance metric may not achieve the intended quality because the side condition for a metric averaged over a solid angle is to have a fixed CW position relative to DUT as in field deployment.

- Scenario 2: If backscatter power is measured at a given position or direction. The question is irrelevant for a single point measurement provided sufficient CW power reaches DUT.

- Scenario 3: sensitivity measurement. The CW position is less critical as long as DUT receives sufficient power from CW.

Above scenarios depend on the performance metric to be defined in RF core requirements.

Observation 1: The CW position should be defined according to the scenarios below.

  - Scenario 1: backscatter power is averaged over a given solid angle. The CW position should then be fixed relative to DUT.

  - Scenario 2: backscatter power is measured at a given position or direction. A single CW position is defined to provide sufficient power to DUT.

  - Scenario 3: sensitivity measurement. A single CW position is defined to provide sufficient power to DUT, for example, [25°] relative to the reader.

Proposal 1: Consider following CW positions:

Option 1: CW antenna is fixed relative to reader with 25 degree offset

Option 2: CW antenna is fixed relative to DUT (e.g. always pointing to the maximum gain direction of DUT).

Another remaining issue is how to ensure the isolation between CW and measurement antenna. The agreements are shown as follows.

Issue 4-2-2: how to ensure the isolation between CW node and measurement antenna Agreement: - Further discuss power difference between backscatter signal and the CW signal leakage, e.g. 6dB, 12dB, etc. - Encourage TE vendors provide the feasibility of TE extracting D2R power when CW source is transmitting.

As discussed during the offline discussion, we would like to firstly elaborate the definition of each terms to align the understanding. As illustrated in the following figure, in the anechoic chamber, we can assume there are two links from CW to measurement antenna: one is the path from CW-device-measurement antenna (Path1), and the other one is the direct path from CW to measurement antenna (Path2). The purpose of having the isolation conditions is to minimize the receive power from Path2, which is so-called CW leakage.

It should be noted that the CW isolation should be distinguished from the CW interference cancelation/suppression, which is discussed during SI phase. As captured in TR, three CW cancellation methods are studied: spatial isolation, analog cancellation and digital cancellation at baseband. Based on following test up, even the power leakage from CW to measurement antenna (path2) is perfectly isolated, which means there is only path1 from CW to measurement antenna, the received backscatter CW is always higher than D2R signal at sideband. Thus, the above CW interference cancelation/suppression is needed to decode the D2R signal.

Figure 1. Isolation between CW and measurement antenna.

Observation 2: The purpose of having the isolation conditions is to minimize the receive power from Path2 (i.e. the direct path from CW to measurement antenna), which is the so-called CW leakage.

Observation 3: The isolation condition shall be distinguished from the CW interference cancelation/suppression.

Based on analysis above, the isolation conditions are determined by following two factors:

- Pathloss between CW and measurement antenna

- Absorbing material

Observation 4: The isolation condition between CW and measurement antenna is determined by following two factors:

- Pathloss between CW and measurement antenna

- Absorbing material

Based on analysis above, we provide the calculation on how to derive the reasonable isolation value.

The received backscatter power is calculated as:

Rpath1= CW Tx power + CW_Tx_gain_1 – Pathloss1*2 – backscatter loss+ Rx gain – 2*polarization mismatch 	(1)

The CW leakage is calculated as:

Rpath2= CW Tx power + CW_Tx_gain_2– Pathloss2 - Iso_ab + Rx gain – polarization mismatch 					(2)

Where Iso_ab is the isolation introduced by absorbing material.

Based on the agreement minimum range in last meeting, pathloss1 is equal to 30.6dB. Assuming that the offset between CW and measurement antenna is 25 degree, the length between CW and measurement antenna is 39cm, and the pathloss is 23.36 dB. The values of CW_Tx_Gain_1 and CW_Tx_Gain_2 differ by a few dBs due to different directions in antenna patterns in path1 and path2 and can be absorbed in X in equation (3)

The difference between backscatter signal and the CW signal leakage is derived:

Rpath1 - Rpath2 > X dB                        (3)

Based on (1) (2) (3)

We can derive

Pathloss2 + Iso_ab> X + 2*Pathloss1 + backscatter loss - polarization mismatch

Based on the value used in TR 38.876 for link budget evaluation. We assume that backscatter loss is 6dB and polarization mismatch is 3dB.

Then, the CW isolation should be larger than 64.2 dB + X dB.

Regarding the value of X. If X is equal to 0, it means the power of CW leakage is same as backscattered R2D signal power. As analysed above, even the CW is perfectly isolated (not perfectly cancelled), the received CW power is always higher than that of backscattered R2D signal as backscattered R2D signal is at sideband. It means that Reader anyway needs some mechanism to extract the sideband signal. Based on the reported value during SI, the CW cancellation could be up to more than 100 dB. We don’t need to require the TE to be as powerful as the reader, but several tens of dB is feasible to decode the R2D signal.

Observation 5: The CW leakage does not need to be higher than backscattered R2D signal power, since even the CW is perfectly isolated (not perfectly cancelled), the received CW power is always higher than that of backscattered R2D signal as backscattered R2D signal is at sideband.

Based on above analysis, the value of 50-60 dB is reasonable for CW isolation conditions. And the CW interference cancellation is up to TE implementation.

Proposal 2: The CW isolation condition is defined as 50-60 dB, and CW interference cancellation is up to TE implementation.

Regarding the test procedure, RAN4 agreed on the baseline test procedures for receiver and backscatter requirements. The details need some updating. The proposed TP for test procedures for receiver requirements and backscatter requirements are updated as follows:

TP for test procedures for receiver requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal.

  - The transmit power of R2D signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device.

  - The CW signal level is set such that the power at DUT’s antenna is R2D signal power + X dB, and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response

- Step 5: Determine whether DUT can send the correct response in D2R channel with correct timing relationship.

- Step 6: Move to next measurement point and repeat Step 4-5 until complete all the testing point in the measurement grid.

Note 1: The test result for each measurement grid depends on performance metric defined in core requirements. (e.g. PASS/Not or sensitivity value for each grid).

Note 2: When correct response is not detected for one measure grid, whether to increase the transmit power of signal generator to repeat the test for the same grid depends on performance metric defined in core requirements.

Note 3: X could be different value for different requirements (e.g. negative value for maximum input power).

The reason of proposed changes are as follows:

- For sensitivity, to better evaluate the Rx performance, the situation where D2R link becomes the bottleneck should be avoided, thus it is proposed that the CW power at DUT as the transmit power of signal generator + X dB to ensure D2R link is not the bottleneck. It should be noted that X may be different for different requirements which depends on core requirements discussion. And it should clarify that CW signal is available during the period that A-IoT device is expected to response.

- For each measurement grid, it should start from step 4 since energy charge may not be a per grid action.

- Some part in the test procedure are still pending on core requirements conclusion. For instance, if it is a spherical coverage like requirements, the result of each gird is PASS/Not and there is no need to increase the power and repeat the test for the same grid. If it is a TRS-like requirements, then the exact sensitivity value needs to be recorded.

TP for test procedures for backscatter requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal.

  - The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB”

  - The CW signal level is set as TBD at DUT’s antenna

- Step 5: Determine whether DUT can send the correct response in D2R channel with correct timing relationship. If the correct response is received, measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 4-5 until complete all the testing point in the measurement grid.

Note 1: The performance metric depends on core requirements discussion.

Note 2: Candidate values of X can be 10 dB which depends on core requirements discussion (e.g. CW conditions)

The reason of proposed changes are as follows:

- For the similar reason, for backscattering requirements, the R2D link should not be the bottleneck. Thus sensitivity+[10]dB seems reasonable.

- For each measurement grid point, it should start from step 4 since energy charge may not be a per grid action.

- The backscattering requirements shall also be based on the backscatter with correct response and timing relationship.

- The performance metric of backscatter requirements and also CW power level depends on core requirements discussion.

Proposal 3: The test procedure for sensitivity requirements and backscattering requirements are updated as follows:

TP for test procedures for receiver requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal.

  - The transmit power of R2D signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device.

  - The CW signal level is set such that the power at DUT’s antenna is R2D signal power + X dB, and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response

- Step 5: Determine whether DUT can send the correct response in D2R channel with correct timing relationship.

- Step 6: Move to next measurement point and repeat Step 4-5 until complete all the testing point in the measurement grid.

Note 1: The test result for each measurement grid depends on performance metric defined in core requirements. (e.g. PASS/Not or sensitivity value for each grid).

Note 2: When correct response is not detected for one measure grid, whether to increase the transmit power of signal generator to repeat the test for the same grid depends on performance metric defined in core requirements.

Note 3: X could be different value for different requirements (e.g. negative value for maximum input power).

TP for test procedures for backscatter requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal.

  - The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB”

  - The CW signal level is set as TBD at DUT’s antenna

- Step 5: Determine whether DUT can send the correct response in D2R channel with correct timing relationship. If the correct response is received, measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 4-5 until complete all the testing point in the measurement grid.

Note 1: The performance metric depends on core requirements discussion.

Note 2: Candidate values of X can be 10 dB which depends on core requirements discussion (e.g. CW conditions)

It should be noted that sensitivity and backscatter power are used for test methodology discussion for receiver band backscattering. There are also other requirements under discussion in core requirements discussion in RF, RRM. For other requirements, test methodology is common (e.g. test setup, configurations) for other requirements. The same OTA test methodology can be used for other requirements with necessary updates and simplifications. For instance, for some requirements, whole sphere scanning may not be needed, and certain point selected from the grids fulfilling sensitivity requirements is sufficient. The detailed updating for specific requirements can be discussed in conformance part in RAN5.

Observation 6: The OTA methodology is common for other requirements.

Proposal 4: The OTA tests for other requirements can be conducted based on the baseline methodology for sensitivity and backscattering. The detailed updating/simplification for specific requirements can be discussed in conformance requirements.

The Measurement Uncertainty (MU) table agreed in the WF of the last meeting [1] represents in fact two tables, one for backscatter power, the other sensitivity as shown below.

Table 1 MU for backscatter power

| UID | Description of uncertainty contribution for backscatter power | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of transmitter chain | Reused |
| 2 | Insertion loss of transmitter chain | Reused |
| 3 | Influence of the measurement antenna cable | Reused |
| 4 | CW for backscatter measurement uncertainty of the absolute output level | -0.65 |
| 5 | Sensitivity measurement: output level step resolution | Reused |
| 6 | Measurement distance | Reused |
| 7 | Quality of quiet zone | Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length. |
| 9 | Uncertainty related to the use of phantoms | No needs |
| 10 | Coarse sampling grid | - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point |
| 11 | Random uncertainty | Reused |
| 12 | Frequency Response | Reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 13 | Uncertainty of network analyzer | Reused |
| 14 | Mismatch of transmitter chain | Reused |
| 15 | Insertion loss of transmitter chain | Reused |
| 16 | Mismatch in the connection of calibration antenna | Reused |
| 17 | Influence of the calibration antenna feed cable | Reused |
| 18 | Influence of the measurement antenna cable | Reused |
| 19 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | Reused |
| 20 | Measurement distance | Reused |
| 21 | Quality of quiet zone | Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length. |
| Systematic Errors |  |  |
| 22 | Systematic Error related to TRS grids | Reused |

In Table 1, a value of 0.65dB is entered for UID 4 with its associated description modified. The value of 0.65dB comes from UID 4 in TRS MU Table B.5.1-2 in TR 38.870. UID 8 is removed as CW power reaching DUT should be well above DUT sensitivity.

Table 2 MU for sensitivity

| UID | Description of uncertainty contribution for sensitivity | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of transmitter chain | Reused |
| 2 | Insertion loss of transmitter chain | Reused |
| 3 | Influence of the measurement antenna cable | Reused |
| 4 | R2D for sensitivity measurement: uncertainty of the absolute output level | 0.27 |
| 5 | Sensitivity measurement: output level step resolution | Reused |
| 6 | Measurement distance | Reused |
| 7 | Quality of quiet zone | Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length. |
| 8 | DUT sensitivity drift | 0.12 |
| 9 | Uncertainty related to the use of phantoms | No needs |
| 10 | Coarse sampling grid | - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point |
| 11 | Random uncertainty | Reused |
| 12 | Frequency Response | Reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 13 | Uncertainty of network analyzer | Reused |
| 14 | Mismatch of transmitter chain | Reused |
| 15 | Insertion loss of transmitter chain | Reused |
| 16 | Mismatch in the connection of calibration antenna | Reused |
| 17 | Influence of the calibration antenna feed cable | Reused |
| 18 | Influence of the measurement antenna cable | Reused |
| 19 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | Reused |
| 20 | Measurement distance | Reused |
| 21 | Quality of quiet zone | Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length. |
| Systematic Errors |  |  |
| 22 | Systematic Error related to TRS grids | Reused |

In Table 2, a value of 0.27dB is entered for UID 4 with the corresponding changes in its description. The value of 0.27dB comes from UID 4 in TRP MU Table B.4.2-2 in TR 38.870. A value of 0.12dB is entered for UID 8. The value of 0.12dB comes from entry 8 in TRS MU Table B.5.1-2 in TR 38.870.

Proposal 5: adopt the MU Table 1 and Table 2 in R4-2505395 as baseline.

# Conclusion

This controbution makes the following observations and proposals.

Observation 1: The CW position should be defined according to the scenarios below.

  - Scenario 1: backscatter power is averaged over a given solid angle. The CW position should then be fixed relative to DUT.

  - Scenario 2: backscatter power is measured at a given position or direction. A single CW position is defined to provide sufficient power to DUT.

  - Scenario 3: sensitivity measurement. A single CW position is defined to provide sufficient power to DUT, for example, [25°] relative to the reader.

Observation 2: The purpose of having the isolation conditions is to minimize the receive power from Path2 (i.e. the direct path from CW to measurement antenna), which is the so-called CW leakage.

Observation 3: The isolation condition shall be distinguished from the CW interference cancelation/suppression.

Observation 4: The isolation condition between CW and measurement antenna is determined by following two factors:

- Pathloss between CW and measurement antenna

- Absorbing material

Observation 5: The CW leakage does not need to be higher than backscattered R2D signal power, since even the CW is perfectly isolated (not perfectly cancelled), the received CW power is always higher than that of backscattered R2D signal as backscattered R2D signal is at sideband.

Observation 6: The OTA methodology is common for other requirements.

Proposal 1: Consider following CW positions:

Option 1: CW antenna is fixed relative to reader with 25 degree offset

Option 2: CW antenna is fixed relative to DUT (e.g. always pointing to the maximum gain direction of DUT).

Proposal 2: The CW isolation condition is defined as 50-60 dB, and CW interference cancellation is up to TE implementation.

Proposal 3: The test procedure for sensitivity requirements and backscattering requirements are updated as follows:

TP for test procedures for receiver requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal.

  - The transmit power of R2D signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device.

  - The CW signal level is set such that the power at DUT’s antenna is R2D signal power + X dB, and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response

- Step 5: Determine whether DUT can send the correct response in D2R channel with correct timing relationship.

- Step 6: Move to next measurement point and repeat Step 4-5 until complete all the testing point in the measurement grid.

Note 1: The test result for each measurement grid depends on performance metric defined in core requirements. (e.g. PASS/Not or sensitivity value for each grid).

Note 2: When correct response is not detected for one measure grid, whether to increase the transmit power of signal generator to repeat the test for the same grid depends on performance metric defined in core requirements.

Note 3: X could be different value for different requirements (e.g. negative value for maximum input power).

TP for test procedures for backscatter requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal.

  - The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB”

  - The CW signal level is set as TBD at DUT’s antenna

- Step 5: Determine whether DUT can send the correct response in D2R channel with correct timing relationship. If the correct response is received, measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 4-5 until complete all the testing point in the measurement grid.

Note 1: The performance metric depends on core requirements discussion.

Note 2: Candidate values of X can be 10 dB which depends on core requirements discussion (e.g. CW conditions)

Proposal 4: The OTA tests for other requirements can be conducted based on the baseline methodology for sensitivity and backscattering. The detailed updating/simplification for specific requirements can be discussed in conformance requirements.

Proposal 5: adopt the MU Table 1 and Table 2 in R4-2505395 as baseline.

# Reference

- R4-2505230 WF on A-IOT device requirements

- Tag Performance Parameters and Test Methods Version 1.1.3