3GPP TSG-RAN WG4 Meeting #115												R4-2505396

Malta, 19 – 23 May, 2025

Agenda item:			7.14.3.3

Source:	Huawei, HiSilicon

Title:	on MIMO OTA tests

Document for:	approval

# Introduction

This contribution continues the discussion on options in combining measurement data from UMa and UMi channels [1].

# Discussion

Measurement data from UMa and UMi channels should be used to form a single performance metric to avoid having two metrics, viz. one for UMa channel and the other UMi channel. Under two metrics, a device could rank high under one metric, but perform poorly in the other metric. Under such contradicting cases, the metrics do not provide any guidance on the performance of devices and could become irrelevant. Granted that a single performance metric could have a narrow range depending on measurement data for UMa and UMi channels. However a single consistent performance metric is still better than two conflicting ones.

Proposal 1: MIMO OTA performance should have one metric to avoid the situation where a device could rank high under one metric for UMa channel, but perform poorly under the other for UMi channel.

There are a few ways to combine the measurement data from UMa and UMi channels and mainly they come into the three categories listed below.

Option 1: Put measurement data from UMa and UMi channels into a single data pool.

Option 2: Calculate metrics for UMa and UMi separately for a given percentile on throughput CDF, then combine the two metrics.

Option 3: put measured throughput against reported 15 CQI values, with pass and fail criterion as [3] allowed failures to meet expected throughputs and non-reported CQI excluded from the 15 CQI values.

Under option 1, the associated data processing is straightforward. The remaining detail could be the resolution to create CDF curves and the performance threshold.

Under option 2, the performance metrics for UMa and UMi channels can be calculated separately for a given percentile on CDF and combined as follows. This can be done on a per segment basis or on the sum of all segments for a given percentile on throughput CDF. In this case, the performance metrics for UMa and UMi channels are available internally and can be made available if necessary. However, the final pass and fail metric is a combined one.

Under option 3, the 15 CQI values correspond a range of SNR and can be used to represent various percentile values in UE throughput CDF. CQI is common for all UE and would avoid differences in a specific percentile value in throughput CDF for different devices. For example, value at x%-ile throughput CDF for one device would not be the same for another device. However, for the same CQI value (or effectively SNR value), devices can be compared directly. How SNR maps to CQI is up to UE implementation. Devices reporting CQI conservatively may be sent a smaller transport block, but could benefit from fewer HARQ retransmission. While devices reporting CQI aggressively may be sent a larger transport block, but could suffer from additional HARQ retransmissions. The same applies to rank reporting.

The performance metric could be 15 expected throughput values on a per CQI basis. Typically rank 2 is reported when CQI is better than 7 and should be taken into account when calculating expected throughput per CQI. The pass and fail criterion could be 12 out 15 CQI values should meet or exceed the expected throughput. Obviously non reported CQI should be excluded.

Proposal 2: down select one metric from the following options

Option 1: use a resolution of [0.2] dB to create CDF curves for the combined data pool from UMa and UMi channels and use [50%-ile] for performance threshold.

Option 2: calculate metrics for UMa and UMi separately for a given percentile on throughput CDF, then combine the two metrics.

Option 3: put measured throughput against reported 15 CQI values, with pass and fail criterion as [3] allowed failures to meet expected throughputs and non-reported CQI excluded from the 15 CQI values.

# Conclusion

This contribution makes the following proposals.

Proposal 1: MIMO OTA performance should have one metric to avoid the situation where a device could rank high under one metric for UMa channel, but perform poorly under the other for UMi channel.

Proposal 2: down select one metric from the following options

Option 1: use a resolution of [0.2] dB to create CDF curves for the combined data pool from UMa and UMi channels and use [50%-ile] for performance threshold.

Option 2: calculate metrics for UMa and UMi separately for a given percentile on throughput CDF, then combine the two metrics.

Option 3: put measured throughput against reported 15 CQI values, with pass and fail criterion as [3] allowed failures to meet expected throughputs and non-reported CQI excluded from the 15 CQI values.

# Reference

- R4-2503495 on MIMO OTA tests