3GPP RAN WG4 Meeting #115																	R4-2505431

Malta, Malta, 19th – 23rd May 2025

Agenda item:	4.2.1

Source:	Apple

Title:	(NR_newRAT-Core) On NS_05 reevaluation

WI/SI:	NR_newRAT-Core

Release:	Rel-15

Document for:	Approval

# 1	Introduction

The WF [1] and [2] capture the state latest agreements on NS_05 and NS_05U for PC3. This contribution provides further tweaks for PC3 A-MPR region implementation and proposes A-MPR for PC2. Finally, suggestions on the RB region split proposal for 20MHz are made.

# 2  Discussion

##### 2.1 Simulation assumptions

The following assumptions and requirements for the simulations are applied:

- Power Class 3 & 2

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

Emission requirements:

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

The carrier arrangements from WF [1] have been analyzed:

##### 2.2 NS_05 simulation results for PC3

Further adjustments to the RB regions could be made for 20MHz CBW. The A1 RB region of the starting point cuts into the region A2 which means that certain RB allocations get unnecessary high A-MPR allowance. A small fix can solve this issue:

| Starting Point | Adjusted RB region |
| --- | --- |

The fix can be implemented as shown below. As a small bonus the region definition even looks a bit simpler.

Table 1: Alternative region definition for 20MHz CBW

| NS_05 | Fc MHz | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | Rbstart |
| 20M | < 1945.7 | >= Rbstart*1.35 + 3.42MHz/12/scs; > 5.76MHz/12/scs; | >=0 | <= 5.76MHz/12/scs; | <= 5.04MHz/12/scs |

Finally, as done in our last contribution we would like to propose to use the format as shown in Table 2 which is more often utilised nowadays.

Table 2: A-MPR for Rel-19 NS_05 (Power Class 3)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | < 1930.7 | ≥ 0 | ≥ 1.35* RBstart*12*SCS + 4.32 | A1 |
|  |  | ≤ 1.8 | ≤ 2.16 | A2 |
| 15 MHz | 1927.5≤fc＜1932.5 | ≥ 0 | ≥ RBstart*12*SCS *1.20 + 4.68 | A3 |
|  |  | ≤ 3.6 | ≤ 3.96 | A4 |
|  | 1932.5≤fc<1938.2 | ≤ 0.54 | ≥ 8.28 | A5 |
|  |  | ≤ 2.16 | ≤ 2.16 | A6 |
| 20MHz | < 1945.7 | ≥ 0 | ≥ RBstart*12*SCS *1.5 + 4.14 > 5.76 | A7 |
|  |  | ≤ 4.68 | ≤ 5.76 | A8 |

Proposal 1: Use the alternative region definition for A2 as it optimises A-MPR.

Proposal 2: Use the more “modern” table layout when writing the CR as proposed in Table 2.

##### 2.3 NS_05 simulation results for PC2

Table 2: A-MPR for Rel-19 NS_05 (PC2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5MHz | < 1925 | ≤ 0.18 | ≥ 3.24 | A6 |
| 10 MHz | < 1930 | ≥ 0 | ≥ 1.35* RBstart*12*SCS + 4.32 | A1 |
|  |  | ≥ 0 | < 1.35* RBstart*12*SCS + 4.32 ≥ 1.35* RBstart*12*SCS + 2.52 | A2 |
|  |  | ≤ 1.8 | ≤ 2.16 | A2 |
|  | 1930≤fc<1935 | ≥ 0 | ≥ 7.92 | A2 |
| 15 MHz | 1927.5≤fc<1932.5 | ≥ 0 | ≥ RBstart*12*SCS *1.20 + 4.68 | A3 |
|  |  | ≥ 0 | < RBstart*12*SCS *1.20 + 4.68 ≥ RBstart*12*SCS *1.20 + 2.16 >3.96 | A2 |
|  |  | ≤ 3.6 | ≤ 3.96 | A4 |
|  | 1932.5≤fc<1938.2 | ≤ 0.54 | ≥ 8.28 | A5 |
|  |  | >0.54, ≤ 1.8 | ≥ 10.44 | A2 |
|  |  | ≤ 2.16 | ≤ 2.16 | A6 |
| 20MHz | < 1945.7 | ≥ 0 | ≥ RBstart*12*SCS *1.5 + 4.14 > 5.76 | A7 |
|  |  | ≥ 0 | < RBstart*12*SCS *1.5 + 4.14 ≥ RBstart*12*SCS *1.5 + 1.08 > 5.76 | A2 |
|  |  | ≤ 4.68 | ≤ 5.76 | A8 |

In the following the simulation results for DFT-s-OFDM and CP-OFDM with QPSK modulation are provided. The results cover 5MHz, 10MHz, 15MHz and 20MHz channels at various positions.

Table 3: A-MPR for Rel-19 NS_05 (PC2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3+1.5 | ≤ 2+1.5 | ≤ 3+1.5 | ≤ 2.5+1.5 | ≤ 2 +1.5 | ≤ 2 +1.5 | ≤ 3.5 +1.5 | ≤ 3+1.5 |
|  | QPSK | ≤ 4+1.5 | ≤ 3+1.5 | ≤ 4+1.5 | ≤ 3+1.5 | ≤ 3 +1.5 | ≤ 2.5 +1.5 | ≤ 4+1.5 | ≤ 3.5 +1.5 |
|  | 16 QAM | ≤ 4+1.5 | ≤ 3+1.5 | ≤ 4.5+1.5 | ≤ 3.5+1.5 | ≤ 3 +1.5 | ≤ 3 +1.5 | ≤ 5 +1.0 | ≤ 4 + 1.5 |
|  | 64 QAM | ≤ 4.5+1.5 | ≤ 3.5+1.5 | ≤ 4.5+1.5 | ≤ 4+1.0 | ≤ 3 +1.5 | ≤ 3 +1.5 | ≤ 5 +1.0 | ≤ 4 +1.5 |
|  | 256 QAM | ≤ 5+1.0 | ≤ 4.5+0.5 | ≤ 5 +1.0 | ≤ 4.5+0.5 | ≤ 4.5+0.5 | ≤ 4.5 | ≤ 5 +1.0 | ≤ 4.5 +1.0 |
| CP-OFDM | QPSK | ≤ 5+1.5 | ≤ 4.5+1.5 | ≤ 6+1.5 | ≤ 5+1.5 | ≤ 4.5 +1.5 | ≤ 4.5 +1.5 | ≤ 6+1.5 | ≤ 5.5+1.5 |
|  | 16 QAM | ≤ 5+1.5 | ≤ 4.5+1.5 | ≤ 6+1.5 | ≤ 5 +1.5 | ≤ 4.5 +1.5 | ≤ 4.5 +1.5 | ≤ 6+1.5 | ≤ 5.5 +1.5 |
|  | 64 QAM | ≤ 5+1.5 | ≤ 4.5+1.5 | ≤ 6+1.5 | ≤ 5 +1.5 | ≤ 4.5 +1.5 | ≤ 4.5 +1.5 | ≤ 6+1.5 | ≤ 5.5 +1.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5+1.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 7+0.5 | ≤ 6.5 +0.5 |

Table 3: RB region tables for NS_05 PC2

PC2 RB regions require modifications due to the higher PC2 output power compared to PC3. As PC2 seems to be second priority it should be considered to focus the discussion on PC3 and continue PC2 in the next meeting.

Proposal 3: PC2 RB regions require modifications due to the higher PC2 output power compared to PC3. As PC2 seems to be second priority it should be considered to focus the discussion on PC3 and continue PC2 in the next meeting.

Proposal 4: With the high similarity of A5 and A6 it is proposed to discuss removing A6 and replacing it with A5. This should be considered for PC3 and PC2.

##### 2.4 NS_05 simulation results for PC2

## Sub-topic 2-1: PUCCH transmissions for meeting limits without A-MPR < Issue 2-1-1: PUCCH transmissions for meeting limits without A-MPR > Agreement: - Encourage companies to evaluate the feasibility of the following: - Split the agreed Region A2 into two new regions

The paper [3] proposes to split the region A2 to optimise A-MPR for PUCCH. The table below is taken from [3]. The region is split and there is also a special handling for DFT-s-OFDM QPSK:

To better understand and analyse the proposal the RB allocation regions are presented for Note 3 and Note 4 in the table below. Furthermore, our simulation results for DFT-s-OFDM QPSK are provided for comparison:

| Note 3 from [3] | Note 4 from [3] | Simulation result (regular Tx impairment assumptions) |
| --- | --- | --- |

The proposed Notes 3 and 4 are based on alternative Tx impairment assumptions which feature tighter LO and Image requirements. Even for an isolated use case this will impact UE design and is therefore not preferred at this point in time. The proposed split of the region is a good approach as it allows to optimise A-MPR without changing fundamental UE performance assumptions.

It is proposed that the RB region ranging between 0 ≤ RBstart*12*SCS ≤ 1.8MHz uses A2. The A-MPR A2 is required since the IMD from wanted+image falls into the -30dBm/MHz limit and there is no room for relaxation. The RB region ranging between 1.8MHz < RBstart*12*SCS ≤ 4.68MHz can receive A-MPR improvements as the IMD only falls into the -25dBm/MHz region. The A-MPR value of 2.5dB is proposed.

Observation: The first RB allocation region ranging between 0 ≤ RBstart*12*SCS ≤ 1.8MHz the IMD from wanted+image falls into the -30dBm/MHz limit. The second RB region ranging between 1.8MHz < RBstart*12*SCS ≤ 4.68MHz the IMD only falls into the -25dBm/MHz region.

Proposal 5: The first RB allocation region requires the A-MPR allowance of A2. The second RB allocation region should use 2.5dB for DFT-s-OFDM and 4.5dB for CP-OFDM.

# 4	Conclusions

This contribution provides simulation results and proposals for NS_05. The following proposals are made:

Proposal 1: Use the alternative region definition for A2 as it optimises A-MPR.

Proposal 2: Use the more “modern” table layout when writing the CR as proposed in Table 2.

Proposal 3: PC2 RB regions require modifications due to the higher PC2 output power compared to PC3. As PC2 seems to be second priority it should be considered to focus the discussion on PC3 and continue PC2 in the next meeting.

Proposal 4: With the high similarity of A5 and A6 it is proposed to discuss removing A6 and replacing it with A5. This should be considered for PC3 and PC2.

Observation: The first RB allocation region ranging between 0 ≤ RBstart*12*SCS ≤ 1.8MHz the IMD from wanted+image falls into the -30dBm/MHz limit. The second RB region ranging between 1.8MHz < RBstart*12*SCS ≤ 4.68MHz the IMD only falls into the -25dBm/MHz region.

Proposal 5: The first RB allocation region requires the A-MPR allowance of A2. The second RB allocation region should use 2.5dB for DFT-s-OFDM and 4.5dB for CP-OFDM.

# 4	References

- R4-2502980, “WF on UE requirements for PHS protection”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #114

- R4-2505222, “WF on PHS protection update”, KDDI, 3GPP TSG-RAN WG4 Meeting #114bis

- R4-2504569, “A-MPR regions for NS_05/NS_05U and optionality in earlier releases by modified MPR behaviour”, TSG-RAN Working Group 4 (Radio) meeting #114bis