3GPP TSG-RAN WG4 Meeting # 115		R4-2505437

Malta, MT, May 19 – 23, 2025

Agenda item:	7.1.2.3.4

Source:	Apple

Title:	On release independence for 6Rx RF requirements

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

Release independence for 6Rx requirements is one of the remaining topics to be addressed before the 6Rx WI is completed. During the previous meeting, an agreement was made to further discuss the topic in the upcoming meetings as captured in the WF [1]. Below is the topic summary taken from that WF.

# Topic #1: General aspects ### Sub-topic 1-1: General aspects of 6Rx requirements Issue 1-1-1: Release independence of 6Rx RF requirements Agreement: FFS whether to define 6Rx RF requirements in a release independent manner - of any solutions should be based on system level throughput evaluations, and implementation complexity

In this discussion paper, we will share our views on the release independence of the 6Rx requirements which are part of the Rel-19 NR_ENDC_RF_Ph4 WID [2].

# Discussion

The two main RF requirements for the 6Rx WI are the reference sensitivity (ΔRIB,6R ) and the values ΔTRxSRS on each SRS antenna branch.

- REFSENS for 6Rx

RAN4 has already agreed on the REFSENS requirements for 6Rx and a CR introducing these requirements in the TS38.101-1 was agreed in RAN4 meeting # 112Bis [3]. A summary of the values are shown in Table 2-1 below.

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.41 -3.72 |
| n77, n78, n79, n104 | -3.01 -3.32 |
| NOTE 1: When 6 Rx operation is supported by handheld UE. NOTE 2: When 6 Rx operation is supported by FWA form factor. |  |

Table 2-1: REFSENS requirements for 6Rx [3]

As can be observed from the above table, the values of REFSENS for handled UEs are different from the values for

for FWA UEs

Observation #1: For 6Rx REFSENS, RAN4 has agreed to have separate requirements for handheld UEs and FWA UEs.

- ΔTRxSRS requirements for 6Rx

Table 2-2 below highlights the agreed  ΔTRxSRS requirements from  RAN4 meeting #114 for different SRS antenna configurations. The SRS antenna configurations for 6Rx include t3r6. For handheld UEs, single-carrier 3Tx is a Rel-19 feature while 4Tx is a Rel-18 feature for FWA UEs.

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

Table 2-2: ΔTRxSRS requirements for 6Rx

Observation #2: The ΔTRxSRS requirements for 6Rx REFSENS include t3r6 antenna configuration. Single-carrier 3Tx is a Rel-19 feature for handheld UEs while 4Tx is a Rel-18 feature for FWA UEs.

Based on observations 1 and 2, we can recommend the following proposal for 6Rx release independence:

Proposal #1: We can make the following recommendations for 6Rx release independence:

- 6Rx RF requirements for handheld UEs can be release independent from Rel-19

- 6Rx RF requirements for FWA UEs can be release independent from Rel-18

# 3	Conclusions

Observation #1: For 6Rx REFSENS, RAN4 has agreed to have separate requirements for handheld UEs and FWA UEs.

Observation #2: The ΔTRxSRS requirements for 6Rx REFSENS include t3r6 antenna configuration. Single-carrier 3Tx is a Rel-19 feature for handheld UEs while 4Tx is a Rel-18 feature for FWA UEs.

Proposal #1: We can make the following recommendations for 6Rx release independence:

- 6Rx RF requirements for handheld UEs can be release independent from Rel-19

- 6Rx RF requirements for FWA UEs can be release independent from Rel-18

# 4	References

RP-2505101 “WF on 6Rx RF requirements and SRS IL imbalance”, by Intel (topic moderator) ,  3GPP TSG-RAN WG4 Meeting #114Bis, Wuhan, China, April 7-11, 2025

RP-240828, “New WID: UE RF enhancements for NR FR1/RF2 and EN-DC, Phase 4”, by Huawei, INC, 3GPP TSG-RAN Meeting#103, Maastricht, Netherlands, March 18-21, 2024

R4-2417094, “Introduction of 6Rx (Rx part)”, by ZTE Corporation, Sanechips, Ericsson, 3GPP TSG-RAN WG4 Meeting #112Bis, Hefei, China, Oct.14th – 18th, 2024

R4-2503014, WF on 6Rx UE requirements, by AT&T, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17th Feb 2025 – 21st Feb 2025