3GPP TSG-RAN WG4 Meeting # 115		R4-2505438

Malta, MT, May 19 – 23, 2025

Agenda item:	7.1.2.3.4

Source:	Apple

Title:	On SRS Antenna Switching IL Imbalance Solution for 6Rx

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

SRS IL imbalance solution was one of the remaining 6Rx aspects that was highly debated during the most recent RAN4 meeting (Meeting #114Bis, Wuhan, China, April 7–11, 2025). Captured below is a summary of the outcome of the discussions related to this issue taken from the  meeting WF on this topic [1].

# Topic #4: SRS IL imbalance ### Sub-topic 4-2: SRS IL imbalance issue solutions Way forward - RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2. - Companies are encouraged to provide views on additional aspects including - SRS output power testing - Enhancements to PCmax equations to improve SRS IL compensation Issue 4-2-1: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behaviour for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behaviour are needed - No RAN1/4 specification impacts (i.e. this behaviour is aligned with existing specifications) Issue 4-2-2: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

As can be seen from the WF summary captured above, the two main candidate solutions that have been in consideration to address the SRS IL imbalance issue so far are the UE self-compensation and the UE assistance and reporting. We previously expressed our support for the self-compensation in the recent contributions [2] [3].The UE can always perform self-compensation of SRS IL imbalances whenever it is possible to do so (when it has enough PHR to accommodate the required PCmax per SRS resource).

In this contribution, we will provide our views on the UE assistance and reporting. We will highlight our preference between the two solutions highlighted above (solution 1 and solution 2 for issue 4-2-2 shown in the WF summary above).

# Discussion

  - UE assistance and reporting

As a baseline solution, the UE can always perform self-compensation on every SRS resource (SRS antenna ports in this case) when it can. This is when it has enough headroom relative to its maximum configured output power to overcome the ΔTRxSRS for each SRS antenna port. In this case, no RAN1 specification will be impacted if the UE self-compensates. Then the UE should indicate whether or not it compensates for insertion losses below through capability signaling. This information could help the gNB to be able to measurements to estimate the differences in the insertion losses between the SRS ports through quick measurements.

Proposal 1:	For UE assistance and reporting, we support Solution 1 from the previous meeting WF: Introduce UE capability indication on whether UE performs SRS IL self-compensation.

This capability can be signalled per SRS resource and should be an optional feature. Solution 1 is a better approach that solution 2 because the signalling can be static with no RAN1 impact. Solution 2 involves dynamic power headroom reporting with potential RAN1 involvements and signalling complexity. Solution 1 could also be used  for addressing the SRS IL imbalance issue for 2Rx/4Rx/8Rx.

Proposal 2:	Solution 1 can also be used to address the SRS IL imbalance issue for 2Rx/4Rx/8Rx.

  - Testing UE self-compensation

If the UE signals to the gNB that it does self-compensation, it would be highly desirable for the network to verify that indeed, self-compensation has been performed on a specific SRS resource (SRS antenna port in this case).

Observation #3 : A simple test could be introduced to confirm if the UE has been pre-compensated or not for SRS insertion loss imbalances.

Proposal 3:  The following test procedure is proposed to determine if the UE did indeed perform power self-compensation on each SRS port and how effective the compensation was:

- The UE is configured for SRS antenna switching

- The UE is configured for CSI reporting – CRI-RI-CQI

- The TE measures SRS and reconstructs the full channel matrix

- The TE determines the rank and CQI of the reconstructed channel

- If the UE has effectively pre-compensated the IL, the reciprocity based CQI and RI will be within certain margin of the UE reported CQI and RI

# 3	Conclusions

Proposal 1:	For UE assistance and reporting, we support Solution 1 from the previous meeting WF: Introduce UE capability indication on whether UE performs SRS IL self-compensation.

Proposal 2:	Solution 1 can also be used to address the SRS IL imbalance issue for 2Rx/4Rx/8Rx.

Observation #3 : A simple test could be introduced to confirm if the UE has been pre-compensated or not for SRS insertion loss imbalances.

Proposal 3:  The following test procedure is proposed to determine if the UE did indeed perform power self-compensation on each SRS port and how effective the compensation was:

- The UE is configured for SRS antenna switching

- The UE is configured for CSI reporting – CRI-RI-CQI

- The TE measures SRS and reconstructs the full channel matrix

- The TE determines the rank and CQI of the reconstructed channel

- If the UE has effectively pre-compensated the IL, the reciprocity based CQI and RI will be within certain margin of the UE reported CQI and RI

# 4	References

RP-2505101 “WF on 6Rx RF requirements and SRS IL imbalance”, by Intel (topic moderator) ,  3GPP TSG-RAN Meeting #114Bis, Wuhan, China, April 7-11, 2025

R4-2503195, “On SRS Antenna Switching IL Imbalance Solution for 6Rx”, by Apple, 3GPP TSG-RAN Meeting #114Bis, Wuhan, China, April 7-11, 2025

R4-2500311, “On SRS Antenna Switching IL Imbalance Solution for 6Rx”, by Apple, 3GPP TSG-RAN Meeting #114,  Athens, GR, February 17 – 21, 2025