# 3GPP TSG-RAN WG4 Meeting #115 	                                  R4-2505468

Malta, MT, 19th – 23rd May, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on existing Tx requirements for SBFD operation

Agenda Item:       7.25.3.2

Document for:     Discussion

# Introduction

In this contribution, we present overview of deployment scenarios for FR1 SBFD macro-BS.

# Discussion

## Deployment scenarios for FR1 SBFD macro-BS

In the RAN4 #114-bis meeting, RAN4 agreed to further study on the deployment scenarios for FR1 SBFD macro-BS with following agreement and WF [1].

- Way forward - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope. - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include: - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.: - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios - Option 2: Co-existence simulation and potentially analysis of co-located scenarios - Expected spectrum allocation and sub-band sizes - Expected feasible filtering performances considering the above two aspects

The SBFD-TDD "Case 3" feasibility study in Release 18 established that a combined ACIR of approximately –25 dB per operator, along with a 45 dB FFS filter mask, achieves at least 20 dB self-interference rejection and maintains adjacent-channel interference below -6 dBm in typical n77 deployments. However, extending this to SBFD-SBFD scenarios would necessitate an additional ACIR improvement of 25-30 dB without a dedicated guard band. This requirement surpasses current RF front-end capabilities, which typically provide up to 20 dB extra rejection using downlink sub-band guard strategies. Furthermore, real-world co-located grid-shifts tend to cluster around 20% (approximately 0.6 MHz at n77), a scenario already addressed by SBFD-TDD analysis. This renders SBFD-SBFD applicable only to rare contiguous-SBFD deployments. Therefore, allocating Release 19 resources to SBFD-SBFD risks delaying critical deliverables, including tightened ACLR masks and refined FFS filter specifications for heterogeneous grid-shift conditions, which directly benefit over 95% of current operators. By deferring SBFD-SBFD to Release 20, we allow for multi-vendor lab and field trials to validate per-operator filter architectures and gather empirical data on ACIR improvements in dense urban co-locations, ensuring any future SBFD-SBFD requirements are both technically achievable and aligned with widespread deployment needs.

Observation 1: SBFD-TDD already addresses coexistence challenges; therefore, pushing SBFD-SBFD into Rel-19 would consume WI time for marginal gains. We can finalize SBFD-TDD improvements now and address SBFD-SBFD in a later release when the ecosystem is ready.

Proposal 1: Exclude SBFD-SBFD from the Rel-19 scope, focus on finalizing SBFD-TDD enhancements, and defer SBFD-SBFD to Rel-20 to allow multi-vendor lab and field trials to validate per-operator filter architectures and realistic ACIR targets before standardization.

# Conclusion

In this contribution, we would like to present our perspectives on the deployment scenarios for FR1 SBFD macro-BS, outlined as follows.

Observation 1: SBFD-TDD already addresses coexistence challenges; therefore, pushing SBFD-SBFD into Rel-19 would consume WI time for marginal gains. We can finalize SBFD-TDD improvements now and address SBFD-SBFD in a later release when the ecosystem is ready.

Proposal 1: Exclude SBFD-SBFD from the Rel-19 scope, focus on finalizing SBFD-TDD enhancements, and defer SBFD-SBFD to Rel-20 to allow multi-vendor lab and field trials to validate per-operator filter architectures and realistic ACIR targets before standardization.

# Reference

- R4-2504752 Way Forward for [114bis][307] NR_duplex_evo_BSRF