3GPP TSG-RAN WG4 Meeting #115        				            R4-2505475

Malta, MT, 19th - 23rd May 2025

Title:	RRM Core requirements on conditional intra-CU LTM

Source:	China Telecom

Agenda item:			7.29.3.3

Document for:	Discussion

- Introduction

In RAN4#114bis meeting, some conclusions on conditional intra-CU LTM have been duplicated as follows [1]. From this meeting, we will continue to discuss the RRM Core requirements on conditional intra-CU LTM.

- Discussion

Issue 1-2-1: UE based TA measurement

In RAN2, both UE based TA measurement and PDCCH ordered RACH are introduced for early UL Synchronization in CLTM. And in R18 LTM, UE based TA measurement is also agreed in RAN2, however RAN4 did not define the relate requirements due to time limit.

We can see the benefactions of UE based TA measurement for CLTM, such as, less signaling. However, if we defined the related requirements, lots of aspects need to be discussed. e.g., reference signals type, transmit timing accuracy, TA accuracy and network scheduling. Consider the workload, we could agree to not define RRM requirements for UE based TA measurement in R19.

Proposal 1: Not define RRM requirements for UE based TA measurement in R19.

Conditional Intra-CU LTM delay

In last RAN4 meeting, some conclusions on conditional intra-CU LTM delay have been duplicated as follows.

- Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

For other components, further discussion is as follow,

TRRC

Legacy definition could be reused, i.e., TRRC is the RRC procedure delay.

TLTM-processing

Legacy definition could be reused, i.e., TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. And the value could be reused.

Tfirst-RS / TRS

TRS-proc

< Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not.

For the case where servicing cell and target cell are in different frequency layer, before Tfirst-RS and TRS-proc. RF retune is necessary, so we think Tfirst-RS and TRS-proc should are included in Tinterrupt.

For the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not. RAN2 has agreed that Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. So, we think the conditions could be reuse.

Proposal:

TLTM-IU

TLTM-IU is the interruption uncertainty during LTM cell switch stated in clause 6.3.1.3.

Proposal 2:

TRRC is the RRC procedure delay.

Tfirst-RS and TRS-proc are included in Tinterrupt.

- Reuse the values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM.

TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. And the value could be reused.

TLTM-IU is the interruption uncertainty during LTM cell switch stated in clause 6.3.1.3.

Subsequent Conditional LTM

According to the agreement in the previous meeting, the difference between subsequent conditional LTM and initial conditional LTM is the starting point, we have agreed that the starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. So, we could define the subsequent conditional LTM delay as it is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL transmission on the new target cell.

For the components of subsequent conditional LTM delay, TRRC will not be considered, and TEvent_DU needs to be updated. Others could keep same as initial conditional LTM.

Proposal 3: Subsequent conditional LTM delay is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL transmission on the new target cell.

Proposal 4: TRRC will not be considered, and TEvent_DU needs to be updated. Others could keep the same as initial conditional LTM.

- Summary

In this paper, we provide our views on conditional intra-CU LTM. From this discussion we have derived the following proposals:

Proposal 1: Not define RRM requirements for UE based TA measurement in R19.

Proposal 2:

TRRC is the RRC procedure delay.

Tfirst-RS and TRS-proc are included in Tinterrupt.

- Reuse the values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM.

TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. And the value could be reused.

TLTM-IU is the interruption uncertainty during LTM cell switch stated in clause 6.3.1.3.

Proposal 3: Subsequent conditional LTM delay is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL transmission on the new target cell.

Proposal 4: TRRC will not be considered, and TEvent_DU needs to be updated. Others could keep the same as initial conditional LTM.

- References

[1] R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom.