3GPP TSG-RAN WG4 Meeting #114bis	R4-2505501

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on NTN testing for NGSO

Agenda item:	7.10.5.3

Document for:	Discussion

Introduction

This document present Nokia’s views on NTN NGSO Channel Model for discussion in RAN4#115.

Issues below are based on those open for discussion in the WF from RAN4#114-bis [1]

Discussion

Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models

During RAN4#114-bis only one option remained open in the WF for discussion at RAN4#115, than being how does RAN4 define performance requirements in a time varying channel.

Nokia proposes that one performance requirement is defined per test case, such the that an aggregate average SNR is taken over the entire test duration, this aligns with option 1 from the way forward.

The SNR may vary throughout the test, however an aggregate average SNR may be used.

RAN4 shall keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied

RAN4 shall keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

Conclusion

The paper discusses the topic of NTN NGSO Channel model for discussion at RAN4#114-bis

The following Observations and Proposals were made:

Observation 1: The SNR may vary throughout the test, however an aggregate average SNR may be used.

Proposal 1: RAN4 shall keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied

Proposal 2: RAN4 shall keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

- References

- R4-2504700, “Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod”, Samsung, Wuhan, RAN4#114-bis