3GPP TSG-RAN4 Meeting #115	R4-2505508

# St Julian, Malta 19th – 23rd May, 2025

Source: 	Qualcomm Incorporated

Title: 	CSI Prediction: Testing Framework

Agenda item:	7.21.3

Document for:	Discussion

# Introduction

In this paper, we focus on testing framework of CSI prediction.

# Prior Agreements

Following agreement happened during the last few meetings [1][5][7]

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4 Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition Issue 1-2: Reference model for CSI prediction Agreement: - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model. Issue 1-3: Training and test data consistency Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations Issue 1-4: Generalization Agreement: - Postpone this discussion until the requirement and test definition is clear

# Discussion of Testing Framework

## Selection of Doppler

According to the simulation results that got submitted to RAN4 114bis [8], most companies observed poor SGCS with 100Hz Doppler. Table 1 shows the reported SGCS results with 100Hz. We have also observed poor SGCS with 100 Hz and reported that in our contribution for this meeting [10].

RAN4 should deprioritize the use of 100Hz for evaluating the next step (throughput) of AI-ML based CSI prediction.

Table 1: Reported SGCS results with 100Hz Doppler (other companies’ last meeting’s result and our recent result)

|  | gNB TX antennas | 100Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Rank1 | Rank2-average | Rank2-layer1 | Rank2-layer2 | Non-AI (Sample and hold) |
| Qualcomm | 16 | 0.4314 |  | 0.4314 | 0.2363 | 0.2992 for rank 1 0.2342, on average, for rank 2 |
| MediaTek | 16 |  | 0.81235 | 0.8636 | 0.7611 |  |
|  | 32 |  | 0.7921 | 0.8501 | 0.7341 |  |
| Apple | 16 | 0.45 |  |  |  | 0.45 |
| CMCC | 16 |  |  |  |  |  |
| ZTE | 16 | 0.94 |  |  |  | 0.81 |
| vivo | 32 | 0.298 |  |  |  |  |
| OPPO | 32 | 0.2846 |  |  |  |  |
| Huawei, HiSilicon | 16 | 0.3011 | 0.2334 |  |  |  |
| Ericsson | 32 | 0.31 | 0.24 | 0.31 | 0.17 |  |
| Samsung | 16 | 0.995 | 0.992 | 0.995 | 0.989 | 0.185 for Rank1 0.1635 for Rank2-ave 0.185 for Rank2-layer 1 0.142 for Rank2-layer 2 |
|  | 32 | 1 | 1 | 1 | 1 | 0.467 for Rank1 0.44 for Rank2-ave 0.467 for Rank2-layer 1 0.413 for Rank2-layer 2 |

Observation 1: According to last meeting’s contributions, majority of companies observed poor SGCS with 100Hz Doppler.

Proposal 1: RAN4 deprioritizes the use of 100Hz for evaluating the next step (throughput) of AI-ML based CSI prediction.

## Selection of Locations for Predicted PMI and Scheduled PDSCH with Predicted PMI in FDD Tests

RAN4 has agreed to introduce a test with Doppler basis, N4 = 1, and is currently investigating 5ms CSI-RS periodicity. RAN4’s considered simulation framework for this work item also assumes the following for FDD tests [14]

“If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4)”

Due to these considerations, the reported PMI gets applied for PDSCH 3-7 ms after the time location of predicted PMI. Figure 1 has been copied from the example FD test setup of last meeting’s agreement [10] and its shows this 3-7 ms time separation between the time location for predicted PMI and scheduled PDSCH with predicted PMI.

Figure 1: Example FDD test setup shown in [10]

Observation 2: RAN4 has agreed to introduce a test with Doppler basis, N4 = 1, and is currently investigating 5ms CSI-RS periodicity.

Observation 3: In FDD tests, RAN4 is currently considering 4 ms time separation between I and II, and II to III where I, II and III are mentioned below:

- Slot containing the last CSI-RS occasion of the observation window

- PUSCH conveying predicted PMI

- Slot where predicted PMI is applied in scheduled PDSCH

This 3-7 ms delay may adversely impact the PDSCH throughput since channel and ideal PMI might have changed significantly during 7ms. It is important to ensure that the time separation between predicted PMI location and scheduled PDSCH remains limited.

Also, the prediction window in Figure 1 is 5ms long. Hence, the PDSCH that gets scheduled 6-7 ms after the last CSI-RS occasion of the observation window falls outside the prediction window.

Observation 4: Combination of observation 2 and 3 mean that the predicted PMI may get applied in scheduled PDSCH 3-7 ms after the time location of predicted PMI. This 3-7 ms delay may adversely impact the PDSCH throughput since channel and ideal PMI might have changed significantly during 7ms.

Observation 7: In Figure 1, the PDSCH that gets scheduled 6-7 ms after the last CSI-RS occasion of the observation window falls outside the prediction window.

The issues of observation 6 and 7 arise because of the combination of following factors:

      - 4 ms time separation between last CSI-RS slot and PMI report

      - 4 ms time separation between PMI report and application of PMI in scheduled PDSCH

      - 5 ms long prediction window

RAN1 is currently discussing required time separation between last CSI-RS slot and AI-ML based predicted PMI report. One way to reduce the scheduling delay would be to reduce the time separation between PMI report and application of PMI in scheduled PDSCH.

Proposal 2: RAN4 considers reducing the time separations between PMI report and scheduled PDSCH with predicted PMI in Rel-19 AI-ML CSI prediction tests compared to the values assumed in Rel-18 non-AI-ML CSI prediction tests.

- For example, RAN4 considers assuming the following in FDD tests: “This reported PMI cannot be applied at the gNB downlink before slot#(n+3)”

## Time Separation between Last CSI-RS Slot and PUSCH Report Containing Predicted PMI

In FDD tests, RAN4 is currently considering allowing 4ms to generate PUSCH containing predicted PMI after receiving the last CSI-RS occasion of the observation window. In Rel-18, spec required UE to generate PUSCH containing predicted PMI, designed based on non-AI-ML prediction, in Doppler domain basis N4 = 1 scenario if time separation between last CSI-RS of observation window and PUSCH is greater than Z2’. Value of Z2’ is less than 3ms.

Due to the additional time complexity of AI-ML based CSI prediction, this minimum time separation between CSI-RS and PUSCH may be higher than Z2’ for Rel-19 AI-ML CSI prediction.

RAN4 should revisit the time separation between CSI-RS and PUSCH in FDD test setup after RAN1 finalizes the minimum time separation between these two channels for CSI computation.

Observation 6: In Rel-18, spec required UE to generate PUSCH containing predicted PMI, designed based on non-AI-ML prediction, in Doppler domain basis N4 = 1 scenario if time separation between last CSI-RS of observation window and PUSCH is greater than Z2’. Value of Z2’ is less than 3ms.

Observation 7: RAN4 should revisit the time separation between CSI-RS and PUSCH in FDD test setup after RAN1 finalizes the minimum time separation between these two channels for computing AI-ML based predicted PMI.

# Conclusions

Observation 1: According to last meeting’s contributions, majority of companies observed poor SGCS with 100Hz Doppler.

Observation 2: RAN4 has agreed to introduce a test with Doppler basis, N4 = 1, and is currently investigating 5ms CSI-RS periodicity.

Observation 3: In FDD tests, RAN4 is currently considering 4 ms time separation between I and II, and II to III.

- Slot containing the last CSI-RS occasion of the observation window

- PUSCH conveying predicted PMI

- Slot where predicted PMI is applied in scheduled PDSCH

Observation 4: Combination of observation 2 and 3 mean that the predicted PMI may get applied in scheduled PDSCH 3-7 ms after the time location of predicted PMI. This 3-7 ms delay may adversely impact the PDSCH throughput since channel and ideal PMI might have changed significantly during 7ms.

Observation 5: In Rel-18, spec required UE to generate PUSCH containing predicted PMI, designed based on non-AI-ML prediction, in Doppler domain basis N4 = 1 scenario if time separation between last CSI-RS of observation window and PUSCH is greater than Z2’. Value of Z2’ is less than 3ms.

Observation 6: RAN4 should revisit the time separation between CSI-RS and PUSCH in FDD test setup after RAN1 finalizes the minimum time separation between these two channels for computing AI-ML based predicted PMI.

Proposal 1: RAN4 deprioritizes the use of 100Hz for evaluating the next step (throughput) of AI-ML based CSI prediction.

Proposal 2: RAN4 considers reducing the time separations between PMI report and scheduled PDSCH with predicted PMI in Rel-19 AI-ML CSI prediction tests compared to the values assumed in Rel-18 non-AI-ML CSI prediction tests.

- For example, RAN4 considers assuming the following in FDD tests: “This reported PMI cannot be applied at the gNB downlink before slot#(n+3)”

# References

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- 3GPP TR 38.843 V 18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- R1 116bis Chairman’s notes, Hunan, China, April 2024.

- R4-2502970, Baseline simulation assumptions for AIML based CSI prediction, 3GPP TSG RAN WG4 #114.

- R4-2502856, WF on the requirements for AI/ML air interface, 3GPP TSG RAN WG4 #114.

- 38.822, v 18.0.0, UE feature list.

- R4-2505105, WF on requirements for AI/ML air interface normative work, 3GPP TSG RAN WG4 #114bis.

- R4-2505153, Collection of simulation result for AI-ML CSI prediction, RAN4 114bis.

- R4-2505513, “CSI Prediction: Simulation Results”, Qualcomm Incorporated, RAN4 115.

- R4-2505229, “Updated simulation assumptions for AIML CSI prediction”, RAN4 #114bis.