# 3GPP TSG-RAN WG4 Meeting # 115	R4-2505532

# Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with DL inter-band CA_n3-n39

Agenda Item:	7.12.2.1

Document for:	Approval

Introduction

In the RAN4#114bis meeting, the remaining open issue is shown below based on the approved WF [1].

Issue 1-3: MSD for CA_n3A-n39A with n3 UL Agreement: - MSD requirements will be decided based on the average values among companies’ simulation results. - Encourage companies to provide results next meeting.

In this paper, we’d like to resubmit our results on MSD for CA_n3A-n39A with n3 UL of ATG UE.

Discussion

We’d like to provide common antenna RF architectures below to allow the transition guard band between DL band n3 and band n39.

figure 1 Common antenna RF architecture for DL CA_n3-n39 with UL in either band n3 or n39

In addition, RAN4 had a way forward on studying the MSD value of band n39 DL with band n3 UL aggressor.

Issue 2-3: Assumed parameters for CA_n3A-n39A with n3 UL WF: - Use the following parameters in the approved Tdoc R4-2410641 with 20dB antenna isolation as baseline to analysis the MSD value for n39 DL. - Other parameters are not precluded. Parameters; Value. Antenna isolation; [20dB]. Tx attenuation at 1880~1920MHz for band n3 duplexer; [30dB]. Rx attenuation at 1710~1785MHz for band n3+n39 Rx filter; [40dB].

The MSD values for different output power are summarized below.

table 1 MSD values of DL band n39 with band n3 UL interference for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| 23dBm | 0.16 |
| 24dBm | 0.19 |
| 25dBm | 0.24 |
| 26dBm | 0.29 |
| 27dBm | 0.36 |
| 28dBm | 0.44 |
| 29dBm | 0.55 |
| 30dBm | 0.68 |
| 31dBm | 0.84 |
| 32dBm | 1.04 |
| 33dBm | 1.28 |
| 34dBm | 1.58 |
| 35dBm | 1.94 |
| 36dBm | 2.36 |
| 37dBm | 2.87 |
| 38dBm | 3.46 |
| 39dBm | 4.16 |
| 40 dBm | 4.95 |

Proposal 1: For ATG UE, RAN4 can specify the following MSD requirements for DL band n39 with band n3 UL interreference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1722.5 | 25 | 15 | 133 (RBstart=0) | 1900 | 30 | MSD Table 1 | >ACLR2 |

Summary

Proposal 1: For ATG UE, RAN4 can specify the following MSD requirements for DL band n39 with band n3 UL interreference

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1722.5 | 25 | 15 | 133 (RBstart=0) | 1900 | 30 | MSD Table 1 | >ACLR2 |

- References

[1] R4-2505112	WF on UE RF requirements for NR ATG, CMCC

[2] R4-2502952, WF on UE RF requirements for ATG, CMCC