# 3GPP TSG-RAN WG4 Meeting # 115	R4-2505537

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Tx requirements for NR NTN HPUE

Agenda Item:	7.10.3.2.1

Document for:	Approval

Introduction

In the RAN4#114bis meeting, there are some remaining open issues in the following way forward [1].

Issue 1-1-2: Power boosting option for PC2 Agreement: - FFS on the power boosting option for PC2, including - Whether or not and how to define power boosting for PC2 as proposed above. - The power boosting discussion not to impact the requirements discussion for PC2 without power boosting, e.g. MPR/AMPR, for PC2. Issue 1-7-2: PC1 ACLR Tentative agreement: - PC1 ACLR is [34] dBc - Companies to further evaluate this, and to be confirmed in next meeting. Issue 1-8-2: PC1 MPR Way forward: - To collect MPR analysis and determine MPR for PC1 in next meeting.

Based on the WF in last meeting, the remaining issues are for PC2 power boosting and PC1 ACLR/MPR requirements.

Consideration about spec architecture

As RAN4 has agreed to consider the following RF architectures for different power classes, RAN4 may need to discuss how to capture the RF requirements for different RF architectures for different power classes.

|  | 1Tx | 2Tx | 4Tx |
| --- | --- | --- | --- |
| PC2 | 1Tx for both handheld and non-handheld UE | 2Tx for both handheld and non-handheld UE | N/A |
| PC1.5 | N/A | 2Tx for non-handheld UE | N/A |
| PC1 | 1Tx for non-handheld UE | N/A | 4Tx for non-handheld UE |

In current TS 38.101-5, only the RF requirements for 1Tx PC3 were defined. In addition, NR NTN UE may not support the UL MIMO. Thus, RAN4 need to create the new suffix for TxD requirements under the assumption of the RF architectures more than 1Tx.

Observation 1: RAN4 need to create the new suffix for TxD requirements under the assumption of more than 1Tx.

PC1 ACLR and MPR requirements

In last meeting, RAN4 didn’t reach a consensus on how to specify the PC1 ACLR requirements for NR NTN non-handheld UE. Based on the adjacent channel coexistence study in the WF [2], it’s observed that the required ACLR for 0km isolation distance is less than 32.7dB. According to our simulation results [3], 28dB ACLR for NR NTN PC1 can be observed.

For LEO1200-Urban Macro scenario, the results for 2GHz scenario 4 at PC1 are shown below.

| Sate UE ACLR1 [dB] | 20 | 22 | 24 | 26 | 28 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput baseline [Mbps] | 35.887 | 35.922 | 35.87 | 35.91 | 35.966 | 35.915 | 35.885 |
| Average throughput with adjacent interference [Mbps] | 35.496 | 35.571 | 35.583 | 35.681 | 35.758 | 35.731 | 35.741 |
| Average throughput loss | 1.09% | 0.98% | 0.80% | 0.64% | 0.58% | 0.51% | 0.40% |
| 5%-tile throughput baseline [Mbps] | 15.085 | 15.043 | 15.03 | 14.973 | 15.349 | 15.194 | 15.086 |
| 5%-tile throughput with adjacent interference [Mbps] | 13.604 | 13.597 | 13.847 | 14.173 | 14.618 | 14.561 | 14.565 |
| 5%-tile throughput loss | 9.82% | 9.61% | 7.87% | 5.34% | 4.76% | 4.17% | 3.45% |

As RAN4 has agreed that 31dB ACLR is applicable to 29dBm PC1.5 NR NTN UE, in theory 33dB ACLR is enough for 31dBm PC1 NR NTN UE to maintain the same interference level as PC1.5 NR NTN UE.

Observation 2: the required ACLR for NR NTN PC1 under 0km isolation distance scenario is less than 32.7dB.

Observation 3: 33dB ACLR is enough for 31dBm PC1 NR NTN UE to maintain the same interference level as PC1.5 NR NTN UE.

As companies discussed in last meeting, TN PC1 devices only focus on the area of public safety other than general IMT communications. That means some special PAs and duplexers should be used to meet the strict requirements of current TN PC1 devices. In order to enrich the ecosystem of NTN PC1 device, it’s better to relax the unnecessarily strict requirements, e.g. PC1 ACLR and MPR requirements.

Proposal 1: RAN4 specify the ACLR requirements no more than 33dB for PC1 NR NTN UE.

We did some MPR simulation based on the PA model calibration approved in the WF [4].

- Aim for MPR/A-MPR analysis in #114bis for different power classes - For PA model calibration: - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB - SEM: existing PC3 SEM - EVM: 17.5%

The higher ACLR requirements we assumed, the higher MPR requirements for both outer region and inner region.

If the ACLR requirements for PC1 NR NTN UE are less than 33dB, the MPR table 6.2D.2-3 of TS 38.101-1 could be reused.

If the ACLR requirements for PC1 NR NTN UE are larger than 33dB, the MPR table 6.2D.2-5 of TS 38.101-1 could be reused.

Observation 4:

If the ACLR requirements for PC1 NR NTN UE are less than 33dB, the MPR table 6.2D.2-3 of TS 38.101-1 could be reused.

If the ACLR requirements for PC1 NR NTN UE are larger than 33dB, the MPR table 6.2D.2-5 of TS 38.101-1 could be reused.

Summary

Observation 1: RAN4 need to create the new suffix for TxD requirements under the assumption of more than 1Tx.

Observation 2: the required ACLR for NR NTN PC1 under 0km isolation distance scenario is less than 32.7dB.

Observation 3: 33dB ACLR is enough for 31dBm PC1 NR NTN UE to maintain the same interference level as PC1.5 NR NTN UE.

Proposal 1: RAN4 specify the ACLR requirements no more than 33dB for PC1 NR NTN UE.

Observation 4:

If the ACLR requirements for PC1 NR NTN UE are less than 33dB, the MPR table 6.2D.2-3 of TS 38.101-1 could be reused.

If the ACLR requirements for PC1 NR NTN UE are larger than 33dB, the MPR table 6.2D.2-5 of TS 38.101-1 could be reused.

- References

[1] R4-2505113, WF on HPUE requirements for NR-NTN, Samsung

[2] R4-2420403, WF on NR NTN HPUE, Samsung

[3] R4-2419071, Discussion on coexistence study for NR NTN HPUE, Huawei, HiSilicon

[4] R4-2502954, WF on HPUE requirments for NR-NTN, Samsung